Safety Instruction

1) Users are required to read the operation manual completely and carefully before installation or operation.

2) The product should be installed and pre-operated by well-trained persons

3) All the instruction marked with sign ，must be observed or executed; Otherwise, bodily injuries might occur.

4) For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection

5) When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250V AC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage. Continue supply 380V power over 5 minutes might damage the fuse F2, burst the electrolytic capacitors and the power module U14 of the main board and even might endanger the person safety.

6) Please don’t operate in direct sun light, outdoors area and where the room temperature is over 45℃or below 0°C.

7) Please avoid operating near the heater at dew area or at the humidity below 10% or above 95%.

8) Please don’t operate in area with heavy dust, corrosive substance or volatile gas.

9) Avoid power cord being applied by heavy objects or excessive force, or over bend.

10) The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

11) All the moving portions must be prevented to be exposed by the parts provided.

12) Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

13) Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine arm.

4. Repairing or doing any mechanical adjustment.

5. Machine is out of work.

14) Repairing and high level maintenance work should only be done by electronic technicians with appropriate training.

15) All the spare parts for repairing work must be provided or approved by the manufacturer.

16) Don’t use any objects to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge.

However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency.

* Note: We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this all a failsafe device. (Such as residual current breaker).

| 1．Operation Panel for each key name and function definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Icon | Description |  | Remarks |
| 1 |  | Function key |  | ①In the normal mode hold [P] key enter to User parameter setting mode.。 ②In the shut-off condition, hold [P] key turn on machine enter to technician parameter setting mode. ③In the shut-off condition, Hold [P] key and [S] key turn on the machine enter to the senior parameter setting mode. ④Under the parameter setting interface, Press [P] key to return but not save the parameter. |
| 2 |  | Up key / Speed increase |  | ①In the normal mode, press this key can increase speed, long-press for 3s can enter to speed setting interface. ②Under the parameter setting interface, press this key can increase the parameter. |
| 3 |  | Down key / Speed decrease |  | ①In the normal mode, press this key can decrease speed, long-press for 3s can enter to speed setting interface. ②Under the parameter setting interface, press this key can decrease the parameter. |
| 4 |  | Left key / LED- |  | ①In the normal mode, press this key can increase brightness. ②Under the parameter setting interface, press this key can check last parameter |
| 5 |  | Right key / LED+ |  | ①In the normal mode, press this key can decrease brightness. ②Under the parameter setting interface, press this key can check next parameter |
| 6 |  | Save key |  | Under the parameter setting interface, press [S] key to save. |
| 7 |  | Forward trimming |  | ①In the normal mode, press this key can execute the cycle shift of forward trimming ON, forward trimming OFF. Press up and down key it can adjustment forward trimming action times. (same as P-27) ②In the normal mode, long-press this key for 3s can enter to trimming setting interface. |
| 8 |  | Backward trimming |  | ①In the normal mode, press this key can execute the cycle shift of backward trimming ON, backward trimming OFF. Press up and down key it can adjustment backward trimming action times. (same as P-28) ②In the normal mode, long-press this key for 3s can enter to trimming setting interface. |
| 9 |  | Auto-suction mode shortcut key |  | ①Press this key it can execute the cycle shift of suction off, forward suction on, backward suction on, suction on. ②Long-press this key for 3s to enter to auto-suction setting. |
| 10 |  | Mode selection shortcut key |  | ①In the normal mode, press this key can execute the cycle shift of the following four modes: 1. Full automatic: start-up according full sensor signal. 2. Free sewing. 3. Semi- automatic: start-up according to front sensor signal + pedal completion (same as P-05). 4. Full manual: start-up according to the pedal completion. ②In the normal mode, long-press this key for 3s to enter to mode selection setting. |
| 11 |  | Presser Foot Lifting Mode shortcut key |  | ①In the normal mode, Press this key it can execute the cycle shift of close, front foot lifting, Rear foot lifting, front and rear foot lifting, close (same as P-10). ②In the normal mode, long-press this key for 3s to enter to presser foot lifting mode setting. |
| 12 |  | Sensor situation check shortcut Key |  | ①In the normal mode, Short-press this key can check sensor real-time sensor value and current threshold value. ②In the normal mode, long-press this key for 3s to enter to sensor sensitivity setting interface. |
| 2． Special function operation definition |  |  |  |  |
| No. | Function name |  | Function Description |  |
| 1 | Restore factory setting |  | ①Under the shutdown state, press and two key at the same time to turn on machine, display screen show P-26，press [S] key confirm. ②Shortcut Mode : Use needle insert RESET hole, after long-press 3 sec after beep then restore factory setting. |  |
| 2 | UP position adjustment by manual |  | 1. Under the shutdown state, press [P] +[S] key at the same time to turn on machine, display P-70; 2. Adjust parameter P-72; 3. From hand-wheel direction, clockwise turn hand-wheel to up position, press [S] key to save current value. |  |
| 3 | Down position adjustment |  | 1. Under the shutdown state, press [P] +[S] key at the same time to turn on machine, display P-70; 2. Adjust parameter P-73; 3. From hand-wheel direction, clockwise turn hand-wheel to down position, press [S] key to save current value. |  |
| 4 | Sensor sensitivity adjustment |  | 1. Under the normal mode interface long-press key, it can direct access sensor sensitivity adjustment interface. 2. Please remove cloth after Three-sensor, press [S] key; 3. Please cover cloth after three-sensor, press [S] key, finished setting. |  |
| 5 | Piecework function |  | Under the normal mode interface press and key at the same time，press or key it can increase or decrease operation to the total piece manually；piecework other corresponding parameter setting: Pressandkeys at the same time to turn on the machine, press key to find P-75 setting pieces of alarm, find P-76 setting alarm pieces plus minus sequence. |  |

| 3．System Parameter Setting Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| User Parameter ( Under the normal mode press [P] key to access) |  |  |  |  |
| No | Function parameters | Range | Default | Description |
| P-01 | Sewing Speed（r/min） |  |  | Maximum sewing speed setting Note: 900E(Range: 100-5000, Default: 4500) 950E(Range: 100-5500, Default: 5000) |
| P-02 | Needle stop positioning selection | 0-1 | 0 | 0:Up needle position 1: Down Needle position |
| P-03 | Start sewing speed（r/min） | 100-6000 | 4000 | With trimming function: the speed setting for front sensor trigger start-up and Back sensor signal finish (before trimming) |
| P-04 | Start-up mode | 0-1 | 1 | （Valid for full-automatic mode） 0: Automatic mode 1: Foot control mode（Start-up by Front sensor + toe down the pedal） |
| P-05 | Automatic/semi-automatic mode selection | 0-1 | 1 | （P-06 is turn on） 0: Semi-automatic(Match with P-04 use) 1: Automatic(Foot control mode) |
| P-06 | Auto sensing switch | 0-1 | 1 | 0: OFF（Full manual mode is turn on） 1: ON（It can execute automatic/ semi-automatic mode also match with P-05 use） |
| P-07 | Auto trimming switch | 0-3 | 3 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: Front trimming（Mid-sensor receive signal then trimming） 2: Rear trimming （Back-sensor receive signal after finished trimming） 3: First and later trimming |
|  | Automatic thread tension release switch | 0-1 | 1 | (P-70=Side knife type, this parameter is valid) 0: OFF 1: Backward thread tension release |
| P-08 | Auto suction | 0-3 | 3 | 0: OFF 1: Front trimming suction 2: Rear trimming suction 3: Front and rear trimming suction |
| P-09 | Automatic cloth feeding | 0-1 | 1 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: ON |
| P-10 | Auto presser foot lifting | 0-3 | 1 | 0: OFF 1: Front foot-lifting（Front sensor receive signal） 2: Rear foot-lifting（Rear sensor signal after finish） 3: Front and Rear foot-lifting |
| P-11 | Presser foot lifting when stop on the sewing | 0-1 | 0 | (Valid for full-manual and semi-automatic mode） 0: OFF 1: ON（when stop on the sewing the presser foot lifting automatically） |
| P-12 | Presser foot lifting after trimming | 0-1 | 1 | 0: OFF 1: ON |
| P-13 | Presser foot lifting when half back treadle | 0-2 | 0 | （Valid for full-manual / semi-automatic mode） 0: Presser foot lifting when half back step/ back step 1: Lay down 2: Reverse step up presser foot |
| P-14 | Manual trimming suction | 0-2 | 1 | 0: No suction 1: Rear suction 2: Front suction and rear suction |
| P-15 | Machine head lamp brightness | 0-4 | 3 | 0: OFF；1→4 the brightness grade, the more value the more brightness. |
| P-16 | Low air pressure mode | 0-1 | 0 | 0: Normal mode 1: Low pressure mode |
| P-17 | Semi-automatic continuous sewing | 0-1 |  | 1: OFF 0: Continuous sewing when toe down the treadle (P-70= Horizontal knife type, Default: 1) (P-70= Side knife type, Default: 0) |
| P-18 | Semi-automatic constant rate trimming | 0-1 | 0 | 0: OFF （after finish before trimming not execute P-03 speed） 1: Constant trimming speed the back sensor speed is fixed with P-03 |
| P19 | Backward thread tension release switch selection | 0-1 | 1 | (P-70=Side knife type, this parameter is valid) 0: Front sensor trigger 1: Rear sensor trigger |
| P-20 | Stop position after trimming selection | 0-1 | 1 | 0: Down position 1: Up position |
| P-21 | Front thread action time(stitches) | 0-50 | 5 | (P-70=Side knife type, this parameter is valid) |
| P-22 | Front Receiver E# switch | 0-1 |  | 0: ON（turn on the front sensor, sensor signal + pedal） 1: OFF (Close front sensor, pedal direct turn on) (P-70= Horizontal knife type, Default: 0) (P-70= Side knife type, Default: 1) |
| P-23 | Intermittent suction opening time（×100ms） | 1-600 | 20 | The more value the more time |
| P-24 | Intermittent air suction off time (×100ms) | 0-600 | 0 | 0: for no intermittent air suction Front suction off time (valid when P-46 function setting is 1) |
| P-25 | Low pressure mode, selvage suction synchronization time （P16=1） | 0-2000 | 200 | Valid for P-16 setting is 1. |
| P-26 | The stitches number between two sensors | 1-600 | 100 | Cycle period parameter. In a circle the rear sensor receive signal then can continue running, otherwise a cycle stop. |
| P-27 | The delayed stitches number of front trimming | 0-50 | 4 | (P-70= Horizontal knife type, this parameter is valid) The smaller value the more in advance, thread will retain more longer. (Mid-sensor trigger ) |
| P-28 | The delayed stitches number of rear trimming | 0-50 | 4 | (P-70= Horizontal knife type, this parameter is valid) The small value the more in advance, the short of end-thread (Back-sensor trigger) |
|  | The starting time for rear thread | 0-50 | 2 | (P-70= Side knife type, this parameter is valid) |
| P-29 | The starting stitches for front suction(stitches) | 0-50 | 1 | (P-70= Side knife type, this parameter is valid) |
| P-30 | The delayed time of front suction off（ms） | 100-5000 | 100 | (P-70= Horizontal knife type, this parameter is valid) |
|  | Number of stitches when forward suction OFF (stitches) | 0-50 | 25 | (P-70= Side knife type, this parameter is valid) |
| P-31 | The delayed time after trimming suction. (ms) | 100-5000 |  | The small value the close is fast. (P-70= Horizontal knife type, Default: 300) (P-70= Side knife type, Default: 1000) |
| Technician parameter (Press [P] key and power on to enter) |  |  |  |  |
| P-32 | The times when cloth-pulling ON | 0-50 | 20 | (P-70= Horizontal knife type, this parameter is valid) |
| P-33 | Times of Backward suction ON (stitches) | 0-50 | 3 | (P-70= Side knife type, this parameter is valid) |
| P-34 | Trimming protection by manually | 0-1 | 1 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: ON |
| P-35 | The delayed stitches number before the machine stops (Stitches) | 1-99 |  | After through cloth back-sensor how many stitches will stop the machine automatically. (Valid for Back-sensor OFF condition） (P-70= Horizontal knife type, Default: 1) (P-70= Side knife type, Default: 3) |
| P-36 | The response time of front sensor (ms) | 10-990 | 50 | Front sensor trigger time: the small value the response is fast, the large value the response is slow. |
| P-37 | The sensitivity of front sensor | 0-700 | 480 | In order to adapt different cloth material setting front sensor receive strength. |
| P-38 | The sensitivity of middle sensor | 0-700 | 540 | In order to adapt different cloth material setting mid-sensor receive strength. |
| P-39 | The delaying time of front presser foot lifter（ms） | 100-2000 | 300 | （Under the full automatic/ semi-automatic） Front trigger retention time after auto-foot-lifting, the larger value the longer retention time. |
| P-40 | Starting time for rear presser foot lifter（ms） | 0-2000 | 120 | The starting time of rear presser foot lifting, the smaller value the faster of response. |
| P-41 | Full-on time setting for presser foot（ms） | 10-990 | 100 | The larger value the higher of foot-lifting (Noted: Not too high) |
| P-42 | Duty-cycle time setting for presser foot（%） | 10-90 | 20 | Adjustment for duty-cycle of AFL. Fine tuning can reduce the over- heating. |
| P-43 | The time for the presser foot laying down（ms） | 10-990 | 100 | The action time of presser foot laying down sequence. |
| P-44 | Presser foot protection(s) | 1-120 | 5 | Stop on the sewing the presser foot lifting, stop after trimming presser foot lifting, positive closing after heeling pedal for retention time. |
| P-45 | Trimming time（ms） | 10-990 | 40 | (P-70= Horizontal knife type, this parameter is valid) the longer time, the greater trimming pressure |
|  | Tension release times（ms） | 0-5000 | 1000 | (P-70= Side knife type, this parameter is valid) |
| P-46 | Suction when continuous feeding | 0-2 |  | 0: No suction 1: Long suction 2: Synchronizer suction (P-70= Horizontal knife type, Default: 0) (P-70= Side knife type, Default: 2) |
| P-47 | Total pieces | 0-65535 | 0 |  |
| P-48 | Needle goes up as power on | 0-1 | 1 | 0: OFF 1: ON |
| P-49 | Stitch（mm） | 1-7 | 3 | Adjustment same as actual needle distance by up and down key. |
| P-50 | Up needle position adjustment | 0-1439 |  | Same as P-72 |
| P-51 | Down position adjustment | 0-1439 |  | Same as P-73 |
| P-52 | Test speed（r/min） | 100-6000 | 5500 | Adjustment by up and down key |
| P-53 | Test working time （×100ms） | 1-250 | 50 | Adjustment by up and down key |
| P-54 | Test stop time （×100ms） | 1-250 | 20 | Adjustment by up and down key |
| P-55 | Item A testing: Continuous running | 0-1 | 0 | 0: STOP 1: RUN |
| P-56 | Item B testing: With function running | 0-1 | 0 | 0: STOP 1: RUN |
| P-57 | Item C testing: Without function running | 0-1 | 0 | 0: STOP 1: RUN |
| P-58 | Machine plate protection switch | 0-1 | 1 | 0: OFF 1: ON |
| P-59 | Presser foot protection switch | 0-1 | 1 | 0: OFF 1: ON |
| P-60 | Electric /Air-powered | 0-1 |  | 0: Electric 1: Air-powered |
| P-61 | Trimming switch by heeling pedal | 0-7 | 7 | 0: All OFF 1: All ON by manually 2: Semi-auto ON 3: Auto ON 4: Manual and semi-auto ON 5: Semi-auto and auto ON 6: Manual and auto ON 7: Manual and semi-auto and auto ON |
| P-62 | Rotate direction | 0-1 | 0 | Through Hand-wheel direction: 0: Corotation (Clockwise) 1: Reversal (Counter clockwise) |
| P-63 | Language | 0-1 | 1 | 0: English 1: Chinese |
| P-64 | The strength of front sensor | 0%-100% | 80% | Adjustment front sensor strength |
| P-65 | The strength of mid-sensor | 0%-100% | 80% | Adjustment mid-sensor strength |
| P-66 | The strength of back sensor | 0%-100% | 80% | Adjustment back sensor strength |
| P-67 | The sensitivity of back sensor | 0-700 | 600 | In order to adapt different cloth material setting back-sensor receive strength. |
| P-68 | Cutter speed by manually （r/min） | 100-6500 | 1000 | (P-70= Side knife type, this parameter is valid) |
| P-69 | Response time of back sensor（ms） | 0-3000 | 0 | Response time of back-sensor, when sewing such as mesh cloth can adjust this item, then can achieve you want effects. |
| Senior parameter setting ( press P + S key at the same time to turn on the machine access) |  |  |  |  |
| P-70 | Model selection | 0-1 |  | 0: Horizontal knife type 1: Side knife type |
| P-71 | Maximum sewing speed limitation（r/min） |  |  | Maximum sewing speed limitation Note: 900E(Range: 100-6500, Default: 5000) 950E(Range: 100-7000, Default: 5500) |
| P-72 | Manually up positioning adjustment | 0-1439 |  | Through hand-wheel direction, clockwise turn hand-wheel to up position press [S] to save the current value. |
| P-73 | Manually down positioning adjustment | 0-1439 |  | Through hand-wheel direction, clockwise turn hand-wheel to down position then press [S] to save current value. |
| P-74 | Parameter reference | 1-4 | 1 | Use up and down key shift: N1: Control box software version N2: Panel software version N3: Rotation N4: Pedal AD N5: Driver software version |
| P-75 | Pedal type | 0-1 | 0 | 0: Normal pedal 1: Stand type pedal |
| P-76 | Password Setting |  |  |  |
| P-77 | Zero-point angle setting |  |  | Firstly to enter P-48 close the needle goes up as power on, turn off the machine then restart the machine, enter this item press automatic to find zero-point, till machine stops, press key to save, then enter to P-48, turn on the needle goes up as power on. |
| P-78 | Suction bucket type | 0-2 | 1 | 0: With brush 1: Without brush 2: Valve |
| P-79 | One key test | 0-1 | 0 | 0: OFF 1: ON |
| P-80 | The needle bar shield protection switch | 0-1 | 0 | 0: OFF 1: ON |
| P-81 | Electromagnet protection | 0+1 | 1 | 0: OFF 1: ON |
| P-82 | Alarm pieces | 0-9990 | 0 |  |
| P-83 | The number of standby display pieces | 0-1 | 0 | 0: OFF 1: ON |
| P-84 | Piecework mode selection | 0-1 | 0 | 0: Ascending 1: Descending |
| P-85 | Piece trimming times | 0-50 | 1 |  |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem | Status / Measurement |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage) If correct, please replace the control box and inform the factory |
| E02 | Low-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use ) If correct, please replace the control box and inform the factory |
| E05 | Speed control unit connection is abnormal | Turn off the system power supply, please check the speed control unit connector if loose or fall off, Will resume normal after the restart system If you still cannot work normally, please replace the speed control unit and notify the manufacturer. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E10 | Solenoid over-current protection | Turn off the system power supply, please check the solenoid (solenoid valve) connector or solenoid (solenoid valve) is broken or not. |
| E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, renew to normal then restart system. If still not work, please replace the motor and notify the manufacture. |
| E15 | Power module abnormal over current protection | Turn off the system power supply, and then restart the machine. If still not work, please replace the control box and notify the manufacture. |
| E16 | Trimming step motor abnormal | Please check trimming mechanism return correct position or not; please check trimming setting correct or not. |
| E17 | The sewing table safety switch is not in the correct position | Turn off the system power supply, please check machine head is open or not, the machine head switch is damage or not. |
| NC | Front operation panel linked to CPU interface had communication error. | Turn off the machine, check if the operation box if loose or fall off. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |

5. Rear panel connector port outline diagram

5.1 Electric machine type port

5.2Air-powered / side knife machine type port