Safety Instruction

1）Users are required to read the operation manual completely and carefully before installation or operation.

2） The product should be installed and pre-operated by well-trained persons

3） All the instruction marked with sign ，must be observed or executed; Otherwise, bodily injuries might occur.

4） For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection

5） When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250V AC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage. Continue supply 380V power over 5 minutes might damage the fuse F2, burst the electrolytic capacitors and the power module U14 of the main board and even might endanger the person safety.

6） Please don’t operate in direct sun light, outdoors area and where the room temperature is over 45℃or below 0°C.

7） Please avoid operating near the heater at dew area or at the humidity below 10% or above 95%.

8） Please don’t operate in area with heavy dust, corrosive substance or volatile gas.

9） Avoid power cord being applied by heavy objects or excessive force, or over bend.

10）The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

11）All the moving portions must be prevented to be exposed by the parts provided.

12）Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

13）Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine arm.

4. Repairing or doing any mechanical adjustment.

5. Machine is out of work.

14）Repairing and high level maintenance work should only be done by electronic technicians with appropriate training.

15）All the spare parts for repairing work must be provided or approved by the manufacturer.

16）Don’t use any objects to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge.

However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency.

* Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this all a failsafe device.  (Such as residual current breaker).

1     Button Displays and operating instructions

1.1    Key Description

| Name | key | Indicate |
| --- | --- | --- |
| Stitches length setting |  | Increase or decrease the stitches length |
| Pattern Backing-tacking Key |  |  |
| Beautiful seaming |  | Press this key can set used or cancelled beautiful seaming function. |
| Pattern Freedom Sewing Key |  | 1. Short-press to shift Pattern freedom sewing mode selection interface. 2. Long-press to shift Pattern freedom sewing mode edite interface. |
| Pattern Constant-Stitch Sewing Key |  |  |
| Start back-tacking sewing |  | Executions starting back seam B segment or execution starting back seam (A, B segment) 1 Execution starting back seam (A, B segment) 2 times. |
| End back-tacking sewing |  | Execution terminates back seam C segment or execution terminates back seam (C, D segment) 1 Execution terminates back seam (C, D segment) 2 time. |
| Consecutive reverse sewing |  | Press this key can set consecutive reverse sewing function. |
| Free Sewing |  | Press this key can set free sewing function. |
| Multi-section Sewing |  | Continuous press the key, the function will in four section sewing, seven section sewing, eight section sewing and user-defined multi-section sewing between the switch. Long-press this key can enter P72, P92, P128. |
| Auto Function |  | 1. Press this key can set used or cancelled automatic function for constant stitch sewing. 2. Long-press can restore factory setting. |
| Thread trimming |  | 1. Press this key can set used or cancelled thread trimming function. 2. Long-press can enter counter interface. |
| Automatic presser foot lifting setting |  | Press this key it can execute the cycle shift of automatic presser foot lifting, automatic presser foot lifting after pause, automatic presser foot lifting after trimming and presser foot lifting is inactive. |
| Manual Presser Foot Lifting Setting |  | Press this key can lift or lower presser foot. |
| Needle stop position selection |  | Short-press to shift halfway stop needle UP/Down position |
| Slow launch setting |  | 1. Press this key can set used or cancelled slow launch function. 2. Long-press to save the current parameter value as factory parameter value. (only in the parameter setting interface) |
| Lift or fill up needle |  | Press this key can lift or fill up half a needle. |
| Function parameter edit |  | 1. Press this key can enter or exit the user parameter setting interface. 2. Long-press can switch to password input interface. Press S key to confirm and enter the advanced parameter setting interface. |
| - Confirm key |  |  |
| Parameter increase |  | Increase the parameter. |
| Parameter decrease |  | Decrease the parameter. |
| Speed increase |  | Increase the sewing speed. |
| Speed decrease |  | Decrease the sewing speed. |

1.2 Auxiliary function

1.2.1 Debugging Mode

Long-press enter to Debugging parameter (P72、P92、P128) interface, short-press 4th column +、- key to adjust parameter, short-press S key to enter corresponding parameter setting interface.

P72 Up position adjustment: At the parameter setting interface, counterclockwise rotate the hand wheel, the displayed value will change with the rotation of the hand wheel, short-press S key can save the current position (value) as the up position.

P92 Encoder start angle: At the parameter setting interface, short-press stitch correction key, the motor will running automatically, read and display encoder start angle, short-press S key can save. The factory default have setting, please don’t modify randomly, if modify randomly will caused control box and motor abnormal or damage.

P128 trimming function testing: At the parameter interface, short-press trimming key, it will according to the setting angel take action to trimming.

1.2.2 Pattern Freedom Sewing Mode

On the main interface short-press Patt key to shift pattern freedom sewing mode selection interface “n0”（n0 is normal freedom sewing mode），short-press 4th column +、-key to adjust require pattern no, short-press S key to confirm then enter corresponding pattern freedom sewing mode to sewing.

1.2.3 Pattern Freedom Sewing Edite

On the main interface long-press Patt key to shift pattern freedom sewing mode edite interface“n-01 01”，short-press 4th column +、-key to adjust n01-n09 pattern no, short-press  key adjust 01-10 segment number, adjust to setting pattern no, segment no, short-press S key to confirm, shift to corresponding pattern no edite interface “01 1 3.0”， short press 1st column or 2nd column +、- key to adjust 00-99 stitches,  short-press 4th column +、- key 1-9 repeat times, short-press  key to adjust needle distance, after finished setting short-press S key to confirm. continuous to short-press P key can exit to interface.

1.2.4 Restore Factory Setting

On the main interface long-press one-shot sewing key, restore factory setting.

2     Parameter List

| Parameter Item | Parameter Function | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed（r/S ） | 100-3700 | 3500 |  |
| P02 | Speed curve adjustment（%） | 10-100 | 80 | The speed controller climbing slope setting. The large of slope value = the fast of the speed. The small of slope value = the slow of the speed. |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle Stops at Up Position DN: Needle Stops at Down Position |
| P04 | Start back-tacking speed（r/S ） | 100-3200 | 2000 |  |
| P05 | End back-tacking speed（r/S ） | 100-3200 | 2000 |  |
| P06 | Bar-tacking speed（r/S ） | 100-3000 | 2000 |  |
| P07 | Soft start speed（r/S ） | 200-1500 | 400 |  |
| P08 | Stitch numbers for soft start | 0-99 | 2 |  |
| P09 | Automatic constant-stitch sewing speed（r/S ） | 200-4000 | 3500 | Constant-Stitch sewing speed adjustment when one shot signal us active. |
| P10 | Automatic end back-tacking sewing | ON/OFF | ON | ON: After last seam of constant stitch sewing, it will automatic execute the End Back-Tacking sewing function. When turn on, the Stitch-Correction is invalid. OFF: Automatic End Back-Tacking will not execute after last seam, but can be done by pedal manually. |
| P11 | Back-tacking mode selection | J/B | J | Press Back-Tacking switch by hand: J: JUKI Mode (it will activate when machine is stopped or running) B: BROTHER mode (It will active the reverse solenoid when machine is running, and it will active stitch-correction action when machine is stopped) |
| P12 | Start back-tacking mode selection | 0-1 | 1 | 0: Manual, Pedal- control can stop and start. 1: Automatic, One shot to pedal, it can automatic execute back-tacking action. |
| P13 | Mode selection at the end of start back-tacking | CON/STP | CON | CON: At the end of Start Back-Tacking, machine continues sewing if pedal pressed or START signal on ( standing operation) STP: At the end of Start Back-Tacking, machine stops |
| P14 | Soft start | ON/OFF | OFF | ON: Soft start function is turn on. OFF: Soft start function is turn off. |
| P15 | Stitch correction mode | 0-4 | 2 | Range (0-4) stitch correction mode: 0: Half stitch 1: One stitch 2:Continuous correct half stitch 3:Continuous correct one stitch 4. Continuous correct stitch and quickly stop machine. |
| P17 | Automatic count mode selection | 0-1 | 1 | 0: P41 parameter value increase auto-count 1: P41 parameter disable auto-count. |
| P20 | Mode selection for end back-tacking | 1-2 | 1 | 2: Manual, Pedal- control can stop and start. 1: Automatic, One shot to pedal, it can automatic execute back-tacking action. |
| P21 | The position of the pedal for running | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for lifting presser foot | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P27 | Pattern seam setting |  |  |  |
| P28 | Mode selection for bar-tacking | 0-1 | 1 | 0: Manual, Pedal- control can stop and start. 1: Automatic, One shot to pedal, it can automatic execute bar-tacking action. |
| P29 | After trimming braking output | 0-50 | 20 |  |
| P32 | Stitch balance for bar-tacking | 0-200 | 167 |  |
| P34 | Mode selection for constant-stitch sewing | A/M | A | A: One shot to pedal, it will automatic execute Constant-Stitch M: Pedal-controlled and motor can stop arbitrarily |
| P36 | Tension release/ big clamp device shift | 0-1 | 1 | 0: Thread tension release function off, big clamp device function on. 1: Thread tension release function on, big clamp device function off. |
| P37 | Wiper, clamp function selection (Pressure setting of the thread clamp device） | 0-11 | 3 | 0: No Action 1: Wiper/ wiper thread action 2-11: Thread Clamp action and the pressure gradually increased） |
| P38 | Trimmer function selection | ON/OFF | ON | ON: Trimmer Valid OFF: Trimmer Invalid |
| P39 | Presser foot up / down at intermediate stop | UP/DN | DN | UP: ON DN: OFF |
| P40 | Presser foot up / Down after trimming | UP/DN | DN | UP: ON DN: OFF |
| P41 | Display the sewing finished quantity | 0-9999 |  | Valid for enter sewing pieces display interface, automatic save; long press minus key to reset the counter. |
| P42 | Information display |  | N-01 | Press S key to enter, press P key to exit NO1 Control box version serial number NO2 Operation panel version number. NO3 Sewing Speed NO4 Pedal AD value NO5 Up positioning angle NO6 Down positioning angle NO7 Bus-bar voltage AD value NO8 Error code record NO9 Running time NO10 Scissors position sensor NO11 Oil level sensor NO12 Sensor by knee moving AD NO13 Presser foot lift height sensor AD NO14 Stepping plate abnormal record |
| P43 | Setting direction of motor rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |
| P44 | Brake force | 1-45 | 16 |  |
| P46 | Reverse angle function selection after trimming | ON/OFF | OFF | ON: ON OFF: OFF |
| P47 | Adjustment of reverse angles after trimming | 50-200 | 160 | Adjusting at reverse direction after trimming |
| P48 | Low (Positioning) speed (rpm) | 100-500 | 210 | Min. speed limited adjustment |
| P49 | Trimming speed (rpm) | 100-350 | 300 | Adjusting trimming speed |
| P52 | Delay motor start, protect presser foot lift lay down time | 10-990 | 150 | Step down time delay start time to cooperate with automatic lifting foot lay down confirmation |
| P53 | Cancel foot lifting when half-heeling the pedal | 0-2 | 1 | 0: OFF 1: Pedal heeling and pedal half heeling with foot lifting function 2: Pedal heeling with foot lifting function, Pedal half heeling without foot lifting function. |
| P54 | Trimming time（ms） | 10-990 | 200 |  |
| P55 | Thread wiping/Thread sweeping time | 10-990 | 380 |  |
| P56 | Needles goes up automatically as power turned on | 0-1 | 0 | 1: Power turned on, needle goes up position automatically 0: No Function |
| P57 | The presser foot protection time（s） | 1-990 | 15 | The presser foot lift exceeds the set value automatically disconnect protection time |
| P58 | Up position adjustment | 0-1439 | 80 | Up Position Adjustment The needle will advance stop when the value decreased. The needle will delay stop when the value increased. |
| P59 | Down position adjustment | 0-1439 | 780 | Down Position Adjustment The needle will advance stop when the value decreased. The needle will delay stop when the value increased. |
| P60 | Testing speed（r/S ） | 100-3700 | 3500 |  |
| P61 | Testing A | OFF/ON | OFF | Keeping run testing mode |
| P62 | Testing B | OFF/ON | OFF | Full |
| P63 | Testing C | OFF/ON | OFF | Without synchonizer |
| P64 | Running time of testing B and C | 1-250 | 20 |  |
| P65 | Stop time of testing B and C | 1-250 | 20 |  |
| P66 | Machine protection switch testing | 0-2 | 0 | 0: Disable 1: Testing zero signal； 2: Testing positive signal |
| P67 | Trimming protection switch testing | ON/OFF | OFF | OFF: Disable ON: Enable |
| P69 | Pattern freedom Sewing Speed | 100-3000 | 2500 |  |
| P71 | Stitch correct needle distance | 0-164 | 76 |  |
| P72 | Upper needle stop position adjustment | 0-1439 |  | Adjustment up needle stop position, the value will changed according to the hand-wheel position, press S key to save current position(value) as up needle stop position. |
| P73 | Down needle stop position adjustment | 0-1439 |  | Adjustment down needle stop position, the value will changed according to the hand-wheel position, press S key to save current position(value) as down needle stop position. |
| P75 | Back-tacking stitch length compensation | 0-100 | 50 |  |
| P78 | Clamp start action angle | 10-359 | 100 |  |
| P79 | Clamp end angle | 10-359 | 270 |  |
| P80 | The angle of trimming first infeed | 0-1439 | 1035 |  |
| P82 | The angle of trimming second infeed | 0-1439 | 1435 |  |
| P83 | The stopping distance of trimming | 10-50 | 20 |  |
| P86 | The distance of up and down position | 100-4000 | 700 | Each 4 value as 1 degree |
| P87 | The thread wiping/ sweep back delay times | 10-990 | 50 |  |
| P88 | Braking distance | 10-50 | 30 |  |
| P92 | Encoder start angle |  |  | Read encoder starting angle, the factory default have setting, please don’t modify randomly. (Parameter value can’t manually modify, if modify randomly will caused control box and motor abnormal or damage) |
| P93 | Half step delay times(ms) | 10-990 | 200 |  |
| P99 | Big clamp device current | 0-200 | 60 |  |
| P100 | Big clamp device start action time | 0-990 | 120 |  |
| P101 | Thread tension release start angel | 1-359 | 30 |  |
| P102 | Thread tension release end angel | 1-359 | 180 |  |
| P103 | Big clamp device start action current | 0-300 | 5 |  |
| P105 | Patter seam shift | 0-9 | 0 | 0: OFF 1~9: |
| P109 | The delay time before thread wiping/ sweep (ms) | 5-990 | 5 | The interval time of thread wiping/ sweep action after finding up needle stop position |
| P110 | Trimming back delay time (ms) | 60-990 | 60 |  |
| P112 | The delay time before hook thread with clamp function without any thrum (ms) | 0-990 | 100 |  |
| P113 | The hook thread action time with clamp function without any thrum (%) | 0-990 | 30 |  |
| P114 | The return back delay time of hook thread with clamp function without any thrum (%) | 0-990 | 30 |  |
| P115 | Duty cycle for Hook thread with clamp function without any thrum (ms) | 1-100 | 30 |  |
| P116 | The suction time for clamp function without any thrum (ms) | 0-5000 | 1000 |  |
| P117 | Duty cycle for wiper thread without any thrum | 0-100 | 30 |  |
| P120 | Oil level alarm switch | 0-1 | 0 | 0: OFF； 1: ON |
| P122 | Sensor by knee start AD value | 0-1023 | 600 |  |
| P125 | Intermediate presser foot lift highness compensation | 0-160 | 80 | The 80 as 0 compensation, the large value as positive compensation, the presser foot lift will be raise; The small value as negative compensation, the presser foot lift will be decline; |
| P128 | Trimming step motor function testing |  |  | NO06 Trimming test NO07 trimming step motor return to zero point |
| P129 | Reverse-feed motor zero point correction | 0-200 | 50 |  |
| P130 | Trimming-feed motor zero point correction | 0-200 | 50 |  |
| P131 | Normal stitches length | 0-164 | 76 | The stitches length under normal sewing mode |
| P132 | Compactness stitches length | 0-164 | 52 |  |
| P133 | Widen stitches length | 0-164 | 100 |  |
| P134 | The second trimming distance (after action) | 0-200 | 145 |  |
| P136 | The highest of presser foot lift highness | 0-150 | 135 |  |
| P137 | The first trimming distance (before action) | 0-200 | 100 |  |
| P140 | Trimming retract speed | 20-250 | 100 |  |
| P141 | Trimming speed 2 | 20-400 | 100 | Second trimming speed |
| P142 | Trimming speed 1 | 20-400 | 200 | First trimming speed |
| P146 | Presser foot speed | 20-300 | 200 |  |
| P153 | Beautiful seam stitch length | 0-60 | 15 |  |
| P154 | Beautiful seam speed | 100-1000 | 500 |  |
| P155 | Machine head forward stitch correction function shift | 0-1 | 0 | 0: Forward stitch correction function 1: Condense-stitch function |
| P159 | Beautiful seam direction | 0-1 | 0 | 0: positive direction 1: negative direction |
| P160 | Beautiful seam stitches | 0-12 | 0 | Setting 0 as beautiful seam function turn off. |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3  Error Code List

|  | Error Code | Problem | Status / Measurement |
| --- | --- | --- | --- |
| E01 |  | Over-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage. ) If correct, please replace the control box and inform the factory |
| E02 |  | Low-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use. ) If correct, please replace the control box and inform the factory |
| E03 |  | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 |  | Speed control unit connection is abnormal | Turn off the system power supply, please check the speed control unit connector if loose or fall off, Will resume normal after the restart system If you still can not work normally, please replace the speed control unit and notify the manufacturer. |
| E07 |  | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E08 |  | Function step motor abnormal | Check P42 parameter N14 value, 0: communicate abnormal; 2: back-tacking step over current 4: back-tacking step start can’t find original point. 16: trimming step over current 32: trimming step can’t find original point, shut off system power, observe step motor whether stuck or not. If stuck the first except the machine head machinery fault. If normal please check the step motor connector if loose or fall off ,renew to normal then restart system. If still not work, please replace the motor and notify the manufacture. |
| E10 |  | Solenoid over-current protection | Turn off the system power supply, please check the solenoid (solenoid valve) connector or solenoid (solenoid valve) is broken or not. |
| E09 E11 |  | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 |  | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, renew to normal then restart system. If still not work, please replace the motor and notify the manufacture. |
| E15 |  | Power module abnormal over current protection | Turn off the system power supply, and then restart the machine. If still not work, please replace the control box and notify the manufacture. |
| E16 |  | Trimming step motor abnormal | Please check trimming mechanism return correct position or not; please check trimming setting correct or not. |
| E17 |  | The sewing table safety switch is not in the correct position | Turn off the system power supply, please check machine head is open or not, the machine head switch is damage or not. |
| E20 |  | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| oil |  | Low oil | Turn off the machine, check if the oil is too little. To fill it up and turn on. If there are still errors, please replace the control box and call for customer service. |

4  Port Outline Diagram: