Safety Instruction

1)  Users are required to read the operation manual completely and carefully before installation or operation.

2） All the instruction marked with sign ，must be observed or executed; otherwise, bodily injuries might occur.

3） The product should be installed and pre-operated by well trained persons

4） For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection

5） When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250V AC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage. Continue supply 380V power over 5 minutes might damage the fuse F2, burst the electrolytic capacitors and the power module U14 of the main board and even might endanger the person safety.

6） Please don’t operate in direct sun light, outdoors area and where the room temperature is over 45℃or below 5°C.

7） Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

8） Please don’t operate in area with heavy dust, corrosive substance or volatile gas.

9） Avoid power cord being applied by heavy objects or excessive force, or over bend.

10）The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

11）All the moving portions must be prevented to be exposed by the parts provided.

12）Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

13）Turn off the power before the following operation：

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine arm.

4. Repairing or doing any mechanical adjustment.

5. Machine is out of work.

14）Repairing and high level maintenance work should only be done by electronic technicians with appropriate training.

15）All the spare parts for repairing work must be provided or approved by the manufacturer.

16）Don’t use any objects to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge.

However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency.

* Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this all a failsafe device.  (Such as residual current breaker).

1 Button Displays and Operating Instructions

1.1. Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Start Back-Tacking Sewing |  | If click, execution starting back seam B segment or execution starting back seam (A, B segment) 1 Execution starting back seam (A, B segment) 2 times or turn off. |
| End Back-Tacking Sewing / Needle Stop Position Selection |  | 1. If click, execution terminates back seam C segment or execution terminates back seam (C, D segment) 1 Execution terminates back seam (C, D segment) 2 times or turn off. 2. If long-press, shift to the needle stop position after shift the sewing mode (Up position / Down position). |
| Clamp Function Setting |  | If click, shift to clamp function (The corresponding icon lights, thread nipper function is turned on / the corresponding icon is not lit, no thread nipper function). |
| Press Foot Lifting after Trimming / Press Foot Lifting after Pause |  | 1. If click, when icon is lighting, set used or cancelled press foot lifting after trimming function. 2. If long-press, when icon is lighting, set used or cancelled press foot lifting after pause function. |
| Free Sewing / Thread Trimming |  | 1. If click, shift to free sewing 2. If long-press, set used or cancelled thread trimming function. |
| Continuous Back Seam Shortcuts / Multi-section Set Needle Shortcuts |  | 1. If click, shift to continuous back seam model. 2. If long-press, shift to multi-section set needle shortcuts. |
| Function Parameter Edit |  | In normal mode, Long-press [P] key to enter User Parameter Mode. Press and hold [P] key, then turn on the power to enter Technician Parameter Mode. |
| Setting Speed Increase / Parameter Increase |  | 1. Speed adjust upper key 2. Parameter value increase key |
| Setting Speed Decrease/ Parameter Decrease |  | 1. Speed adjust lower key. 2. Parameter value decrease key. |
| Left Selection Parameter Content |  | Left selection parameter content |
| Right Selection Parameter Content |  | Right selection parameter content |
| Restore Factory |  | Long-press 3 sec can restore factory setting. |

2 Control System after Installation Debugging

2.1 Integrate Control Box Debugging

2.1.1： Enter to User Parameter Mode and Save.

Long press  key enter to User Parameter Mode. Press  or  to change parameter item, remove cursor, the corresponding digit value is flashing, then press or to modify this digit value, press key, the whole parameter value is flashing, save the parameter. Press  key again to exit.

2.1.2： Enter to Technician Parameter Mode and Save

Hold on  key to turn on the machine (show P27) enter to technician parameter mode.  Press or  change parameter item, remove cursor, the corresponding digit value is flashing, then press or to modify this digit value, presskey, the whole parameter value is flashing, save parameter. Press key again to exit.

2.2   Soft start:

P14 adjustment in user parameters, Parameter is ON, Parameter is Off (The default value).

2.3   Adjustment speed:

Upper key and lower key to adjustment speed.  Press to accelerate speed, press  to decelerate, each press the adjust vale is 50. For temporary adjust speed, the parameter in accordance with internal parameter of speed adjustment 01. 01 parameter is limited 4000

2.4 Adjust up needle position setting

3 Parameter Definition

3.1 User Parameter (Turn on the machine long-press P key for 3 sec. to enter)

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum Sewing Speed（rpm） | 200-5000 | 4000 | Maximum speed of machine sewing |
| P02 | Set Accelerated Curve（%） | 10-100 | 80 | Set the acceleration slope |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle Stops at Up Position DN: Needle Stops at Down Position |
| P04 | Start Back-tacking Speed（rpm） | 200-3200 | 1800 |  |
| P05 | End Back-tacking Speed（rpm ） | 200-3200 | 1800 |  |
| P06 | Bar-tacking Speed（rpm） | 200-3200 | 1800 |  |
| P07 | Soft Start Speed（rpm ） | 200-1500 | 400 |  |
| P08 | Stitch Numbers for Soft Start | 0-99 | 2 |  |
| P09 | Automatic Constant-stitch Sewing Speed（rpm） | 200-4000 | 3500 | Speed adjustment for automatic constant-stitch sewing |
| P10 | Automatic End Back-tacking Sewing | ON/OFF | ON | ON: After last seam of constant stitch sewing, it will automatic execute the End Back-Tacking sewing function. When turn on, the Stitch-Correction is invalid. OFF: Automatic End Back-Tacking will not execute after last seam, but can be done by pedal manually. |
| P11 | Back-tacking Mode Selection | J/B | J | J: JUKI Mode (Press TB switch will activate reverse solenoid when either machine is stopped or running). B: BROTHER Mode (Press TB switch will activate reverse solenoid only when machine is running). |
| P12 | Start Back-tacking Running Mode Selection | A/M | A | A: Automatically perform actions M: Could be arbitrarily stop and start |
| P13 | Ending mode of Start Back-tacking | CON/STP | CON | CON：Start Back-Tacking is completed automatically continued for next action. STP：After the number of stitches is completed, stop automatically. |
| P14 | Slow Start Function Selection | ON/OFF | OFF |  |
| P15 | Mending stitch mode | 0-2 | 2 | 0: half stitch; 1: one stitch; 2: continuous half stitch. |
| P16 | Back-tacking Speed Limit | 0-3200 | 0 | When the parameter is 0, the speed limit function is OFF. |
| P18 | Start Back-tacking Compensation 1 | 0-200 | 131 | Stitch Balance for end back-tacking A section, 0~200 action gradually delay; the large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start Back-tacking Compensation 2 | 0-200 | 158 | Stitch Balance for end back-tacking B section, 0~200 action gradually delay; the large value, the longer of the B section first stitch. |
| P20 | End Back-tacking Running Mode Selection | A/M | A | A: Automatically perform actions M: Could be arbitrarily stop and start |
| P21 | The Position of the Pedal for Running | 30-1000 | 520 |  |
| P22 | The Position of the Pedal for the Stop | 30-1000 | 420 |  |
| P23 | The position of the Pedal for Lifting Presser Foot | 30-1000 | 260 |  |
| P24 | The position of the Pedal for Thread Trimming | 30-500 | 130 |  |
| P25 | End Back-tacking Compensation 3 | 0-200 | 131 | Stitch Balance for end back-tacking C section, 0~200 action gradually delay; the large value, the shorter of the C section first stitch. |
| P26 | End Back-tacking Compensation 4 | 0-200 | 158 | Stitch Balance for end back-tacking D section, 0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P28 | Bar-tacking Running Mode Selection | A/M | A | A: Automatically perform actions M: Could be arbitrarily stop and start |
| P29 | The Rate of Thread Trimming Stop | 1-50 | 24 |  |
| P32 | Bar-tacking Compensation 5 | 0-200 | 131 | Stitch Balance for start back-tacking A(C) section, 0~200 action gradually delay; the large value, the longer of the A(C) section last stitch; the shorter of the B (D) section first stitch. |
| P33 | Bar-tacking Compensation 6 | 0-200 | 158 | Stitch Balance for start back-tacking B(D) section, 0~200 action gradually delay; the large value, the longer of the B(D) section last stitch; the shorter of the C section first stitch. |
| P34 | Constant-stitch Sewing Running Mode Selection | 0-1 | 1 | 1: Automatically perform actions 0: Could be arbitrarily stop and start |
| P35 | Counting Mode Selection | 0-1 | 0 | 0: Manual counting 1: Automatic counting |
| P36 | Thread Slacking Function Selection | 0-11 | 0 | 0: OFF 1-11:The strength of thread slacking |
| P37 | Automatic Thread Wiping Function / Automatic Thread Clamping Function Selection | 0-11 | 8 | 0: OFF 1: Automatic thread wiping Function 2-11: Automatic thread clamping Function and the strength of automatic thread clamping. |
| P38 | Automatic Thread Trimming Function Selection | ON/OFF | ON | ON: ON OFF: OFF |
| P39 | Automatic Presser Foot Lifting When Pause Function Selection | UP/DN | DN | UP：ON DN：OFF |
| P40 | Automatic Presser Foot Lifting after Trimming Function Selection | UP/DN | DN | UP：ON DN：OFF |
| P41 | Counter Display | 0-9999 |  | Display the quantity of finished sewing piece. |
| P42 | Information Display | N01-N07 | N01 | N01 The control system version number N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(up position) N06 The mechanical angle(down position) N07 Busbar voltage AD N12 Kneeling device position sensor AD |
| P43 | Direction of Motor Rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |

3.2 Technician Parameter (Press P key to enter)

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P44 | The Rate of Normal Stop | 1-50 | 16 | Adjustment of efforts while machine stop. |
| P45 | Back-tacking Operation Duty Cycle (%) | 1-50 | 30 | Back-tacking operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P46 | Motor Stop with A Reverse Angle after Trimming Function | ON/OFF | OFF | ON: ON OFF: OFF |
| P47 | Adjust the Reverse Angle When Motor Stop after Trimming | 50-200 | 160 | Adjusting at reverse direction after trimming. |
| P48 | The Minimum Speed Limit（rpm ） | 100-500 | 210 | Adjust the minimum speed |
| P49 | Thread Trimming Speed（rpm） | 100-500 | 250 | Adjust thread trimming speed |
| P50 | Toot Lifter Operation Time to Full Output（ms） | 10-990 | 200 |  |
| P51 | Presser Foot Lifter Operation Duty Cycle (%) | 10-40 | 25 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Motor Starting Delay Time（ms） | 10-990 | 120 | Delay the start time, with automatic presser foot down. |
| P53 | Half Back Pedaling Lifting Presser Foot Function to Cancel | ON/OFF | OFF | ON: Function to cancel OFF: Half back pedaling lifting presser foot |
| P54 | Thread Trimming Action Time（ms） | 10-990 | 200 | Completion of the thread trimming action required time. |
| P55 | Thread Wiping Action Time（ms） | 10-990 | 30 | Completion of the thread wiping action required time. |
| P56 | Power On and Positioning | 0-2 | 2 | 0: always not to find the up needle position 1: always to find the up needle position 2: If the motor is not into the up needle position, it will find the up needle position |
| P57 | Hold Time of Presser Foot Lifter（s） | 1-120 | 10 | Forced shut-down after hold time. |
| P58 | Up Needle Position Adjustment | 0-1799 | 460 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down Needle Position Adjustment | 0-1799 | 1360 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing Speed（rpm ） | 100-4000 | 2000 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | After setting A, continuous running at the speed of 【P60.TV】 |
| P62 | Testing B | ON/OFF | OFF | Option of Testing B, after setting press 【P60. TV】to set the speed execute the cycle of Start – Sewing –Stop - Trimming |
| P63 | Testing C | ON/OFF | OFF | Option of Testing C, after setting press【060.TV】to set the speed execute the cycle of Start – Sewing –Stop without positioning function |
| P64 | Running Time of Testing B and C | 1-250 | 20 | Setting running time of testing B and C |
| P65 | Stop Time of Testing B and C | 1-250 | 20 | Setting stop time of testing B and C |
| P66 | Machine Protection Switch Selection | 0-2 | 1 | 0：Disable 1：Testing zero signal 2：Testing positive signal |
| P71 | Presser Foot Release Operation Duty Cycle (%) | 0-40 | 2 | The buffer force of presser foot release operation. |
| P72 | Up Needle Position Adjustment | 0-1799 |  |  |
| P73 | Down Needle Position Adjustment | 0-1799 |  |  |
| P76 | Back-tacking Operation Time to Full Output（ms） | 10-990 | 250 | Back tacking start action, full output action times. |
| P78 | The Start Angle of Thread Clamping | 10-150 | 110 | Positioning angle when start clipping. |
| P79 | The Stop Angle of Thread Clamping | 160-359 | 280 | Positioning angle when finish clipping. |
| P80 | The Start Angle of Thread Trimming | 5-359 | 18 | Shear line infeed id angle setting(under defined as 0°) |
| P82 | The Stop Angle of Thread Trimming | 5-359 | 172 | Feeder back angle of thread trimming setting Down position as 0°. |
| P83 | The Stopping Distance | 10-100 | 33 |  |
| P84 | The Angle of the Motor Start to Increase Torque When Sewing | 0-359 | 9 |  |
| P85 | The Angle of the Motor Stop to Increase Torque When Sewing | 0-359 | 57 |  |
| P86 | The Distance of Up and Down Position | 100-1700 | 900 |  |
| P88 | The Thread Sweep Back Times | 10-990 | 30 |  |
| P89 | Alternating Current (AC) Over-voltage Value Set | 500-1023 | 880 |  |
| P92 | Correct the Electrical Angle of Motor | 0-479 | 445 | Reading the initial angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P99 | The Voice Switch | 0-2 | 1 | 0: OFF 1: Chinese 2: English |
| P108 | Boot up voice selection | 0-2 | 0 | 0: With welcome speech and key tone 1: With welcome speech but without key tone 2: With key tone but Without welcome speech |
| P109 | The Delay Time Before Thread Sweep | 5-990 | 5 |  |
| P110 | Trimming Back Delay Time | 10-990 | 80 | It only works when thread sweep off, if thread sweep on it will execute delay time before thread sweep. |
| P113 | The Periodic Signal of Solenoid Full Output | 0-100 | 95 |  |
| P117 | The Delay Time of the Presser Foot Release Damping Force (ms) | 0-990 | 12 |  |
| P119 | Electromagnet Over-current Protection Selection | 0-1 | 1 | 0: OFF 1: ON |
| P120 | Oil Level Alarm Switch |  |  | 0: OFF 1: ON |
| P124 | Factory Reset |  |  |  |
| P125 | The Number of Times For Piecework Trimming | 1-9 | 1 | Trimming number of times as setting number of times, piecework add 1. |
| P126 | Bobbin Total Stitches Setting | 0-9999 | 2000 | When the P127 turn on, the machine working to setting stitches then alarm. |
| P127 | Count Stitches Mode Selection | 0-2 | 0 | 0: OFF 1: Count full reset but still running 2: Count full to stop machine |
| P130 | Tighter Seam / Short-thread | 0-3 | 2 | 0: OFF 1: Short-thread 2: Tighter sea 3: Tighter sea + short-thread |
| P131 | Tight Seam Speed | 100-1800 | 1000 |  |
| P132 | Tight Seam Stitches | 0-10 | 2 |  |
| P133 | Tight Seam Solenoid Action Angle | 20-330 | 115 | Tight seam solenoid forward with the lower position start action angle. |
| P136 | Start Back-tacking the First Stitch Speed Limit | 0-1500 | 600 | Invalid for parameter value as 0 |
| P137 | Start Back-tacking the Second Stitch Speed Limit | 0-1500 | 0 | Invalid for parameter value as 0 |
| P138 | Start Back-tacking the Third Stitch Speed Limit | 0-1500 | 0 | Invalid for parameter value as 0 |
| P139 | Thread Slacking When Start Sewing Function Selection | 0-1 | 0 | 0: OFF 1: ON |
| P141 | The Start Angle of Function that Thread Slacking When Start Sewing | 1-359 | 1 | The start angle of function that thread slacking when start sewing.(as upper needle position angle) |
| P142 | The Stop Angle of Function that Thread Slacking When Start Sewing | 1-359 | 35 | The stop angle of function that thread slacking when start sewing. |
| P156 | Presser Foot Lifter Operation Duty Cycle When Start Sewing | 0-100 | 30 | Close the function when the parameter value is 0. |
| P157 | The Start Angle of Function that Presser Foot Lifting When Start Sewing | 1-359 | 1 | The start angle of function that presser foot lifting when start sewing. |
| P158 | The Stop Angle of Function that Presser Foot Lifting When Start Sewing | 1-359 | 100 | The stop angle of function that presser foot lifting when start sewing. |
| P159 | Start Back-tacking Presser Foot Output Function Switch | 0-1 | 0 | 0: OFF 1: ON |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage.(Or whether exceed the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage.(Or whether under the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E03 | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 | Pedal connection error | Turn off the machine, please check if the pedal in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E08 | Manual reverse feeding operation more than 15 seconds | Back-Tacking electromagnet response time is too long; you have only to restart the machine. If there are still errors E-08 after restart the machine, check if the manual reverse feeding switch is damaged. |
| E10 | Solenoid over-current | Turn off the machine, please check the related circuit is shorted or damaged. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E15 | Power supply module abnormal over-current | Turn off the machine, then turn on again. If there are still errors, please replace the control box and call for customer service. |
| E17 | Machine protection switch is not in the correct position | Turn off the machine, check if the machine is turn over, or the Mercury Switch is shifted or damaged. |
| E20 | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E25 | Disconnection-protection | Turn off the power, please kindly check the sewing machine thread is whether disconnected, and make sure the thread connect well to restart the system. If still can't normal work, please replace the disconnect thread sensor and inform the after-sales service. |

5 Port Diagram

5.1 Port Description

16P Function Port

12P Function Port

Motor Encoder

Speed Control unit

Motor Power

5.2 16P function port description

1. Thread trimming electromagnet: 1、9

2. Thread clamping (thread wiping) electromagnet: 2、10

3. LED Light: 4（DGND）、12（+5V）

4. Reverse stitch key: 5（signal）、13（DGND）

5. Back-tacking electromagnet: 6、14

6. Mending stitch key: 7（signal）

5.3 12P function port description

1. Presser foot electromagnet: 6、12

2. Tight seam solenoid: 5、11