QD3621系列控制系统说明书V1.0.01

No. 10404027

安全指示

- 在安装或使用本产品前，使用者必须详细阅读本操作手册。

- 本产品须由受过正确训练的人员来安装或操作。安装作业时必须关闭所有电源，切记不可带电操作。

3） 所有标有符号的指示，必须特别注意并按照说明书上的执行，以免造成不必要的损害。

4） 为安全起见，禁止以延长线作电源座供应二项以上的电器产品使用。

5） 在连接电源线时，必须确定工作电压低于AC 250V，且符合本产品标识中规定的额定电压值。

※注意  ：电控箱电源规格如为AC220V时，请勿插接至AC380V的电源插座上，否则将出现异常且电机无法动作。此时请立即关闭电源开关，重新检查电源。持续供应380V超过五分钟以上，将可能烧损电控箱内器件，而危及人身安全。

6） 请不要在日光直接照射的场所、室外及室温45℃以上或0℃以下的场所操作。

7） 请不要在暖气（电热器）旁、有露水的场所及在相对湿度10%以下或90%以上的场所操作。

8） 请不要在灰尘多的场所、具有腐蚀性物质的场所及有挥发性气体的场所操作。

9） 请注意所有电源线、信号线、接地线等接线时不要受压或过度扭曲，以确保使用安全。

10）电源线的接地端须以适当大小的导线和接头连接到生产工厂的系统地线，此连接必须被永久固定。

11）所有可转动的部分，必须以所提供的零件加以防范露出。

12）在安装完成第一次开电后，先关闭切线功能以低速操作缝纫机并检查转动方向是否正确、运转是否稳定。

13）在进行以下操作前，请先关闭所有电源：

1.在控制箱与马达上插拔任何连接插头时。

2.穿针线时。

3.翻抬缝纫机机头时。

4.修理或做任何机械上的调整时。

5.机器闲置不用时。

14）修理或高层次的保养工作，仅能由受过训练的机电技师来执行。

所有维修用的零件，须由本公司提供认可，方可使用。

15）使用本产品请远离高频电磁波和电波发射器等，以免所产生的电磁波干扰伺服驱动装置而发生误动作。

16）请不要以不适当物体来敲击或撞击本产品及各装置。

保修期限

本产品保修期限为购买日期起一年内或出厂月份起两年内。

保修内容

本产品在正常情况使用且无人为操作失误的前提下，于保修期间无偿为客户维修使能正常操作。

但以下情况于保修期间将收取维修费用：

1.不当使用包括误接高压电源、将产品移做其它用途、自行拆卸、维修、更改、或不依规格范围使用、进水进油及插入异物于本产品。

2.火灾、地震、闪电、风灾、水灾、盐蚀、潮湿、异常电压及其它天灾或不当场所造成的损害。

3.客户购买后摔落本产品，或客户自行运输（或托付运输公司）造成的损害。

* 本产品在生产及测试上皆尽最大努力和严格控制使其达到高品质及高稳定的标准，但外部的电磁或静电干扰或不稳定的供应电源，仍可能对本产品造成影响或损害，因此操作场所的接地系统一定要确实做好，并建议用户安装故障安全防护装置（如漏电保护器）

1 液晶显示说明

1.1 液晶显示

1.2 按键说明

| 名称 | 按键 | 注明 |
| --- | --- | --- |
| 起始回缝键 |  | 若点击，切换起始回缝（A、B段）1次、起始回缝（A、B段）2次或关闭。 |
| 终止回缝键 |  | 若点击，切换终止回缝（C、D段）1次、起始回缝（C、D段）2次或关闭。 |
| 夹线功能键/停针位键 |  | 1、若点击，设定使用或取消夹线功能。 2、若长按，切换缝制后机针的停止位置（上停针位/下停针位）。 |
| 剪线抬压脚/停车抬压脚键 |  | 1、若点击，图标显示，设定使用或取消剪线后自动抬压脚功能。 2、若长按，图标显示，设定使用或取消中途停车自动抬压脚功能。 |
| 自由缝/剪线功能键 |  | 1、若点击，切换为自由缝模式。 2、若长按，设定使用或取消剪线功能。 |
| W缝/多段定针缝键 |  | 1、若点击，切换为W缝模式。 2、若长按，切换为多段定针缝模式。 |
| 进入参数区功能键 |  | 一般模式下长按P键进入用户参数模式。 按住P键开机进入技术员参数模式。 |
| 设置速度递增/参数递增 |  | 1、向上调速度键。 2、参数内容区内当设定数值递增键。 |
| 设置速度递减/参数递减 |  | 1、向下调速度键。 2、参数内容区内当设定数值递减键。 |
| 向左选择内容参数 |  | 向左选择内容参数 |
| 向右选择内容参数 |  | 向右选择内容参数 |
| 恢复出厂设置键 |  | 长按3S后恢复出厂设置 |

2控制系统安装后的调试

2.1 一体化控制箱的调试说明

2.1.1:  进入用户参数模式及保存

长按键进入用户参数模式。按或更改参数项，移动光标，相应位数值闪烁，再按或修改该位数值，按键，整个参数值闪烁，保存参数。再按键退出。

2.1.2： 进入技术员参数模式及保存

按住键开机（显示P27）进入技术员参数模式。按或更改参数项，移动光标，相应位数值闪烁，再按或修改该位数值，按键，整个参数值闪烁，保存参数。再按键退出。

2.2 软启动

在用户参数P14调整，参数为ON开启，参数为OFF关闭（默认）。

2.3 调速度

调速度。按调快，按调慢，每按一次调整数值50。为临时调速，与内部调速参数01号参数显示一致，01号参数限制4000，解除限制按目前其它参数控制。

2.4 调整上针位设置

3  参数说明

3.1  用户参数 (开机后长按P键3秒进入)

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P01 | 最高转速（rpm） | 200-5000 | 3800 | 车缝时的最高转速设定 |
| P02 | 加速曲线调整（%） | 10-100 | 80 | 控速器爬升斜率设定 斜率值愈大，速度愈陡；斜率值愈小，速度愈慢 |
| P03 | 针停定位选择 | DN/UP | DN | UP：上停针 DN：下停针 |
| P04 | 起始回缝速度（rpm） | 200-3200 | 1800 | 前段回缝（起始回缝）时的速度设定 |
| P05 | 终止回缝速度（rpm） | 200-3200 | 1800 | 后段回缝（终止回缝）时的速度设定 |
| P06 | 连续回缝速度（rpm） | 200-3200 | 1800 | 连续回缝时的速度设定 |
| P07 | 慢速起缝速度（rpm） | 200-1500 | 400 | 慢速起缝时的速度设定 |
| P08 | 慢速起缝针数（针） | 0-99 | 2 | 慢速起缝时的针数设定 |
| P09 | 自动定针缝速度（rpm） | 200-4000 | 3500 | 触发自动功能键按下时的速度设定 |
| P10 | 定针缝后自动执行终止回缝功能（不补针功能设定） | ON/OFF | ON | ON：在执行完最后一段定针缝后，将自动执行终止回缝动作。即在任何缝制模式下，终止回缝前不能作补针功能。 OFF：在执行完最后一段定针缝后，将无法自动执行终止回缝功能，必须重新再作前或全后踏动作时始可。 |
| P11 | 手按回缝时功能模式选择 | J/B | J | J：JUKI方式（在车缝中和中途停止时均执行倒缝） B：BROTHER方式（即在车缝中执行倒缝动作，在停止时执行补针动作） |
| P12 | 起始回缝运动模式选择 | A/M | A | A：轻触踏板，即自动执行起始回缝 M：受踏板控制，可任意停止 |
| P13 | 起始回缝结束点操作模式选择 | CON/STP | CON | CON：起始回缝段完成后，自动连续下一段功能 STP：起始回缝段针数完成后自动停止 |
| P14 | 慢速启动 | ON-OFF | OFF |  |
| P15 | 补针方式 | 0-2 | 0 | 0：半针；1：一针；2：连续补半针； |
| P18 | 起始回缝补偿1 | 0-200 | 131 | 起始回缝A段针迹补偿，0～200动作逐步滞后；数值越大，A短最后一针越长，B段第一针越短 |
| P19 | 起始回缝补偿2 | 0-200 | 158 | 起始回缝B段针迹补偿，0～200动作逐步滞后；数值越大，B段最后一针越长 |
| P24 | 踏板剪线位置 | 30-500 | 110 |  |
| P25 | 终止回缝补偿3 | 0-200 | 131 | 终止回缝C段针迹补偿，0～200动作逐步滞后；数值越大，C段第一针越短 |
| P26 | 终止回缝补偿4 | 0-200 | 158 | 终止回缝D段针迹补偿，0～200动作逐步滞后；数值越大，C段最后一针越长，D段第一针越短 |
| P28 | 连续回缝运动模式选择 | A-M | A | A：轻触踏板，即自动执行连续回缝 M：受踏板控制，可任意停止 |
| P29 | 切线停车力度 | 1-50 | 24 |  |
| P30 | 过厚力度 | 0-100 | 0 |  |
| P31 | 剪线力度 | 0-100 | 40 |  |
| P32 | 连续回缝补偿5 | 0-200 | 131 | 起始回缝A（C）段针迹补偿，0～200动作逐步滞后；数值越大，A（C）段最后一针越长；B（D）段第一针越短 |
| P33 | 连续回缝补偿6 | 0-200 | 158 | 起始回缝B（D）段针迹补偿，0～200动作逐步滞后；数值越大，B（D）段最后一针越长，C段第一针越短 |
| P34 | 定针缝运动模式选择 | A-M | A | A：轻触踏板，即自动执行连续回缝 M：受踏板控制，可任意停止 |
| P37 | 自动拨线/夹线功能设定 | 0-11 | 8 | 0：无动作； 1为拔线动作 2～11 夹线动作，动作力度逐步变大 |
| P38 | 自动切线功能设定 | ON/OFF | ON | ON：有剪线动作 OFF：无剪线动作 |
| P39 | 中途停车自动抬压脚设定 | DN/UP | DN | UP：车缝停止时，自动抬起压脚 DN：车缝停止时，无自动抬压脚（由踏板后踏控制） |
| P40 | 切线自动抬压脚设定 | DN/UP | DN | UP：切完线后，自动抬起压脚 DN：车切完线后，无自动抬压脚（由踏板后踏控制） |
| P41 | 计数器显示 | 0-9999 |  | 车缝完成件数显示；长按夹线键可计数清零； |
| P42 | 信息显示 | N01-N07 | N01 | N01 电控版本号 N02 选针盒版本号 N03 转速 N04 脚踏板AD N05 上定位角度 N06 下定位角度 N07 母线电压AD |
| P43 | 马达转动方向设定 | CCW/CW | CCW | CW：顺时针方向 CCW： 逆时针方向 |

3.2  技术员参数内容表（按P键开机进入）

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P44 | 正常停车力度 | 1-50 | 16 | 机器停车时的力度选择。 |
| P45 | 回缝出力的周期信号（%） | 1-50 | 25 | 回缝动作时，以周期性省电输出，避免电磁铁发烫 |
| P46 | 切线后，反转提针功能选择 | ON/OFF | OFF | ON：开启 OFF：关闭 |
| P47 | 切线后，反转提针角度的调整 | 50-200 | 160 | 切完线后，由上针位算起，以反向运转作提针的角度调整。 |
| P48 | 最低速度（定位速度）（rpm） | 100-500 | 210 | 最低速度限制调整 |
| P49 | 切线速度（rpm） | 100-500 | 250 | 调整切线周期时的电机速度 |
| P50 | 抬压脚全额出力的工作时间（ms） | 10-990 | 200 |  |
| P51 | 压脚出力的周期信号（%） | 10-40 | 25 | 压脚动作时，以周期性省电输出，避免压脚电磁铁发烫 |
| P52 | 延迟马达启动，保护压脚下放时间（ms） | 10-990 | 120 | 踩下时延迟启动时间，以配合自动抬压脚放下的确认 |
| P53 | 半后踏抬压脚功能取消 | ON/OFF | OFF | ON：半后踏时，无抬压脚 OFF：半后踏时，有抬压脚 |
| P54 | 剪线动作时间（ms） | 10-990 | 200 | 剪线时序所需的动作时间 |
| P55 | 拨 / 扫线动作时间/ | 10-990 | 380 | 拨 / 扫时序的动作时间 |
| P56 | 开电后自动找上定位 | 0-2 | 2 | 0：始终不找上定位 1：始终找上定位 2：若电机已经处于上定位时不再找上定位 |
| P57 | 抬压脚保护时间（s） | 1-120 | 10 | 抬压脚保持时间后强制关闭 |
| P58 | 上定位调整 | 0-1799 | 40 | 上定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P59 | 下定位调整 | 0-1799 | 750 | 下定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P60 | 测试速度（r/s） | 100-4000 | 2000 | 设置测试速度 |
| P61 | A项测试 | ON/OFF | OFF | A项测试选项，设定后将按【P60. TV】所设定之速度持续运行 |
| P62 | B项测试 | ON/OFF | OFF | B项测试选项，设定后将按【P60. TV】所设定之速度执行启动-车缝-停车-剪线的循环 |
| P63 | C项测试 | ON/OFF | OFF | C项测试选项，设定后将按【P60. TV】所设定之速度执行无定位动能的启动-车缝-停车的循环 |
| P64 | 测试B、C导通时间 | 1-250 | 20 | B、C项测试中，设置导通时间 |
| P65 | 测试B、C停车时间 | 1-250 | 20 | B、C项测试中，设置停车时间 |
| P66 | 机头保护开关检测 | 0-2 | 1 | 0：不检测 1：检测零信号 2：检测正信号 |
| P71 | 压脚释放缓冲占空比(%) | 0-40 | 2 | 压脚下放时的力度调整 |
| P72 | 上定位快捷调整 | 0-1799 |  |  |
| P73 | 下定位快捷调整 | 0-1799 |  |  |
| P76 | 倒缝全额出力时间(ms) | 10-990 | 250 | 倒缝开始动作时，全额出力的动作时间 |
| P78 | 夹线器起夹角度 | 10-150 | 100 | 夹线器起始的定位角 |
| P79 | 夹线器结束角度 | 160-359 | 270 | 夹线器结束的定位角 |
| P80 | 剪线进刀角度 | 5-359 | 18 | 剪线进刀角度设置（下定位为0°计算） |
| P81 | 剪线加力角度 | 5-359 | 120 | 剪线加力角度设置（下定位为0°计算） |
| P82 | 剪线退刀角度 | 5-359 | 172 | 剪线退刀角度设置（下定位为0°计算） |
| P83 | 刹车距离 | 10-100 | 20 |  |
| P84 | 厚料开始加力角度 | 0-359 | 9 |  |
| P85 | 厚料结束加力角度 | 0-359 | 57 |  |
| P89 | 交流过压值设定 | 500-1023 | 880 |  |
| P92 | 编码器起始角度 | 0-479 | 445 | 按夹线键读取编码器起始角度，出厂已设置，请勿随意更改（参数值不可手动更改，随意更改会导致控制箱、电机出现异常或损坏） |
| P117 | 压脚释放缓冲延迟时间（ms） | 0-990 | 12 | 压脚释放缓冲延迟起作用的时间调整 |
| P119 | 电磁铁过流保护功能开关 | 0-1 | 1 | 0：关闭 1：开启 |

注：参数初始值仅供参考，实际参数值以实物为准。

4  错误代码表

| 错误码 | 问题描述 | 解决措施 |
| --- | --- | --- |
| E01 | 超电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否超过使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E02 | 低电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否低于使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E03 | CPU通信异常 | 关闭系统电源，检查控制面板的连线是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E05 | 控速器接触异常 | 关闭系统电源，检查控速器接头是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控速器并通知售后服务。 |
| E07 | 电机堵转 | 转动机头手轮观察是否卡住。如卡住则先排除机械故障。 如转动正常，检查电机编码器接头和电机电源线接头是否松动。如有松动请修正。 如接触良好，检查供应电源电压是否异常或转速设置过高。如有请调整。 若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E08 | 手动倒缝动作时间超过15秒 | 倒缝电磁铁吸合时间过长，重启产品即可。 若重启产品后还是报E-08，请检查手动倒缝开关是否损坏。 |
| E10 | 电磁铁过流保护 | 关闭系统电源，检查电磁铁（电磁阀）是否损坏或短路。 |
| E09 E11 | 定位信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E14 | 编码器信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E15 | 电力模块不正常过流保护 | 关闭系统电源，再重新开启。若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E17 | 机头保护开关没到正确位置 | 关闭系统电源，检查机头是否掀开，控制箱内滚珠开关是否移位或损坏。 |
| E20 | 电机启动失败（电角度错误） | 关闭系统电源，检查电机编码器接口和电机电源接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控制箱并通知售后服务。 |

5 端口示意图

5.1 各个端口名称

16P功能端口

12P功能端口

电机编码器端口

控速器端口

电机电源端口

5.2   16P功能端口对应表

1.剪线电磁铁：1、9

2.夹线（扫线）电磁铁：2、10

3.衣车灯：4（信号地）、12（+5V）

4.手动倒缝：5（信号）、13（信号地）

5.倒缝电磁铁：6、14

6.手动补针：7（信号）

  - 12P功能端口对应表

1. 压脚电磁铁：6、12

Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign   must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250 VAC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

14． Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker)

1. Button Display and Operating Instructions

  - Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Start Back-Tacking Sewing |  | If click, execution starting back seam B segment or execution starting back seam (A, B segment) 1 Execution starting back seam (A, B segment) 2 times or turn off. |
| End Back-Tacking Sewing |  | If click, execution terminates back seam C segment or execution terminates back seam (C, D segment) 1 Execution terminates back seam (C, D segment) 2 time or turn off. |
| Clamp Function Setting / Needle Stop Position Selection |  | 1. If click, set used or cancelled clamp function 2. If long-press, the needle stop position after shift the sewing mode ( Up position / Down position) |
| Press Foot Lifting after Trimming / Press Foot Lifting after Pause |  | 1. If click, when icon is lighting, set used or cancelled press foot lifting after trimming function. 2. If long-press, when icon is lighting, set used or cancelled press foot lifting after pause function. |
| Free Sewing / Thread Trimming |  | 1. If click, shift to free sewing 2. If long-press, set used or cancelled thread trimming function. |
| Continuous Back Seam Shortcuts/ multi-section set needle shortcuts. |  | 1. If click, shift to continuous back seam model. 2. If long-press, shift to multi-section set needle shortcuts. |
| Function Parameter Edit |  | In [Normal Mode], Long-press【P】key to enter User Parameter Mode. Press and hold【P】key, then turn on the power to enter Technician Parameter Mode. |
| Setting Speed Increase / Parameter Increase |  | 1. Speed adjust upper key 2. Parameter value increase key |
| Setting Speed Decrease/ Parameter Decrease |  | 1. Speed adjust lower key. 2. Parameter value decrease key. |
| Left Selection Parameter Content |  | Left selection parameter content |
| Right Selection Parameter Content |  | Right selection parameter content |
| Restore Factory |  | Long-press 3 sec can restore factory setting. |

2 Control System after Installation Debugging

2.1 Integrate Control Box Debugging

2.1.1： Enter to User Parameter Mode and Save.

Long press  key enter to User Parameter Mode. Press  or  to change parameter item, remove cursor, the corresponding digit value is flashing, then press or to modify this digit value, press key, the whole parameter value is flashing, save the parameter. Press  key again to exit.

2.1.2： Enter to Technician Parameter Mode and Save

Hold on key to turn on the machine (show P27) enter to technician parameter mode.  Press or  change parameter item, remove cursor, the corresponding digit value is flashing, then press or to modify this digit value, presskey, the whole parameter value is flashing, save parameter. Press key again to exit.

2.2   Soft start:

P14 adjustment in user parameters, Parameter is ON, Parameter is Off (The default value).

2.3   Adjustment speed:

Upper key and lower key to adjustment speed.  Press to accelerate speed, press  to decelerate, each press the adjust vale is 50. For temporary adjust speed, the parameter in accordance with internal parameter of speed adjustment 01. 01 parameter is limited 4000

2.4 Adjust up needle position setting

3 Parameter Definition

3.1 User Parameter (Turn on the machine long-press P key for 3 sec. to enter)

| Parameter Item | Parameter Function | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum Sewing Speed（r/S） | 200-5000 | 3800 | Maximum sewing speed setting |
| P02 | Speed Curve Adjustment（%） | 10-100 | 80 | The speed controller climbing slope setting. The large of slope value = the fast of the speed. The small of slope value = the slow of the speed. |
| P03 | Needle UP / DOWN Selection | DN/UP | DN | Same as panel shortcut function key UP: Needle Stops at Up Position DN: Needle Stops at Down Position |
| P04 | Start Back-Tacking Speed（r/S） | 200-3200 | 1800 | Speed setting of Front-end back tacking (start back tacking) |
| P05 | End Back-Tacking Speed（r/S） | 200-3200 | 1800 | Speed setting of rear-end back tacking (End back tacking) |
| P06 | Bar-Tacking Speed（r/S ） | 200-3200 | 1800 | Speed setting of Bar-tacking |
| P07 | Soft Start Speed（r/S ） | 200-1500 | 400 | Soft Start Speed Adjustment |
| P08 | Stitch Numbers for Soft Start（SLS） | 0-99 | 2 | Soft Start Stitches Setting ( one unit = half stitch) |
| P09 | Automatic Constant-Stitch Sewing Speed（r/S ） | 200-4000 | 3500 | Constant-Stitch sewing speed【034.SMP】 is set at A(or when AUTO signal is active) |
| P10 | Automatic End Back-Tacking Sewing after constant sewing(Can invalidate the stitch compensation function) | ON/OFF | ON | ON: It will automatic execute end back-tacking action after finish last section of constant sewing. Note: if under any sewing mode, invalidate for stitch compensation function. OFF: It can’t automatic execute end back-tacking function after finish last section of constant sewing. You should forward step pedal or full heeling the pedal. |
| P11 | Function Mode Selection when manually push Back-Tacking | J/B | J | Manually push Back-Tacking switch action: J: JUKI Mode(During sewing or stop sewing both have this action) B: BROTHER Mode ( Only acts during sewing) |
| P12 | Start Back-Tacking Mode Selection | A/M | A | Start Back-Tacking, reverse solenoid action: A：One shot to pedal, it will automatic execute Start Back-Tacking. M：Pedal-controlled and motor can stop arbitrarily |
| P13 | Mode Selection at the end of Start Back-Tacking | CON/STP | CON | CON：At the end of Start Back-Tacking, machine continues sewing if pedal pressed or START signal on ( standing operation) STP：At the end of Start Back-Tacking, machine stops |
| P14 | Soft Start | ON-OFF | OFF |  |
| P15 | Stitch Compensation Mode | 0-2 | 0 | 0： Fill half needle，1：Fill a needle 2：Continuous half needle； |
| P18 | Start back tacking compensation 1 | 0-200 | 131 | Stitch Balance for end back-tacking A section, 0~200 action gradually delay; the large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start back tacking compensation 2 | 0-200 | 158 | Stitch Balance for end back-tacking B section, 0~200 action gradually delay; the large value, the longer of the B section first stitch. |
| P24 | The voltage of pedal heeling | 30-500 | 110 |  |
| P25 | Stitch compensation for End Back-Tacking 3 | 0-200 | 131 | Stitch Balance for end back-tacking C section, 0~200 action gradually delay; the large value, the shorter of the C section first stitch. |
| P26 | Stitch compensation for End Back-Tacking 4 | 0-200 | 158 | Stitch Balance for end back-tacking D section, 0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P28 | Bar-tacking sewing Mode Selection | A-M | A | Bar-Tacking, reverse solenoid action: A：One shot to pedal, it will automatic execute Bar-Tacking. M：Pedal-controlled and motor can stop arbitrarily |
| P29 | Brake pressure after trimming | 1-50 | 24 |  |
| P30 | Torque increase of over thick material | 0-100 | 0 |  |
| P31 | Torque increase of trimming | 0-100 | 40 |  |
| P32 | Stitch compensation for Bar-Tacking 5 | 0-200 | 131 | Stitch Balance for start back-tacking A(C) section, 0~200 action gradually delay; the large value, the longer of the A(C) section last stitch; the shorter of the B (D) section first stitch. |
| P33 | Stitch compensation for Bar-Tacking 6 | 0-200 | 158 | Stitch Balance for end back-tacking D section, 0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P34 | Mode Selection for Constant-Stitch Sewing | A-M | A | A：One shot to pedal, it will automatic execute Constant-Stitch M：Pedal-controlled and motor can stop arbitrarily |
| P37 | Wiper Function Selection (Pressure setting of the Thread Clamp Device） | 0-11 | 8 | 0: No Action 1: Wiper action 2~11 : Clamp action, the action pressure gradually increased |
| P38 | Trimmer Output Function Selection | ON/OFF | ON | ON：Trimmer Valid OFF: Trimmer Invalid |
| P39 | Presser Foot UP / Down at intermediate stop | DN/UP | DN | Same as panel shortcut function key UP：Presser foot goes up automatically DN：Presser foot keeps down（Controlled by heeling pedal） |
| P40 | Presser Foot UP / Down after Trimming | DN/UP | DN | Same as panel shortcut function key UP：Presser foot goes up automatically DN：Presser foot keeps down（Controlled by heeling pedal） |
| P41 | Display the sewing finished quantity | 0-9999 |  | Counting the finished-sewing quantity |
| P42 | Information display | N01-N07 | N01 | N01 Control box version no. N02 Panel version no. N03 Rotational Speed N04 Pedal AD N05 Up position angle N06 Down position angle N07 Busbar Voltage AD |
| P43 | Setting Direction of Motor Rotation (Clockwise/ Counter Clockwise) | CCW/CW | CCW | CW：Clockwise CCW：Counter Clockwise |

3.2 Technician Parameter (Press P key to enter)

| Parameter Item | Parameter Function | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P44 | Brake efforts | 1-50 | 16 | Adjustment of efforts while machine stop |
| P45 | Periodic signal of back seam output （%） | 1-50 | 25 | Periodic energy saving output while back seam proceeding |
| P46 | Inversion of the needle angle of choice after trimming | ON/OFF | OFF | ON: ON OFF: OFF |
| P47 | Adjustment of reverse angles after trimming | 50-200 | 160 | Reverse running is subject to the adjustment of needle angle after trimming |
| P48 | low(positioning)speed（r/S ） | 100-500 | 210 | Setting positioning speed |
| P49 | Trimming speed（r/S ） | 100-500 | 250 | Adjustment of motor running speed while trimming proceeding |
| P50 | Time of foot lifting（ms） | 10-990 | 200 |  |
| P51 | Periodic signal of foot lifter output（%） | 10-40 | 25 | Setting as periodic power saving output of foot lifter can avoid electromagnet’s over-heating coming |
| P52 | Time of foot lifter down（ms） | 10-990 | 120 | Delay down time setting for foot lifter |
| P53 | Cancel foot lifting if half-heeling pedal | ON/OFF | OFF | ON: without foot lifting while half-heeling pedal OFF: with function |
| P54 | Trimming time（ms） | 10-990 | 200 | Setting time trimming sequence required |
| P55 | Wiper time | 10-990 | 380 | Setting time wiper sequence required |
| P56 | Automatically find UP position | 0-2 | 2 | 0: always not to find the up needle position 1: always to find the up needle position 2: If the motor is not into the up needle position, it will find the up needle position |
| P57 | Protection time for foot lifter（s） | 1-120 | 10 | Foot lifter Come down automatically after setting time is over. |
| P58 | Up Position Adjustment | 0-1799 | 40 | Needle stops ahead of time with value decrease; Needle delayed stopping with value increase. |
| P59 | Down Position Adjustment | 0-1799 | 750 | Needle stops ahead of time with value decrease; Needle delayed stopping with value increase. |
| P60 | Testing Speed（r/s） | 100-4000 | 2000 | Testing speed setting |
| P61 | Testing A | ON/OFF | OFF | After setting A, continuous running at the speed of【P60. TV】 |
| P62 | Testing B | ON/OFF | OFF | After setting B, continuous running at the speed of【P60. TV】 |
| P63 | Testing C | ON/OFF | OFF | After setting C, continuous running at the speed of【P60. TV】 |
| P64 | Running Time of Testing B and C | 1-250 | 20 | Setting running time of Testing B and C |
| P65 | Stopping time of Testing B and C | 1-250 | 20 | Setting time of Testing B and C |
| P66 | Machine protection switch Testing | 0-2 | 1 | 0： Disable 1：Testing zero signal 2： Testing positive signal |
| P71 | Buffer of Presser foot release(ms) | 0-40 | 2 | The buffer force of presser foot release operation. |
| P72 | Up needle position adjustment | 0-1799 |  |  |
| P73 | Down needle position adjustment | 0-1799 |  |  |
| P76 | Full output time of backstitch(ms) | 10-990 | 250 | Back to the beginning of the action, the full force of the action time. |
| P78 | Starting clip angle of thread clamp | 10-150 | 100 | Positioning angle when start clipping. |
| P79 | Stopping clip angle of thread clamp | 160-359 | 270 | Positioning angle when finish clipping. |
| P80 | Feeding angle of thread trimming | 5-359 | 18 | Feeding angle of thread trimming setting(Down position as 0°) |
| P81 | Torque angle of thread trimming | 5-359 | 120 | Torque angle of thread trimming setting(Down position as 0°) |
| P82 | Feeder back angle of thread trimming | 5-359 | 172 | Feeder back angle of thread trimming setting Down position as 0°) |
| P83 | The stopping distance | 10-100 | 20 |  |
| P84 | Starting torque angle of Thick material machine | 0-359 | 9 |  |
| P85 | Stopping torque angle of Thick material machine | 0-359 | 57 |  |
| P89 | AC Overpressure Value Setting | 500-1023 | 880 |  |
| P92 | Encoder start angle | 0-479 | 445 | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P117 | The delay time of the presser foot release damping force (ms) | 0-990 | 12 |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 1 | 0: OFF 1: ON |

Note: the parameter initial value is for reference only, the actual parameter value is subject to the object.

4 Error Code List

| Error Code | Problem description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage. (Or whether exceed the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage.(Or whether under the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E03 | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 | Pedal connection error | Turn off the machine, please check if the pedal in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E08 | Manual reverse feeding operation more than 15 seconds | Back-Tacking electromagnet response time is too long, you have only to restart the machine. If there are still errors E-08 after restart the machine, check if the manual reverse feeding switch is damaged. |
| E10 | Solenoid over-current | Turn off the machine, please check the related circuit is shorted or damaged. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E15 | Power supply module abnormal over-current | Turn off the machine, then turn on again. If there are still errors, please replace the control box and call for customer service. |
| E17 | Machine protection switch is not in the correct position | Turn off the machine, check if the machine is turn over, or the Mercury Switch is shifted or damaged. |
| E20 | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |

5 Port Diagram

5.1 Port Description

16P Function Port

12P Function Port

Motor Encoder

Speed Control unit

Motor Power

5.2 16P Function Port Description

1. Thread trimming electromagnet：1、9

2. Thread clamping (thread wiping) electromagnet：2、10

3. LED Light：4（DGND）、12（+5V）

4. Reverse stitch key：5（signal）、13（DGND）

5. Back-tacking electromagnet：6、14

6. Mending stitch key：7（signal）

  - 12P Function Port Description

1. Presser foot electromagnet：6、12