QD289系列控制系统说明书V1.0.02

No. 10404023

安全指示

- 在安装或使用本产品前，使用者必须详细阅读本操作手册。

- 本产品须由受过正确训练的人员来安装或操作。安装作业时必须关闭所有电源，切记不可带电操作。

3） 所有标有符号的指示，必须特别注意并按照说明书上的执行，以免造成不必要的损害。

4） 为安全起见，禁止以延长线作电源座供应二项以上的电器产品使用。

5） 在连接电源线时，必须确定工作电压低于AC 250V，且符合本产品标识中规定的额定电压值。

※注意  ：电控箱电源规格如为AC220V时，请勿插接至AC380V的电源插座上，否则将出现异常且电机无法动作。此时请立即关闭电源开关，重新检查电源。持续供应380V超过五分钟以上，将可能烧损电控箱内器件，而危及人身安全。

6） 请不要在日光直接照射的场所、室外及室温45℃以上或0℃以下的场所操作。

7） 请不要在暖气（电热器）旁、有露水的场所及在相对湿度10%以下或90%以上的场所操作。

8） 请不要在灰尘多的场所、具有腐蚀性物质的场所及有挥发性气体的场所操作。

9） 请注意所有电源线、信号线、接地线等接线时不要受压或过度扭曲，以确保使用安全。

10）电源线的接地端须以适当大小的导线和接头连接到生产工厂的系统地线，此连接必须被永久固定。

11）所有可转动的部分，必须以所提供的零件加以防范露出。

12）在安装完成第一次开电后，先关闭切线功能以低速操作缝纫机并检查转动方向是否正确、运转是否稳定。

13）在进行以下操作前，请先关闭所有电源：

1.在控制箱与马达上插拔任何连接插头时。

2.穿针线时。

3.翻抬缝纫机机头时。

4.修理或做任何机械上的调整时。

5.机器闲置不用时。

14）修理或高层次的保养工作，仅能由受过训练的机电技师来执行。

所有维修用的零件，须由本公司提供认可，方可使用。

15）使用本产品请远离高频电磁波和电波发射器等，以免所产生的电磁波干扰伺服驱动装置而发生误动作。

16）请不要以不适当物体来敲击或撞击本产品及各装置。

保修期限

本产品保修期限为购买日期起一年内或出厂月份起两年内。

保修内容

本产品在正常情况使用且无人为操作失误的前提下，于保修期间无偿为客户维修使能正常操作。

但以下情况于保修期间将收取维修费用：

1.不当使用包括误接高压电源、将产品移做其它用途、自行拆卸、维修、更改、或不依规格范围使用、进水进油及插入异物于本产品。

2.火灾、地震、闪电、风灾、水灾、盐蚀、潮湿、异常电压及其它天灾或不当场所造成的损害。

3.客户购买后摔落本产品，或客户自行运输（或托付运输公司）造成的损害。

* 本产品在生产及测试上皆尽最大努力和严格控制使其达到高品质及高稳定的标准，但外部的电磁或静电干扰或不稳定的供应电源，仍可能对本产品造成影响或损害，因此操作场所的接地系统一定要确实做好，并建议用户安装故障安全防护装置（如漏电保护器）

1．按键说明

| 名称 | 按键 | 注明 |
| --- | --- | --- |
| 进入参数去功能键 |  | 1、一般模式下按下P键进入用户参数设置模式。 2、在关机状态，按住P开机进入技术员参数模式。 |
| 参数查看保存键 |  | 对所选参数号内容进行查看和保存：选择好参数号后按此键可以进行查看和修改操作，修改参数值后按此键则退出并保存参数。 |
| 上调节键 |  | 1、参数选择区内参数项递增键。 2、参数内容区内设定数值递增键。 3、主界面时为快捷加速功能。 |
| 下调节键 |  | 1、参数选择区内参数项递减键。 2、参数内容区内设定数值递减键。 3、主界面时为快捷减速功能。 |
| 慢速起缝键 |  | 若点击，设定使用或取消慢速起缝功能。 |
| 停针位键 |  | 若点击，切换缝制后机针的停止位置（上停针位/下停针位）。 |

2.用户参数＆技术员参数

2.1 用户参数

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P01 | 最高转速（rpm） | 020-370 | 370 | 车缝时的最高转速设定（显示的数*10=实际速度） |
| P02 | 停针位选择 | 000-002 | 000 | 停针位设定（0：上针位；1：下针位；2：无针位） |
| P03 | 慢速起缝开关 | 000-001 | 000 | 慢速起缝开关设定（0：关 1：开） |
| P04 | 慢速起缝速度 | 020-150 | 040 | 慢速起缝时的速度设定（显示的数*10=实际速度） |
| P05 | 慢速起缝针数 | 001-015 | 001 | 慢速起缝时的针数设定。每一单位代表半针 |
| P06 | 最低转速 | 200-500 | 200 | 车缝时的最低转速设定（显示的数*10=实际速度） |

2.2 技术员参数

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P07 | 上定位调整 | 000-024 | 000 |  |
| P08 | 下定位调整 | 000-024 | 012 |  |
| P09 | 开机自动找上定位 | 000-001 | 001 | 0：开机不找上定位；1：开机找上定位 |
| P10 | 机头保护开关检测 | 000-002 | 001 | 0：不检测 1：检测零信号 2：检测正信号 |
| P11 | 加速曲线调整（%） | 000-100 | 032 | 控速器的加速爬升斜率设定 |
| P15 | 补针方式 | 000-003 | 000 | 0：半针；1：一针；2：连续补半针；3：连续补一针； |
| P21 | 马达运转方向 | 000-001 | 000 | 马达运转方向设定（0：顺时针 1：逆时针） |
| P22 | 自动跑合速度 | 020-500 | 350 | 跑合速度的设置（显示的数*10=实际速度） |
| P23 | 自动跑合跑时间 | 010-250 | 020 | 测试中，设置跑时间 |
| P24 | 自动跑合停时间 | 010-250 | 020 | 测试中，设置停时间 |
| P25 | A项测试 | 000-001 | 000 | A项测试设定（速度持续运行） |
| P26 | B项测试 | 000-001 | 000 | B项测试设定（执行定位循环运行） |
| P36 | 机型 | 000-010 |  |  |

3.错误代码表

| 错误码 | 问题描述 | 解决措施 |
| --- | --- | --- |
| E01 | 超电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否超过使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E02 | 低电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否低于使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E05 | 控速器接触异常 | 关闭系统电源，检查控速器接头是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控速器并通知售后服务。 |
| E07 | 电机堵转 | 转动机头手轮观察是否卡住。如卡住则先排除机械故障。 如转动正常，检查电机编码器接头和电机电源线接头是否松动。如有松动请修正。 如接触良好，检查供应电源电压是否异常或转速设置过高。如有请调整。 若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E09 E11 | 定位信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E12 | 电源开启时，忘记插入定位器 | 自动进入无定位器模式运作，且切线、扫线、上定位等以及所有定位缝式样缝制功能亦无效。马达可正常操作。（请检查定位器是否忘记插入或异常） |
| E14 | 编码器信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E15 | 电力模块不正常过流保护 | 关闭系统电源，再重新开启。若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E17 | 机头保护开关没到正确位置 | 关闭系统电源，检查机头是否掀开，控制箱内滚珠开关是否移位或损坏。 |

4.端口示意图

4.1：14P功能端口说明

Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well-trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250 VAC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

14． Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker)

1．Key Function

| Name | Key | Indicate |
| --- | --- | --- |
| Entering the Parameter Mode |  | 1. Press key P to enter into user parameter setting mode. 2. Long press key P, in the same time turn on the power switch from off to enter into system parameter setting mode. |
| Setting Parameter Check and Save |  | 1. After setting the function code, press this key to check the preset parameter and then can edit the parameter accordingly; 2. When the parameter is fixed, press key to save the setting and quit. |
| On the Adjustment Key |  | 1. Choose the region parameter items incrementing key. 2. Parameter setting value increment key 3. In the main interface, press this key can increase speed. |
| Under the Adjustment Key |  | 1. Choose the region parameter items decrementing key. 2. Parameter setting value decrement key 3. In the main interface, press this key can decrease speed. |
| Slow Launch Setting |  | Set used or cancelled slow Launch Setting function. |
| Needle Stop Position Selection |  | If click, the needle stop position after shift the sewing mode ( Up position / Down position) |

2. User Parameter & Technician Parameter

2.1 User Parameter

| Parameters | Parameter Function | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum speed | 020-370 | 370 | Set the max sewing speed (show the actual number*10=speed) |
| P02 | Needle stop position selection | 000-002 | 000 | Set the needle stop position (0: needle up 1: needle down 2: OFF) |
| P03 | Soft start switch | 000-001 | 000 | Soft start switch setting(0: OFF 1:ON) |
| P04 | Soft start sewing speed | 020-150 | 040 | Soft start sewing speed setting(show the actual number *10=speed) |
| P05 | Stitches number of soft start | 001-015 | 001 | Set the stitches number of soft start, each unit is half stitch |
| P06 | Minimum speed | 200-500 | 200 | Set the minimum sewing speed (show the actual number*10=speed) |

2.2 Technician Parameter

| Parameters | Parameter Function | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P07 | Up position adjustment | 000-024 | 000 |  |
| P08 | Down position adjustment | 000-024 | 012 |  |
| P09 | Needles goes up automatically as power turned on | 000-001 | 001 | 0: No Function 1: Power turned on, needle goes up position automatically |
| P10 | Machine protection switch detection | 000-002 | 001 | O: OFF 1: Zero detection signal 2: Positive detection signal |
| P11 | Speed curve adjustment (%) | 000-100 | 032 | The lager the value, the faster to increase speed |
| P15 | Stitch correction direction | 000-003 | 000 | 0: Half stitch 1: One stitch 2: Continuous correct half stitch 3: Continuous correct stitch and quickly stop machine |
| P21 | Motor rotate direction | 000-001 | 000 | Motor rotate direction setting (0: clockwise 1: counter-clockwise) |
| P22 | Auto running speed | 020-500 | 350 | Auto running speed setting (show the actual number*10=speed) |
| P23 | Auto running time | 010-250 | 020 | Set the auto running time in testing |
| P24 | Auto running stop time | 010-250 | 020 | Set the auto running stop time in testing |
| P25 | Item A test | 000-001 | 000 | Item A test setting(continue running with constant speed) |
| P26 | Item B test | 000-001 | 000 | Item B test setting(carry out the set loop running) |
| P36 | Type | 000-010 |  |  |

3 Error Code List

| Error Code | Problem Description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage. (Or whether exceed the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage.(Or whether under the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E05 | Pedal connection error | Turn off the machine, please check if the pedal in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E12 | Power is turned on without the synchronizer signal | Motor still can run, but it automatically starts the clutch mode, all constant-stitch sewing pattern and trimmer / wiper function is invalid. Please check the synchronizer. |
| E14 | Encoder error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E15 | Power supply module abnormal over-current | Turn off the machine, then turn on again. If there are still errors, please replace the control box and call for customer service. |
| E17 | Machine protection switch is not in the correct position | Turn off the machine, check if the machine is turn over, or the Mercury Switch is shifted or damaged. |

4 Port Diagram

4. 1 Function port description