Safety Instruction

1）Users are required to read the operation manual completely and carefully before installation or operation.

2） The product should be installed and pre-operated by well-trained persons

3） All the instruction marked with sign ，must be observed or executed; Otherwise, bodily injuries might occur.

4） For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection

5） When connecting power supply cords to power sources, it is necessary to make sure that the power voltage is lower than 250V AC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage. Continue supply 380V power over 5 minutes might damage the fuse F2, burst the electrolytic capacitors and the power module U14 of the main board and even might endanger the person safety.

6） Please don’t operate in direct sun light, outdoors area and where the room temperature is over 45℃or below 0°C.

7） Please avoid operating near the heater at dew area or at the humidity below 10% or above 95%.

8） Please don’t operate in area with heavy dust, corrosive substance or volatile gas.

9） Avoid power cord being applied by heavy objects or excessive force, or over bend.

10）The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

11）All the moving portions must be prevented to be exposed by the parts provided.

12）Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

13）Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine arm.

4. Repairing or doing any mechanical adjustment.

5. Machine is out of work.

14）Repairing and high level maintenance work should only be done by electronic technicians with appropriate training.

15）All the spare parts for repairing work must be provided or approved by the manufacturer.

16）Don’t use any objects to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge.

However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency.

* Note: We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this all a failsafe device.  (Such as residual current breaker).

1 Button Displays and operating instructions

1.1 Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Function Parameter Edit |  | Press this key to enter or exit function parameter edit mode. |
| Setting Parameter Check And Save |  | For the content of the selected parameters check and save: after select parameter press this key to check and modify operation, after modified parameter value press this key to exit and save the parameter. |
| Parameter Increase |  | Increase the parameter. |
| Parameter Decrease |  | Decrease the parameter. |
| Slow Launch Setting / Counter Key |  | 1. If click, set used or cancelled slow launch function. 2. If long press, shift to end tight seam stitches interface P122, P131 P132. |
| Needle Stop Position Selection |  | 1. If click, the needle stop position after shift the sewing mode ( Up position / Down position) 2. If long press, shift to start tight seam stitches interface P122, P135, P134. |
| Thread Trimming |  | If click, set used or cancelled thread trimming function. |
| Automatic Presser Foot Lifting Setting |  | 1．Automatic presser foot lifting after trimming function when icon is lighting. 2．Automatic presser foot lifting after pause function when icon is lighting. 3．Automatic presser foot lifting after pause function or trimming function when icon and icon are lighting. 4．Presser Foot is inactive when icon and icon are not lighting. |
| Clamp Function Setting / Loose Line Function Key |  | Set used or cancelled clamp function and loose line function. |
| Mode shift key |  | If click, shift to free sewing, constant stitch sewing, continuous back seam shortcuts and multi-section set needle shortcuts. |
| Start Back-Tacking Sewing |  | 1. Execution starting back seam B segment or execution starting back seam (A, B segment) 1 Execution starting back seam (A, B segment) 2 times. 2. If long press, can increase screen backlighting brightness. |
| End Back-Tacking Sewing |  | 1. Execution terminates back seam C segment or execution terminates back seam (C, D segment) 1 Execution terminates back seam (C, D segment) 2 time. 2. If long press, can decrease screen backlighting brightness. |
| Auto Function |  | 1. under the free sewing mode: press this key no function. 2. Under the Constant-Stitch Sewing mode press this key: As the treadle is toed down, it will automatic performs E,F or G,H section by section, after finished section stitches, it will stop automatically. When the treadle is toed down again, it will automatically execute next section stitches of setting till finished trimming, wiper and so on action. The LED OFF indicate the corresponding function is turn off. 3. If long press, can restore factory setting. |
| Locking Screen Function |  | If click, set used or cancelled locking screen function. |

2 Auxiliary function

2.1 Restore Factory Setting

On the main interface long-press one-shot sewing key, restore factory setting.

2.2 Needle Stop Position Function Setting

In the main interface, click the P key to enter the parameters, and set the parameters to P72. Manually rotate the handwheel to up position. The value displayed will change with the change of the wheel position. Press S key to save the current position (value) as the upper pin stop and calculate down position automatically.

3 Parameter List

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed（rpm） | 100-3700 | 3700 | Maximum speed of machine sewing |
| P02 | Set accelerated curve（%） | 10-100 | 80 | Set the acceleration slope |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle Stops at Up Position DN: Needle Stops at Down Position |
| P04 | Start back-tacking speed（rpm） | 200-3200 | 2000 | Set Start Back-Tacking Speed |
| P05 | End back-tacking speed（rpm ） | 200-3200 | 2000 | Set End Back-Tacking Speed |
| P06 | Bar-tacking speed（rpm） | 200-3200 | 2000 | Set Repeat Bar-Tacking Speed |
| P07 | Soft start speed（rpm ） | 200-1500 | 400 | Set Soft Start Speed |
| P08 | Stitch numbers for soft start | 0-99 | 2 | Soft Start Stitches Setting |
| P09 | Automatic constant-stitch sewing speed（rpm） | 200-4000 | 3700 | Speed adjustment for automatic constant-stitch sewing |
| P10 | Automatic end back-tacking sewing | ON/OFF | ON | ON: After last seam of constant stitch sewing, it will automatic execute the End Back-Tacking sewing function. When turn on, the Stitch-Correction is invalid. OFF: Automatic End Back-Tacking will not execute after last seam, but can be done by pedal manually. |
| P11 | Manual back-tacking mode selection | 0-3 | 0 | 0: JUKI Mode (Press TB switch will activate reverse solenoid when either machine is stopped or running). 1: BROTHER Mode (Press TB switch will activate reverse solenoid only when machine is running). 2: Perform tight seam action when machine sewing or stopping 3: Perform the tight seam when machine sewing, and perform the fill up needle when stopping halfway |
| P12 | Start back-tacking running mode selection | 0-3 | 1 | 0:Could be arbitrarily stop and start 1:Automatically perform actions 2:Pause mode |
| P13 | Ending mode of Start Back-Tacking | CON/STP | CON | CON：Start Back-Tacking is completed automatically continued for next action. STP：After the number of stitches is completed, stop automatically. |
| P14 | Slow start function selection | ON/OFF | ON |  |
| P15 | Mending stitch mode | 0-4 | 2 | 0:half stitch; 1: one stitch; 2: continuous half stitch; 3: continuous one stitch; 4: continuous one stitch, quick stop. |
| P16 | Back-Tacking speed limit | 0-3200 | 3000 | When the parameter is 0, the speed limit function is OFF. |
| P17 | Auto count selection | 0-50 | 1 | 0: P41 item counter not count automatically. 1-50: P41 item counter increment count automatically. |
| P18 | Start Back-Tacking compensation 1 | 0-200 | 131 | Stitch Balance for end back-tacking A section, 0~200 action gradually delay; the large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start Back-Tacking compensation 2 | 0-200 | 152 | Stitch Balance for end back-tacking B section, 0~200 action gradually delay; the large value, the longer of the B section first stitch. |
| P20 | End Back-Tacking running mode selection | 0-3 | 1 | 0: Manual, pedal-control can stop and start 1: Automatically perform actions 2: Pause mode |
| P21 | The position of the pedal for running | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for lifting presser foot | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming | 30-500 | 130 |  |
| P25 | End Back-Tacking compensation 3 | 0-200 | 131 | Stitch Balance for end back-tacking C section,0~200 action gradually delay; the large value, the shorter of the C section first stitch. |
| P26 | End Back-Tacking compensation 4 | 0-200 | 152 | Stitch Balance for end back-tacking D section,0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P27 | Auxiliary function setting |  |  | N01 Lock screen clock(s) N10 Backlight brightness settings N11 Backlight extinguishing time setting N12 start machine display counter interface selection（0：OFF 1：ON） N13 Counter mode selection（0：addend sewing counter 1：subtrahend sewing counter） |
| P28 | Bar-Tacking running mode selection | 0-3 | 1 | 0: Could be arbitrarily stop and start 1: Automatically perform actions 2: Pause mode |
| P29 | The rate of thread trimming stop | 1-45 | 20 |  |
| P30 | The strength of the motor torque when sewing | 0-100 | 0 | The larger the value the greater the intensity, the adjustment is too large may cause the motor abnormalities. |
| P31 | The strength of the motor torque when thread trimming | 0-100 | 40 | The larger the value the greater the intensity, the adjustment is too large may cause the motor abnormalities. |
| P32 | Bar-Tacking compensation 5 | 0-200 | 131 | Stitch Balance for start back-tacking A (C) section, 0~200 action gradually delay; the large value, the longer of the A (C) section last stitch; the shorter of the B(D) section first stitch. |
| P33 | Bar-Tacking compensation 6 | 0-200 | 152 | Stitch Balance for start back-tacking B (D) section, 0~200 action gradually delay; the large value, the longer of the B (D) section last stitch; the shorter of the C section first stitch. |
| P34 | Constant-Stitch Sewing running mode selection | A/M | A | A：Automatically perform actions M：Could be arbitrarily stop and start |
| P36 | Thread slacking function selection | 0-11 | 0 | 0: OFF 1-11: The strength of thread slacking |
| P37 | Automatic thread wiping function / Automatic thread clamping function selection | 0-11 | 8 | 0 : OFF 1: Automatic thread wiping Function 2-11: Automatic thread clamping Function and the strength of automatic thread clamping. |
| P38 | Automatic thread trimming function selection | ON/OFF | ON | ON: ON OFF: OFFF |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN | UP: ON DN: OFF |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN | UP: ON DN: OFF |
| P41 | Counter display | 0-9999 |  | Display the quantity of finished sewing piece. |
| P42 | Information Display |  |  | N01 The control system version number N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(up position) N06 The mechanical angle(down position) N07 Busbar voltage AD N12 Kneeling device position sensor AD |
| P43 | Direction of Motor Rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |
| P44 | The rate of normal stop | 1-45 | 16 |  |
| P45 | Back-tacking operation duty cycle (%) | 1-50 | 30 | Back-tacking operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P46 | Motor stop with a reverse angle after trimming function | ON/OFF | OFF | ON: ON OFF: OFF |
| P47 | Adjust the reverse angle when motor stop after trimming | 50-200 | 160 | Adjusting at reverse direction after trimming. |
| P48 | The minimum speed Limit（rpm ） | 100-500 | 210 | Adjust the minimum speed |
| P49 | Thread trimming speed（rpm） | 100-500 | 300 | Adjust thread trimming speed |
| P50 | Toot lifter operation time to full output（ms） | 10-990 | 200 |  |
| P51 | Presser foot lifter operation duty cycle (%) | 1-50 | 30 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Motor starting delay time（ms） | 10-990 | 120 | Delay the start time, with automatic presser foot down. |
| P53 | Half back pedaling lifting presser foot function to cancel | ON/OFF | OFF | ON: Function to cancel OFF: Half back pedaling lifting presser foot |
| P54 | Thread trimming action time（ms） | 10-990 | 200 | Completion of the thread trimming action required time. |
| P55 | Thread wiping action time（ms） | 10-990 | 30 | Completion of the thread wiping action required time. |
| P56 | Power on and positioning | 0-2 | 0 | 0: always not to find the up needle position 1: always to find the up needle position 2:If the motor is not into the up needle position, it will find the up needle position |
| P57 | Hold time of presser foot lifter（s） | 1-120 | 10 | Forced shut-down after hold time. |
| P58 | Up needle position adjustment | 0-1439 | 1120 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-1439 | 360 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing Speed（rpm ） | 100-3500 | 3500 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Running Time of Testing B and C | 1-250 | 30 |  |
| P65 | Stop Time of Testing B and C | 1-250 | 10 |  |
| P66 | Machine protection switch selection | 0-2 | 1 | 0：Disable 1：Testing zero signal 2：Testing positive signal |
| P71 | Presser foot release operation duty cycle (%) | 0-50 | 8 | ### The buffer force of presser foot release operation. |
| P72 | Up needle position adjustment | 0-1439 |  | Hand rotation manual to the appropriate position, press S key to save. |
| P73 | Down needle position adjustment | 0-1439 |  | Hand rotation manual to the appropriate position, press S key to save. |
| P76 | Back-tacking operation time to full output（ms） | 10-990 | 250 | Back tacking start action, full output action times. |
| P78 | The start angle of thread clamping | 5-359 | 100 |  |
| P79 | The stop angle of thread clamping | 5-359 | 270 |  |
| P80 | The start angle of thread trimming | 5-359 | 18 | Shear line infeed id angle setting(under defined as 0°) |
| P81 | The angle of the motor start to increase torque when thread trimming | 5-359 | 140 | Torque angle of thread trimming setting(Down position as 0°, the value need greater than the P80 parameter item) |
| P82 | The stop angle of thread trimming | 5-359 | 175 | Feeder back angle of thread trimming setting Down position as 0°, the value need greater than the P80 parameter item) |
| P84 | The angle of the motor start to increase torque when sewing | 0-359 | 9 |  |
| P85 | The angle of the motor stop to increase torque when sewing | 0-359 | 57 |  |
| P87 | The thread wiping back delay times | 10-990 | 50 |  |
| P92 | Correct the electrical angle of motor |  | 160 | Review the user manual 1.5 Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delay time of the half back pedaling function | 10-900 | 100 |  |
| P99 | End Back-Tacking compensation C1 | 0-250 | 120 | Prevent taking off the thread after thread trimming mode, compensation for C part of End Back-Tacking. |
| P100 | End Back-Tacking compensation D1 | 0-250 | 175 | Prevent taking off the thread after thread trimming mode, compensation for D part of End Back-Tacking. |
| P101 | The start angle of thread slacking | 1-359 | 30 | Song line start angle (defined as 0°under calculation) |
| P102 | The stop angle of thread slacking | 1-359 | 180 | Loose wire end angle (defined as 0°under calculation) |
| P109 | The delay time before thread wiping | 0-990 | 5 |  |
| P110 | Trimming back delay time(ms) | 60-990 | 65 |  |
| P117 | The delay time of the presser foot release damping force (ms) | 0-990 | 5 |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 0 | 0: OFF 1: ON |
| P120 | The oil level alarm switch selection | 0-1 | 0 | 0: OFF 1: ON |
| P121 | Tight seam function | 0-1 | 0 | 0: OFF 1: ON |
| P122 | Tight seam mode | 0-1 | 0 | 0: Positive direction 1: Negative direction |
| P131 | End tight seam speed | 100-1200 | 1000 |  |
| P132 | End tight seam stitches | 0-10 | 2 | Tight seam stitches setting before trimming 0: OFF 1-10: Tight seam stitches before trimming |
| P133 | The start angle of tight seam | 20-330 | 100 |  |
| P134 | Start tight seam stitches | 0-10 | 0 | Tight seam stitches setting when machine start running 0: OFF 1-10: Tight seam stitches when machine start running |
| P135 | Start tight seam or manual tight seam speed | 100-1200 | 1000 | Initial speed at the beginning of start tight seam or manual tight seam |
| P136 | Periodic signal of tight seam electromagnet on closing | 1-100 | 100 |  |
| P137 | Periodic signal after tight seam electromagnet on closing | 1-50 | 30 |  |
| P138 | The time of tight seam electromagnet on closing | 10-990 | 115 |  |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem | Status / Measurement |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage.) If correct, please replace the control box and inform the factory |
| E02 | Low-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use.) If correct, please replace the control box and inform the factory |
| E03 | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 | Speed control unit connection is abnormal | Turn off the system power supply, please check the speed control unit connector if loose or fall off, Will resume normal after the restart system If you still cannot work normally, please replace the speed control unit and notify the manufacturer. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E08 | Manual reverse feeding operation more than 15 seconds | Back-Tacking electromagnet response time is too long, you have only to restart the machine. If there are still errors E-08 after restart the machine, check if the manual reverse feeding switch is damaged. |
| E10 | Solenoid over-current protection | Turn off the system power supply, please check the solenoid (solenoid valve) connector or solenoid (solenoid valve) is broken or not. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, renew to normal then restart system. If still not work, please replace the motor and notify the manufacture. |
| E15 | Power module abnormal over current protection | Turn off the system power supply, and then restart the machine. If still not work, please replace the control box and notify the manufacture. |
| E16 | Trimming step motor abnormal | Please check trimming mechanism return correct position or not; please check trimming setting correct or not. |
| E17 | The sewing table safety switch is not in the correct position | Turn off the system power supply, please check machine head is open or not, the machine head switch is damage or not. |
| E20 | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| oil | Low oil | Turn off the machine, check if the oil is too little. To fill it up and turn on. If there are still errors, please replace the control box and call for customer service. |

5 Port Outline Diagram:

5.1 Port Description

14P function port table illustration

12Pfunction port table illustration

Speed controller port city

Power connector socket

Motor connector socket

5.2 14P function port description

1. Thread trimming electromagnet：1、8（+32V）

2. Thread wiping/ thread clamping electromagnet：2、9（+32V）

3. Thread slacking electromagnet：3、10（+32V）

4. LED Light：4（DGND）、11（+5V）

5. Reverse stitch key：5（signal）

6. Back-tacking electromagnet：6、13（+32V）

7. Mending stitch key：7（signal）

5.3 12P function port description

1. Tight Seam electromagnet: 2、8(+32V)

2. Presser foot electromagnet: 6、12（+32V）