Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign   must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well-trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage matches the rated voltage indicated on the motor’s name plate.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker)

1 Button Displays and Operating Instructions

1.1 Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Start Back-tacking Selection / Slow Launch Setting |  | If click, executions starting back seam B segment or execution starting back seam (A, B segment) 1 Execution starting back seam (A, B segment) 2 times. If long-press can set used or cancelled slow launch function. |
| End Back-tacking Selection / Needle Stop Position Selection |  | If click, execution terminates back seam C segment or execution terminates back seam (C, D segment) 1 Execution terminates back seam (C, D segment) 2 time. If long-press, the needle stop position after shift the sewing mode ( Up position / Down position) |
| Free Sewing / Constant Stitch Sewing |  | # If click, it can execute free sewing. If long-press can execute constant stitch sewing. |
| Continuous Back Seam Shortcuts / Multi-section Sewing |  | # If click, it can set consecutive reverse sewing function. If long-press can execute multi-section sewing. |
| Trimming Switch / Clamp Function Setting |  | # If click, it can set used or cancelled thread trimming function. If long-press can set used or cancelled clamp function. |
| Automatic Presser Foot Lifting Setting / Automatically Trigger Button |  | # If click, it can execute the cycle shift of automatic presser foot lifting, automatic presser foot lifting after pause, automatic presser foot lifting after trimming and presser foot lifting is inactive. # If long-press can set used or cancelled automatic function. |
| Parameter Increase |  | Parameter value increase key |
| Parameter Decrease |  | Parameter value decrease key |
| Left Selection Parameter Content |  | 1. If click, left selection parameter content. 2. If long-press can switch to the counter interface |
| Right Selection Parameter Content |  | 1. If click, right selection parameter content. 2. If long-press can switch to P121, P132 and P134 simulation parameters. |
| Function Parameter Edit |  | Press this key to enter or exit function parameter edit mode. At the parameter interface, for the content of the selected parameters check and save. |
| Loosen the Thread when Lifting the Foot |  | No function. |
| Restore Factory |  | Long-press can restore factory setting. |

1.2 Auxiliary function

1.2.1 Needle Stop Position Function Setting

In the main interface, click the P key to enter the parameters, and set the parameters to P72. Manually rotate the handwheel to up position. The value displayed will change with the change of the wheel position. Press S key to save the current position (value) as the upper pin stop and calculate down position automatically.

1.2.2 Correct the Electrical Angle of Motor

Enter the parameter, and set the parameter to P92, press P key into the parameter，and press Needle Stop Position Key, then the motor will automatic running to calculate the electrical angle of motor. After the motor stops, the parameter will automatic change, press P key to save。

1.2.3 Save the parameter value as the initial value.

In the parameter value interface, long press Slow Launch Setting, the display screen will display "SAVE" 1second, then press key P to confirm. After that, the parameter value will revert to the saved parameter value every time the factory setting is restored.

2 User Parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed (rpm) | 100-3700 | 3700 | Maximum speed of machine sewing |
| P02 | Set accelerated curve (%) | 10-100 | 80 | Set the acceleration slope |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle stops at Up position DN: Needle stops at Down position |
| P04 | Start back-tacking speed (rpm) | 200-3200 | 1800 | Set start back-tacking speed |
| P05 | End back-tacking speed (rpm ) | 200-3200 | 1800 | Set end back-tacking speed |
| P06 | Bar-tacking speed (rpm) | 200-3200 | 1800 | Set repeat bar-tacking speed |
| P07 | Soft start speed (rpm ) | 200-1500 | 400 | Set soft start speed |
| P08 | Stitch numbers for soft start | 0-99 | 2 | Soft start stitches setting |
| P09 | Automatic constant-stitch sewing speed (rpm) | 200-3700 | 3700 | Speed adjustment for automatic constant-stitch sewing |
| P10 | Automatic end back-tacking sewing | ON/OFF | ON | ON: After last seam of constant stitch sewing, it will automatic execute the End Back-Tacking sewing function. When turn on, the Stitch-Correction is invalid. OFF: Automatic End Back-Tacking will not execute after last seam, but can be done by pedal manually. |
| P11 | Back-tacking mode selection | J/B | J | J: JUKI Mode (Press TB switch will activate reverse solenoid when either machine is stopped or running). B: BROTHER Mode (Press TB switch will activate reverse solenoid only when machine is running). |
| P12 | Start back-tacking running mode selection | 0-3 | 1 | 0:Could be arbitrarily stop and start 1:Automatically perform actions 2:Pause mode |
| P13 | Ending mode of Start Back-Tacking | CON/STP | CON | CON：Start Back-Tacking is completed automatically continued for next action. STP：After the number of stitches is completed, stop automatically. |
| P14 | Slow start function selection | ON/OFF | OFF |  |
| P15 | Mending stitch mode | 0-4 | 2 | 0:half stitch; 1: one stitch; 2: continuous half stitch; 3: continuous one stitch; 4: continuous one stitch, quick stop. |
| P16 | Back-tacking speed limit | 0-3200 | 0 | When the parameter is 0, the speed limit function is OFF. |
| P17 | Auto count selection | 0-50 |  | 0: P41 item counter not count automatically. 1-50: P41 item counter increment count automatically. |
| P18 | Start back-tacking compensation 1 | 0-200 | 131 | Stitch balance for end back-tacking A section, 0~200 action gradually delay; the large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start back-tacking compensation 2 | 0-200 | 158 | Stitch balance for end back-tacking B section, 0~200 action gradually delay; the large value, the longer of the B section first stitch. |
| P20 | End back-tacking running mode selection | 1-3 | 1 | 1: Automatically perform actions 2: Pause mode |
| P21 | The position of the pedal for running | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for lifting presser foot | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming | 30-500 | 130 |  |
| P25 | End back-tacking compensation 3 | 0-200 | 131 | Stitch balance for end back-tacking C section, 0~200 action gradually delay; the large value, the shorter of the C section first stitch. |
| P26 | End back-tacking compensation 4 | 0-200 | 158 | Stitch Balance for end back-tacking D section, 0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P27-N04 | Speech selection | 0-2 | 1 | 0: OFF 1: Chinese 2: English |
| P27-N05 | Voice broadcast selection | 0-3 | 3 | 0: OFF 1: With welcome speech but without key tone 2: With key tone but Without welcome speech 3: With welcome speech and key tone |
| P28 | Bar-tacking running mode selection | 0-3 | 1 | 0: Could be arbitrarily stop and start 1: Automatically perform actions 2: Pause mode |
| P29 | The rate of thread trimming stop | 1-45 | 20 |  |
| P30 | The strength of the motor torque when sewing | 0-100 | 0 | The larger the value the greater the intensity, the adjustment is too large may cause the motor abnormalities. |
| P31 | The strength of the motor torque when thread trimming | 0-100 | 40 | The larger the value the greater the intensity, the adjustment is too large may cause the motor abnormalities. |
| P32 | Bar-tacking compensation 5 | 0-200 | 131 | Stitch balance for start back-tacking A (C) section, 0~200 action gradually delay; the large value, the longer of the A (C) section last stitch; the shorter of the B (D) section first stitch. |
| P33 | Bar-tacking compensation 6 | 0-200 | 158 | Stitch balance for start back-tacking B (D) section, 0~200 action gradually delay; the large value, the longer of the B (D) section last stitch; the shorter of the C section first stitch. |
| P34 | Constant-stitch sewing running mode selection | A/M | A | A：Automatically perform actions M：Could be arbitrarily stop and start |
| P36 | Thread slacking function selection | 0-11 | 0 | 0: OFF 1-11: The strength of thread slacking |
| P37 | Automatic thread wiping function / Automatic thread clamping function selection | 0-11 | 8 | 0 : OFF 1: Automatic thread wiping Function 2-11: Automatic thread clamping Function and the strength of automatic thread clamping. |
| P38 | Automatic thread trimming function selection | ON/OFF | ON | ON: ON OFF: OFFF |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN | UP: ON DN: OFF |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN | UP: ON DN: OFF |
| P41 | Counter display | 0-9999 |  | Display the quantity of finished sewing piece. |
| P42 | Information display |  |  | N01 The control system version number N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(up position) N06 The mechanical angle(down position) N07 Busbar voltage AD N12 Kneeling device position sensor AD |
| P43 | Direction of motor rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |
| P44 | The rate of normal stop | 1-45 | 12 |  |
| P45 | Back-tacking operation duty cycle (%) | 1-50 | 25 | Back-tacking operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P46 | Motor stop with a reverse Angle after Trimming Function | ON/OFF | OFF | ON: ON OFF: OFF |
| P47 | Adjust the reverse angle when motor stop after trimming | 50-200 | 160 | Adjusting at reverse direction after trimming. |
| P48 | The minimum speed limit (rpm ) | 100-500 | 210 | Adjust the minimum speed |
| P49 | Thread trimming speed (rpm) | 100-500 | 300 | Adjust thread trimming speed |
| P50 | Toot lifter operation time to full output (ms) | 10-990 | 150 |  |
| P51 | Presser foot lifter operation duty cycle (%) | 1-50 | 25 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Motor starting delay time (ms) | 10-990 | 120 | Delay the start time, with automatic presser foot down. |
| P53 | Half back pedaling lifting presser foot function to cancel | ON/OFF | OFF | ON: Function to cancel OFF: Half back pedaling lifting presser foot |
| P54 | Thread trimming action time (ms) | 10-990 | 200 | Completion of the thread trimming action required time. |
| P55 | Wiper / Sweep thread action time | 10-990 | 30 | Completion of the wiper / sweep thread action required time. |
| P56 | Power on and positioning | 0-2 | 1 | 0: always not to find the up needle position 1: always to find the up needle position 2:If the motor is not into the up needle position, it will find the up needle position |
| P57 | Hold time of presser foot lifter (s) | 1-120 | 5 | Forced shut-down after hold time. |
| P58 | Up needle position adjustment | 0-1439 | 40 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-1439 | 750 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed (rpm ) | 100-3500 | 3500 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Running time of testing B and C | 1-250 | 30 |  |
| P65 | Stop time of testing B and C | 1-250 | 10 |  |
| P66 | Machine Protection Switch Selection | 0-2 | 1 | 0：Disable 1：Testing zero signal 2：Testing positive signal |
| P71 | Presser foot release operation duty cycle (%) | 0-50 | 0 | ### The buffer force of presser foot release operation. |
| P72 | Up needle position adjustment | 0-1439 |  | Hand rotation manual to the appropriate position, press S key to save. |
| P73 | Down needle position adjustment | 0-1439 |  | Hand rotation manual to the appropriate position, press S key to save. |
| P76 | Back-tacking operation time to full output (ms) | 10-990 | 250 | Back tacking start action, full output action times. |
| P78 | The start angle of thread clamping | 5-359 | 100 |  |
| P79 | The stop angle of thread clamping | 5-359 | 270 |  |
| P80 | The start angle of thread trimming | 5-359 | 18 | Shear line infeed id angle setting(under defined as 0°) |
| P81 | The angle of the motor start to increase torque when thread trimming | 5-359 | 120 | Torque angle of thread trimming setting(Down position as 0°, the value need greater than the P80 parameter item) |
| P82 | The stop angle of thread trimming | 5-359 | 175 | Feeder back angle of thread trimming setting Down position as 0°, the value need greater than the P80 parameter item) |
| P84 | The angle of the motor start to increase torque when sewing | 0-359 | 9 |  |
| P85 | The angle of the motor stop to increase torque when sewing | 0-359 | 57 |  |
| P87 | The thread wiping back delay times | 10-990 | 50 |  |
| P92 | Correct the electrical angle of motor |  | 160 | Review the user manual 1.5 Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delay time of the half back pedaling function | 10-900 | 200 |  |
| P101 | The Start angle of thread slacking | 1-359 | 30 | Song line start angle (defined as 0°under calculation) |
| P102 | The stop angle of thread slacking | 1-359 | 180 | Loose wire end angle (defined as 0°under calculation) |
| P109 | The delay time before thread sweep | 0-990 | 200 |  |
| P110 | Trimming back time (ms) | 60-990 | 200 |  |
| P117 | The delay time of the presser foot release damping force (ms) | 0-990 | 12 |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 0 | 0: OFF 1: ON |
| P120 | The oil level alarm switch selection | 0-1 | 0 | 0: OFF 1: ON |
| P121 | Tighter seam / Short-thread | 0-3 | 3 | 0: OFF 1: Short-thread 2: Tighter sea 3: Tighter sea + Short-thread |
| P131 | Tight seam speed | 100-1200 | 1000 |  |
| P132 | Tight seam stitches | 0-10 | 2 |  |
| P133 | Tight seam start action angle | 20-330 | 100 |  |
| P139 | Thread slacking when start sewing function selection | 0-1 | 0 | 0: OFF 1: ON |
| P140 | Thread slacking mode setting | 0-2 | 0 | 0: Thread slacking when trimming mode 1: Thread slacking when start sewing mode 2: Thread slacking when protect nest-type stitch mode |
| P141 | The start angle of function that thread slacking when start sewing | 1-359 | 1 | The start angle of function that thread slacking when start sewing.(as upper needle position angle) |
| P142 | The stop angle of function that thread slacking when start sewing | 1-359 | 35 | The stop angle of function that thread slacking when start sewing. (as upper needle position angle, must be greater than P141 parameter values) |
| P146 | Protect nest-type stitch mode, the delay time of thread slacking | 0-990 | 150 |  |
| P147 | Protect nest-type stitch mode, the duration time of thread slacking | 0-990 | 120 |  |
| P148 | Protect nest-type stitch mode, the back tracking time of thread slacking | 0-990 | 30 |  |
| P151 | Thread hooking function selection | 0-1 | 1 | 0: OFF 1: ON |
| P152 | The delay time of thread hooking | 0-990 | 100 |  |
| P153 | The duration time of thread hooking | 0-990 | 30 |  |
| P154 | The back tracking time of thread hooking | 0-990 | 30 |  |
| P155 | Thread hooking operation duty cycle (%) | 0-100 | 90 |  |
| P159 | Suction switch selection | 0-1 | 1 | 0: OFF 1: ON |
| P160 | Suction time (ms) | 5-5000 | 500 |  |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Error Code List

| Error Code | Problem description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage. (Or whether exceed the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage. (Or whether under the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E03 | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 | Pedal connection error | Turn off the machine, please check if the pedal in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E08 | Manual reverse feeding operation more than 15 seconds | Back-Tacking electromagnet response time is too long, you have only to restart the machine. If there are still errors E-08 after restart the machine, check if the manual reverse feeding switch is damaged. |
| E10 | Solenoid over-current | Turn off the machine, please check the related circuit is shorted or damaged. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E15 | Power supply module abnormal over-current | Turn off the machine, then turn on again. If there are still errors, please replace the control box and call for customer service. |
| E17 | Machine protection switch is not in the correct position | Turn off the machine, check if the machine is turn over, or the Mercury Switch is shifted or damaged. |
| E20 | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| oil | Low oil | Turn off the machine, check if the oil is too little. To fill it up and turn on. If there are still errors, please replace the control box and call for customer service. |

4 Port Diagram

4.1 Port Description

4. 2 14P function port description

1. Thread trimming electromagnet: 1, 8 (+32V)

2. Thread wiping/ thread clamping electromagnet: 2, 9 (+32V)

3. Thread slacking electromagnet: 3, 10 (+32V)

4. LED Light: 4 (DGND), 11 (+5V)

5. Reverse stitch key: 5 (signal)

6. Back-tacking electromagnet: 6, 13 (+32V)

7. Mending stitch key: 7 (signal)

4.3 12P function port description

1. Suction: 1, 7(+32V)

2. Hook thread: 2, 8(+32V)

3. Oil level sensor：9 (+5V), 10 (DGND), 4 (signal)

4. Presser foot position sensor：3 (+5V), 11 (DGND), 5 (signal)

5. Presser foot electromagnet：6, 12 (+32V)