Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well-trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage matches the rated voltage indicated on the motor’s name plate.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker).

| 1 Operation Panel for Each Key Name and Function Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Icon | Description |  | Remarks |
| 1 |  | Function key |  | ①In the normal mode, press this key enter to user parameter setting mode. ②In the normal mode, long-press this key enter to technician parameter setting mode. ③In the normal mode, press【P key】＆【S key】at the same time enter to senior parameter setting mode. ④In the parameter setting interface, Press this key to return but not save the parameter. |
| 2 |  | Up key / Speed increase |  | ①In the normal mode, press this key can increase speed, long-press for 3s can enter to speed setting interface. ②In the parameter setting interface, press this key can increase the parameter. |
| 3 |  | Down key / Speed decrease |  | ①In the normal mode, press this key can decrease speed, long-press for 3s can enter to speed setting interface. ②In the parameter setting interface, press this key can decrease the parameter. |
| 4 |  | Left key / Cloth choice |  | ①In the normal mode, long-press this key can enter to cloth choice interface. ②Under the parameter setting interface, press this key can check last parameter. |
| 5 |  | Right key / LED brightness adjustment |  | ①In the normal mode, press this key can adjust LED brightness. ②In the parameter setting interface, press this key can check next parameter. |
| 6 |  | Save key |  | In the parameter setting interface, press this key to save. |
| 7 |  | Forward trimming / Backward trimming key (only apply to horizontal knife type) |  | ①In the normal mode, press this key can enter to forward trimming setting interface, long-press this key for 3s can enter to backward trimming setting interface. ②In the forward trimming setting interface, press this key can execute the cycle shift of forward trimming on, forward trimming off. Press up or down key it can adjust forward trimming action times. (same as P-27) ③In the backward trimming setting interface, press this key can execute the cycle shift of backward trimming on, backward trimming off. Press up or down key it can adjust backward trimming action times. (same as P-28) |
| 8 |  | Thread tension release key (only apply to side knife type) |  | ①In the normal mode, press this key can enter to thread tension release setting interface, long-press this key for 3s can enter to thread tension release and trimming parameter setting interface. ② In the thread tension release setting interface, press this key can execute the cycle shift of thread tension release off, forward thread tension release on, backward thread tension release on, forward and backward thread tension release on. |
| 9 |  | Operation mode selection / Auto-suction mode key |  | ①In the normal mode, press this key can enter to operation mode selection interface, long-press this key for 3s to can enter to auto-suction setting interface. ②In the operation mode selection interface, press this key can execute the cycle shift of the following four modes: 1. Full automatic: start-up according full sensor signal. 2. Free sewing. 3. Semi- automatic: start-up according to front sensor signal + pedal completion 4. Full manual: start-up according to the pedal completion. ③In the auto-suction setting interface, press this key can execute the cycle shift of suction off, forward suction on, backward suction on, suction on. |
| 10 |  | Presser foot lifting / Sensor key |  | ①In the normal mode, press this key it can enter to presser foot lifting mode setting interface. Long-press this key for 3s can check sensor real-time sensor value and current threshold value. ②In the presser foot lifting mode setting interface, press this key can execute the cycle shift of foot lifting off, front foot lifting, rear foot lifting, front and rear foot lifting. ③In the parameter setting interface, long-press this key can save the parameter just saved as the user default parameter. (The current parameter will not be restored by reset key on the operation panel.) |
| 2 Special Function Operation Definition |  |  |  |  |
| No. | Function name |  | Function Description |  |
| 1 | Reset |  | ①In the normal mode, press 【Left key】＆【Right key】at the same time until "RESET" is displayed on the interface. ②In the normal mode, insert the needle into the hole and press it until "RESET" is displayed on the interface. |  |
| 2 | UP position adjustment |  | 1. In the normal mode, press【P key】＆【S key】at the same time, display P-70; 2. Adjust parameter P-72; 3. From hand-wheel direction, clockwise turn hand-wheel to up position, press 【S key】 to save current value. |  |
| 3 | Down position adjustment |  | 1. In the normal mode, press【P key】＆【S key】at the same time, display P-70; 2. Adjust parameter P-73; 3. From hand-wheel direction, clockwise turn hand-wheel to down position, press 【S key】 to save current value. |  |
| 4 | Fabric recognition mode 1 |  | 1. In the normal mode, press 【Up key】＆【Down key】 at the same time to enter sensor sensitivity adjustment interface; 2. Please remove cloth after three-sensor, press【Presser foot lifting/ Sensor key】; 3. Please cover cloth after three-sensor, press【Presser foot lifting/ Sensor key】, finished setting. |  |
| 5 | Fabric recognition mode 2 |  | In the normal mode, press【P key】＆【Presser foot lifting / Sensor key】at the same time to enter fabric recognition mode , select the type of fabric to be recognized according to the prompts, and wait for the completion of fabric recognition to return to the main interface. |  |
| 6 | Piecework function |  | In the normal mode, press【Forward trimming / Backward trimming key 】＆【Presser foot lifting / Sensor key】at the same time to enter piecework interface. |  |
| 7 | One-key test function |  | In the normal mode, press 【Forward trimming / Backward trimming key】＆【Operation mode selection / Auto-suction mode key】 to enter one-key test parameter interface. |  |
| 8 | Lock screen function |  | Under normal state, the panel will automatically enter the lock screen state when no operation time reaches P-93 (lock screen time), and a "lock" icon will be displayed on the right side of the main interface. When locking the screen, press【P key】to unlock it. |  |

| 3 System Parameter Setting Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| User Parameter (In the normal mode, press【P key】to access) |  |  |  |  |
| No | Function parameters | Range | Default | Description |
| P-01 | Sewing Speed(r/min) | 100-7000 | 5000 | Maximum sewing speed setting |
| P-02 | Needle stop positioning selection | 0-1 | 0 | 0: Up needle position 1: Down Needle position |
| P-03 | Start sewing speed(r/min) | 100-6000 | 5000 | With trimming function: the speed setting for front sensor trigger start-up and Back sensor signal finish (before trimming) |
| P-04 | Start-up mode | 0-1 | 1 | (Valid for full-automatic mode) 0: Automatic mode 1: Foot control mode(Start-up by Front sensor + toe down the pedal) |
| P-05 | Automatic/semi-automatic mode selection | 0-1 | 1 | (P-06 is turn on) 0: Semi-automatic (Match with P-04 use) 1: Automatic (Foot control mode) |
| P-06 | Auto sensing switch | 0-1 | 1 | 0: OFF (Full manual mode is turn on) 1: ON (It can execute automatic/ semi-automatic mode also match with P-05 use) |
| P-07 | Auto trimming switch | 0-3 | 3 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: Front trimming (Mid-sensor receive signal then trimming) 2: Rear trimming (Back-sensor receive signal after finished trimming) 3: First and later trimming |
|  | Automatic thread tension release switch | 0-1 | 1 | (P-70=Side knife type, this parameter is valid) 0: OFF 1: Backward thread tension release |
| P-08 | Auto suction | 0-3 | 0 | 0: OFF 1: Front trimming suction 2: Rear trimming suction 3: Front and rear trimming suction |
| P-09 | Automatic cloth feeding | 0-1 | 0 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: ON |
| P-10 | Auto presser foot lifting | 0-3 | 0 | 0: OFF 1: Front foot-lifting(Front sensor receive signal) 2: Rear foot-lifting(Rear sensor signal after finish) 3: Front and Rear foot-lifting |
| P-11 | Presser foot lifting when stop on the sewing | 0-1 | 0 | (Valid for full-manual and semi-automatic mode) 0: OFF 1: ON (when stop on the sewing the presser foot lifting automatically) |
| P-12 | Presser foot lifting after trimming | 0-1 | 1 | 0: OFF 1: ON |
| P-13 | Presser foot lifting when half back treadle | 0-2 | 0 | (Valid for full-manual / semi-automatic mode) 0: Presser foot lifting when half back step/ back step 1: Lay down 2: Reverse step up presser foot |
| P-14 | Manual trimming suction | 0-2 | 0 | 0: No suction 1: Rear suction 2: Front suction and rear suction |
| P-15 | Machine head lamp brightness | 0-4 | 3 | 0: OFF 1→4 the brightness grade, the more value the more brightness. |
| P-16 | Low air pressure mode | 0-1 | 0 | 0: Normal mode 1: Low pressure mode |
| P-17 | Semi-automatic continuous sewing | 0-1 | 1 | 1: OFF 0: Continuous sewing when toe down the treadle (P-70= Horizontal knife type, Default: 1) (P-70= Side knife type, Default: 0) |
| P-18 | Semi-automatic constant rate trimming | 0-1 | 0 | 0: OFF (after finish before trimming not execute P-03 speed) 1: Constant trimming speed the back sensor speed is fixed with P-03 |
| P-19 | Backward thread tension release switch selection | 0-1 | 1 | (P-70=Side knife type, this parameter is valid) 0: Front sensor trigger 1: Rear sensor trigger |
| P-20 | Stop position after trimming selection | 0-1 | 1 | 0: Down position 1: Up position |
| P-21 | Front thread action time(stitches) | 0-50 | 5 | (P-70=Side knife type, this parameter is valid) |
| P-22 | Front Receiver E# switch | 0-1 | 1 | 0: ON (turn on the front sensor, sensor signal + pedal) 1: OFF (Close front sensor, pedal direct turn on) (P-70= Horizontal knife type, Default: 0) (P-70= Side knife type, Default: 1) |
| P-23 | Intermittent suction opening time(×100ms) | 1-600 | 20 | The more value the more time |
| P-24 | Intermittent air suction off time (×100ms) | 0-600 | 0 | 0: for no intermittent air suction Front suction off time (valid when P-46 function setting is 1) |
| P-25 | Low pressure mode, selvage suction synchronization time (P16=1) | 0-2000 | 200 | Valid for P-16 setting is 1. |
| P-26 | The stitches number between two sensors | 1-600 | 100 | Cycle period parameter. In a circle the rear sensor receive signal then can continue running, otherwise a cycle stop. |
| P-27 | The delayed stitches number of front trimming | 0-50 | 4 | (P-70= Horizontal knife type, this parameter is valid) The smaller value the more in advance, thread will retain longer. (Mid-sensor trigger ) |
| P-28 | The delayed stitches number of rear trimming | 0-50 | 4 | (P-70= Horizontal knife type, this parameter is valid) The small value the more in advance, the short of end-thread (Back-sensor trigger) |
|  | The starting time for rear thread | 0-50 | 2 | (P-70= Side knife type, this parameter is valid) |
| P-29 | The starting stitches for front suction(stitches) | 0-50 | 1 | (P-70= Side knife type, this parameter is valid) |
| P-30 | The delayed time of front suction off (ms) | 100-5000 | 100 | (P-70= Horizontal knife type, this parameter is valid) |
|  | Number of stitches when forward suction OFF (stitches) | 0-50 | 25 | (P-70= Side knife type, this parameter is valid) |
| P-31 | The delayed time after trimming suction (ms) | 100-5000 | 300 | The small value the close is fast. (P-70= Horizontal knife type, Default: 300) (P-70= Side knife type, Default: 1000) |
| Technician Parameter(In the normal mode, long-press【P key】to access) |  |  |  |  |
| P-32 | The times when cloth-pulling ON (stitches) | 0-50 | 20 | (P-70= Horizontal knife type, this parameter is valid) |
| P-33 | Times of backward suction ON (stitches) | 0-50 | 3 | (P-70= Side knife type, this parameter is valid) |
| P-34 | Trimming protection by manually | 0-1 | 1 | (P-70= Horizontal knife type, this parameter is valid) 0: OFF 1: ON |
| P-35 | The delayed stitches number before the machine stops (Stitches) | 1-99 |  | After through cloth back-sensor how many stitches will stop the machine automatically. (Valid for Back-sensor OFF condition) (P-70= Horizontal knife type, Default: 1) (P-70= Side knife type, Default: 3) |
| P-36 | The response time of front sensor (ms) | 10-990 | 50 | Front sensor trigger time: the small value the response is fast, the large value the response is slow. |
| P-37 | The sensitivity of front sensor | 0-900 | 480 | In order to adapt different cloth material setting front sensor receive strength. |
| P-38 | The sensitivity of middle sensor | 0-900 | 540 | In order to adapt different cloth material setting mid-sensor receive strength. |
| P-39 | The delaying time of front presser foot lifter (ms) | 100-2000 | 200 | (Under the full automatic/ semi-automatic) Front trigger retention time after auto-foot-lifting, the larger value the longer retention time. |
| P-40 | Starting time for rear presser foot lifter(ms) | 0-2000 | 120 | The starting time of rear presser foot lifting, the smaller value the faster of response. |
| P-41 | Full-on time setting for presser foot(ms) | 10-990 | 100 | The larger value the higher of foot-lifting (Noted: Not too high) |
| P-42 | Duty-cycle time setting for presser foot (%) | 10-90 | 20 | Adjustment for duty-cycle of AFL. Fine tuning can reduce the over- heating. |
| P-43 | The time for the presser foot laying down (ms) | 10-990 | 20 | The action time of presser foot laying down sequence. |
| P-44 | Presser foot protection(s) | 1-120 | 5 | Stop on the sewing the presser foot lifting, stop after trimming presser foot lifting, positive closing after heeling pedal for retention time. |
| P-45 | Trimming time (ms) | 10-990 | 40 | (P-70= Horizontal knife type, this parameter is valid) the longer time, the greater trimming pressure |
|  | Tension release times (ms) | 0-5000 | 1000 | (P-70= Side knife type, this parameter is valid) |
| P-46 | Suction when continuous feeding | 0-2 |  | 0: No suction 1: Long suction 2: Synchronizer suction (P-70= Horizontal knife type, Default: 0) (P-70= Side knife type, Default: 2) |
| P-47 | Total pieces | 0-65535 | 0 |  |
| P-48 | Needle goes up as power on | 0-1 | 1 | 0: OFF 1: ON |
| P-49 | Stitch(mm) | 1-7 | 3 | Adjustment same as actual needle distance by up and down key. |
| P-50 | Up needle position adjustment | 0-24 |  | Same as P-72 |
| P-51 | Down position adjustment | 0-24 |  | Same as P-73 |
| P-52 | Test speed(r/min) | 100-6000 | 5500 | Adjustment by up and down key |
| P-53 | Test working time (×100ms) | 1-250 | 50 | Adjustment by up and down key |
| P-54 | Test stop time (×100ms) | 1-250 | 20 | Adjustment by up and down key |
| P-55 | Item A testing: Continuous running | 0-1 | 0 | 0: STOP 1: RUN |
| P-56 | Item B testing: With function running | 0-1 | 0 | 0: STOP 1: RUN |
| P-57 | Item C testing: Without function running | 0-1 | 0 | 0: STOP 1: RUN |
| P-58 | Machine plate protection switch | 0-1 | 1 | 0: OFF 1: ON |
| P-59 | Presser foot protection switch | 0-1 | 1 | 0: OFF 1: ON |
| P-60 | Electric /Air-powered | 0-1 | 0 | 0: Electric 1: Air-powered |
| P-61 | Trimming switch by heeling pedal | 0-7 | 0 | 0: All OFF 1: All ON by manually 2: Semi-auto ON 3: Auto ON 4: Manual and semi-auto ON 5: Semi-auto and auto ON 6: Manual and auto ON 7: Manual and semi-auto and auto ON |
| P-62 | Rotate direction |  | 0 | 0: Corotation (Clockwise) |
| P-63 | Language | 0-1 | 1 | 0: English 1: Chinese |
| P-64 | The strength of front sensor | 0%-100% | 50% | Adjustment front sensor strength |
| P-65 | The strength of mid-sensor | 0%-100% | 50% | Adjustment mid-sensor strength |
| P-66 | The strength of back sensor | 0%-100% | 50% | Adjustment back sensor strength |
| P-67 | The sensitivity of back sensor | 0-900 | 530 | In order to adapt different cloth material setting back-sensor receive strength. |
| P-68 | Cutter speed by manually (r/min) | 100-6500 | 1000 | (P-70= Side knife type, this parameter is valid) |
| P-69 | Response time of back sensor(ms) | 0-3000 | 0 | Response time of back-sensor, when sewing such as mesh cloth can adjust this item, then can achieve you want effects. |
| Advanced parameter setting (In the normal mode, press【P key】＆【S key】at the same time to access) |  |  |  |  |
| P-70 | Model selection | 0-1 |  | 0: Horizontal knife type 1: Side knife type 2: Wrap the thread |
| P-71 | Maximum speed limit (r/min) | 100-7000 | 5500 | Maximum speed limit |
| P-72 | Manually up positioning adjustment | 0-24 |  | Through hand-wheel direction, clockwise turn hand-wheel to up position press [S] to save the current value. |
| P-73 | Manually down positioning adjustment | 0-24 |  | Through hand-wheel direction, clockwise turn hand-wheel to down position then press [S] to save current value. |
| P-74 | Parameter reference | 1-5 | 1 | Use up and down key shift: N1: Control box software version N2: Panel software version N3: Rotation N4: Pedal AD N5: Driver software version |
| P-75 | Pedal type | 0-1 | 0 | 0: Normal pedal 1: Stand type pedal |
| P-76 | Password Setting | 0-9999 | 8012 |  |
| P-77 | Zero-point angle setting |  |  | No function |
| P-78 | Suction bucket type | 0-2 | 1 | 0: With brush 1: Without brush 2: Valve |
| P-79 | One key test | 0-1 | 0 | 0: OFF 1: ON |
| P-80 | The needle bar shield protection switch | 0-1 | 0 | 0: OFF 1: ON |
| P-81 | Electromagnet protection | 0-1 | 1 | 0: OFF 1: ON |
| P-82 | Alarm pieces | 0-9990 | 0 |  |
| P-83 | The number of standby display pieces | 0-1 | 0 | 0: OFF 1: ON |
| P-84 | Piecework mode selection | 0-1 | 0 | 0: Ascending 1: Descending |
| P-85 | Piece trimming times | 0-50 | 1 |  |
| P-86 | Voice volume | 0-4 | 4 |  |
| P-87 | Voice broadcast selection | 0-3 | 3 |  |
| P-88 | User model |  | 2 |  |
| P-89 | Reserved |  |  |  |
| P-90 | Voice chip selection | 0-1 | 1 |  |
| P-91 | Rear sensor switch | 0-1 | 1 | 0: OFF 1: ON After the back sensor is turned off, the back sensor is based on the second sensor (0 = 7 pins). |
| P-92 | Reserved |  |  |  |
| P-93 | Lock screen time (s) | 0-300 | 20 | 0: Turn off the lock screen |
| P-94 | Thin light transmission | 1-800 | 10 | The smaller the number, the stronger the sensitivity. The larger the number, the weaker the sensitivity. |
| P-95 | Normal material light transmission | 1-800 | 50 | The smaller the number, the stronger the sensitivity. The larger the number, the weaker the sensitivity. |
| P-96 | Grid material light transmission | 1-800 | 20 | The smaller the number, the stronger the sensitivity. The larger the number, the weaker the sensitivity. |
| P-97 | Number of special material detection needles | 0-50 (stitch) | 7 | The larger the grid, the larger the number of stitches to be detected. |
| P-98 | Trimming speed switch | 0-4 | 0 | 0: Close 1: Open front 2: Open rear 3: Open front and rear |
| P-99 | Front trimming protection | 0-1 | 0 | 0: ON 1: OFF |
| P-100 | Sewing fabric type | 0-2 | 0 | 0: Thin material 1: Normal material 2: Grid material |
| P-101 | Difference of front induction recognition signal | 0-50 | 10 | Set the transmission strength of the front sensor to suit different fabrics. |
| P-102 | Difference of middle induction recognition signal | 0-50 | 10 | Set the transmission strength of the middle sensor to suit different fabrics. |
| P-103 | Difference of rear induction recognition signal | 0-50 | 10 | Set the transmission strength of the rear sensor to suit different fabrics |
| P-104 | Pedal acceleration slope | 1-50 | 32 | The higher the number, the faster the pedal response |
| P-105 | Speed ratio | 0-100 | 33 | The higher the number, the higher the speed (real number) |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem | Status / Measurement |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage) If correct, please replace the control box and inform the factory |
| E02 | Low-voltage | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use ) If correct, please replace the control box and inform the factory |
| E03 | Abnormal communication between the main CPU and the driver CPU | Turn off the machine and turn it back on. |
| E05 | Speed control unit connection is abnormal | Turn off the system power supply, please check the speed control unit connector if loose or fall off, Will resume normal after the restart system If you still cannot work normally, please replace the speed control unit and notify the manufacturer. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E10 | Solenoid over-current protection | Turn off the system power supply, please check the solenoid (solenoid valve) connector or solenoid (solenoid valve) is broken or not. |
| E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, renew to normal then restart system. If still not work, please replace the motor and notify the manufacture. |
| E15 | Power module abnormal over current protection | Turn off the system power supply, and then restart the machine. If still not work, please replace the control box and notify the manufacture. |
| E16 | Trimming step motor abnormal | Please check trimming mechanism return correct position or not; please check trimming setting correct or not. |
| E17 | The sewing table safety switch is not in the correct position | Turn off the system power supply, please check machine head is open or not, the machine head switch is damage or not. |
| NC | Front operation panel linked to CPU interface had communication error. | Turn off the machine, check if the operation box if loose or fall off. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E23 | Manual thread cutting lock | Press the P key to cancel the alarm. When using the thread cutting function, try to avoid the ultra-fast continuous press of the manual thread cutting key. |

5 Port Diagram

5.1 Electric machine type port

5.2 Air-powered / side knife machine type port

5.3 4P electric suction bucket port description

Electric suction bucket: 2 (N line）、3 (GND)、4 (L line）

5.4 8P function port description

Forward trimming: 1（-）、3（+）

Presser foot emitter: 2（-）、3（+）

Backward trimming emitter: 3（+）、4（-）

LED lamp: 5（+）、6（-）

Manual trimming: 3（+5V）、7（signal）、8（GND）

5.5 Presser foot receiver port description

Presser foot receiver: 1（+）、2（-）

5.6 Forward trimming emitter socket port description

Forward trimming receiver: 1（+）、2（-）

5.7 4P function port description

Machine plate safety switch: 1（signal）、2（GND）、4（+5V）

Backward trimming receiver: 2（-）、3（+）

5.8 6P function port description

Presser foot lift safety switch: 1（+5V）、2（GND）、3（signal）

5.9 Trimming port description

Trimming: 1、2（+32V）

5.10 Cloth puller port description

Cloth puller: 1、2（+32V）

5.11 Presser foot port description

Presser foot: 1、2（+32V）

5.12 14P function port description

Trimming: 1(-)、8(+)

Backward suction: 2(-)、9(+)

Cloth puller: 3(-)、10(+)

Presser foot: 6(-)、13(+)

Forward suction: 7(-)、14(+)