安全指示

- 在安装或使用本产品前，使用者必须详细阅读本操作手册。

- 本产品须由受过正确训练的人员来安装或操作。安装作业时必须关闭所有电源，切记不可带电操作。

3） 所有标有符号的指示，必须特别注意并按照说明书上的执行，以免造成不必要的损害。

4） 为安全起见，禁止以延长线作电源座供应二项以上的电器产品使用。

5） 在连接电源线时，必须确定工作电压低于AC 250V，且符合本产品标识中规定的额定电压值。

※注意  ：电控箱电源规格如为AC220V时，请勿插接至AC380V的电源插座上，否则将出现异常且电机无法动作。此时请立即关闭电源开关，重新检查电源。持续供应380V超过五分钟以上，将可能烧损电控箱内器件，而危及人身安全。

6） 请不要在日光直接照射的场所、室外及室温45℃以上或0℃以下的场所操作。

7） 请不要在暖气（电热器）旁、有露水的场所及在相对湿度10%以下或90%以上的场所操作。

8） 请不要在灰尘多的场所、具有腐蚀性物质的场所及有挥发性气体的场所操作。

9） 请注意所有电源线、信号线、接地线等接线时不要受压或过度扭曲，以确保使用安全。

10）电源线的接地端须以适当大小的导线和接头连接到生产工厂的系统地线，此连接必须被永久固定。

11）所有可转动的部分，必须以所提供的零件加以防范露出。

12）在安装完成第一次开电后，先关闭切线功能以低速操作缝纫机并检查转动方向是否正确、运转是否稳定。

13）在进行以下操作前，请先关闭所有电源：

1.在控制箱与马达上插拔任何连接插头时。

2.穿针线时。

3.翻抬缝纫机机头时。

4.修理或做任何机械上的调整时。

5.机器闲置不用时。

14）修理或高层次的保养工作，仅能由受过训练的机电技师来执行。

所有维修用的零件，须由本公司提供认可，方可使用。

15）使用本产品请远离高频电磁波和电波发射器等，以免所产生的电磁波干扰伺服驱动装置而发生误动作。

16）请不要以不适当物体来敲击或撞击本产品及各装置。

保修期限

本产品保修期限为购买日期起一年内或出厂月份起两年内。

保修内容

本产品在正常情况使用且无人为操作失误的前提下，于保修期间无偿为客户维修使能正常操作。

但以下情况于保修期间将收取维修费用：

1.不当使用包括误接高压电源、将产品移做其它用途、自行拆卸、维修、更改、或不依规格范围使用、进水进油及插入异物于本产品。

2.火灾、地震、闪电、风灾、水灾、盐蚀、潮湿、异常电压及其它天灾或不当场所造成的损害。

3.客户购买后摔落本产品，或客户自行运输（或托付运输公司）造成的损害。

* 本产品在生产及测试上皆尽最大努力和严格控制使其达到高品质及高稳定的标准，但外部的电磁或静电干扰或不稳定的供应电源，仍可能对本产品造成影响或损害，因此操作场所的接地系统一定要确实做好，并建议用户安装故障安全防护装置（如漏电保护器）

1.1按键说明示意图

1.2: 按键说明

| 名称 | 按键 | 注明 |
| --- | --- | --- |
| 进入参数区功能键 |  | 1、一般模式下按下P键进入用户参数设置模式 2、在关机状态，按住P开机进入技术员参数模式 |
| 参数查看保存键 |  | 对所选参数号内容进行查看和保存：选择好参数号后按此键可以进行查看和修改操作，修改参数值后按此键则退出并保存参数 |
| 上调节键 |  | 1、参数选择区内参数项递增键 2、参数内容区内设定数值递增键 |
| 下调节键 |  | 1、参数选择区内参数项递减键 2、参数内容区内设定数值递减键 |
| 左位移键 |  | 在参数选择区和参数内容区内，向左位移 |
| 右位移键 |  | 在参数选择区和参数内容区内，向右位移 |
| 上下停针 |  | - 第二个小数点亮表示停车时在上停针位 - 第二个小数点灭表示停车时在下停针位 |
| 剪线开关 |  | - 长按按键，第四个小数点亮表示剪线打开 - 长按按键，第四个小数点灭表示剪线关闭 |
| 恢复出厂设置键 |  | 长按3S后恢复出厂设置 |

2  参数说明

2.1  用户参数 (开机后 长按P键3秒进入)

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P01 | 最高转速（rpm） | 200-5000 | 3500 | 车缝时的最高转速设定 |
| P02 | 加速曲线调整（%） | 10-100 | 80 | 控速器爬升斜率设定 斜率值愈大，速度愈陡；斜率值愈小，速度愈慢 |
| P03 | 针停定位选择 | 0-1 | 1 | 0：上停针 1：下停针 |
| P07 | 慢速起缝速度（rpm） | 200-1500 | 800 | 慢速起缝时的速度设定 |
| P08 | 慢速起缝针数（针） | 0-99 | 1 | 慢速起缝时的针数设定 |
| P14 | 慢速启动 | 0-1 | 1 | 1：慢速启动功能开启 0：慢速启动功能关闭 |
| P15 | 补针方式 | 0-2 | 2 | 0：半针，1：一针，2：连续补半针； |
| P21 | 脚踏板前踏点电压 | 30-1000 | 520 |  |
| P22 | 脚踏板中踏点电压 | 30-1000 | 480 |  |
| P23 | 脚踏板半反踏点电压 | 30-1000 | 240 |  |
| P24 | 脚踏板反踏点电压 | 30-500 | 110 |  |
| P29 | 剪线后刹车力度 | 1-50 | 30 |  |
| P35 | 计件数模式选择 | 0-1 | 0 | 0：手动计件数 1：自动计件数 |
| P38 | 剪线出力功能设定 | 0-1 | 1 | ON：有剪线动作 OFF：无剪线动作 |
| P41 | 车缝件数与针数查看项 | 0-9999 | 0 | 自动累计完成件数或针数。 |
| P42 | 信息显示 | N01-N07 | N01 | N01 电控版本号 N02 选针盒版本号 N03 转速 N04 脚踏板AD N05 上定位角度 N06 下定位角度 N07 母线电压AD |

2.2  技术员参数内容表（按P键+S键开机进入）

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P44 | 刹车力度 | 1-50 | 25 | 机器停车时的力度选择。 |
| P46 | 剪线后，反转提针角度的功能选择 | 0-1 | 0 | ON：切完线后，自动作反转的功能。（角度由【P47.TR8】调整决定） OFF：无作用 |
| P47 | 剪线后，反转提针角度的调整 | 50-200 | 160 | 切完线后，由针上算起以反向运转作提针的角度调整。 |
| P48 | 低速（定位速度）（rpm） | 100-500 | 210 | 定位速度设定 |
| P49 | 剪线速度（rpm） | 100-500 | 250 | 调整剪线周期时的电机速度 |
| P54 | 剪线动作时间（ms） | 5-990 | 200 | 剪线时序所需的动作时间 |
| P56 | 开电后自动找上定位 | 0-2 | 2 | 0：始终不找上定位 1：始终找上定位 2：若电机已经处于上定位时不再找上定位（仅限于有磁钢电机） |
| P58 | 上定位调整 | 0-1799 | 1125 | 上定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P59 | 下定位调整 | 0-1799 | 225 | 下定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P60 | 测试速度（r/s） | 100-4000 | 2000 | 设置测试速度 |
| P61 | A项测试 | 0-1 | 0 | A项测试选项，设定后将按【P60. TV】所设定之速度持续运行 |
| P62 | B项测试 | 0-1 | 0 | B项测试选项，设定后将按【P60. TV】所设定之速度执行启动-车缝-停车-剪线的循环 |
| P63 | C项测试 | 0-1 | 0 | C项测试选项，设定后将按【P60. TV】所设定之速度执行无定位动能的启动-车缝-停车的循环 |
| P64 | 测试B、C导通时间 | 1-250 | 20 | B、C项测试中，设置导通时间 |
| P65 | 测试B、C停车时间 | 1-250 | 20 | B、C项测试中，设置停车时间 |
| P66 | 机头保护开关检测 | 0-2 | 1 | 0：不检测 1：检测零信号 2：检测正信号 |
| P72 | 上停针位校正 |  |  |  |
| P73 | 下停针位校正 |  |  |  |
| P80 | 剪线进刀角度 | 0-359 | 18 | 剪线进刀角度设置（下定位为0°计算） |
| P81 | 剪线加力角度 | 0-359 | 110 | 剪线加力角度设置（下定位为0°计算） |
| P82 | 剪线退刀角度 | 0-359 | 175 | 剪线退刀角度设置（下定位为0°计算） |
| P84 | 厚料开始加力角度 | 0-359 | 9 |  |
| P85 | 厚料结束加力角度 | 0-359 | 57 |  |
| P92 | 编码器起始角度 |  |  | 按夹线键读取编码器起始角度，出厂已设置，请勿随意更改（随意更改会 |

3  错误代码表

| 错误码 | 内容 | 对策 |
| --- | --- | --- |
| E01 | 1）电源ON时，主电压检测过高 2）供应电源电压过高时 | 关闭系统电源，检测供应电源电压是否正确。（或是否超过使用规定的额定电压）。 若正确，请更换控制箱并通知厂方。 |
| E02 | 1）电源ON时，主电压检测过低 2）供应电源电压过低时 | 关闭系统电源，检测供应电源电压是否正确。（或是否低于使用规定的额定电压）。 若正确，请更换控制箱并通知厂方。 |
| E05 | 控速器接触异常 | 关闭系统电源，检查控速器接头是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控速器并通知厂方。 |
| E07 | a)马达插头配线接触不良导致不转 b)车头机构死锁或马达皮带异物卷入卡死。 c)加工物过厚，马达扭力不足无法贯穿。 d模块驱动出力异常 | 转动机头电机手轮观察是否卡住。如卡住则先排除机头机械故障。 如转动正常，检查电机编码器接头和电机电源线接头是否松动。如有松动请修正。 如接触良好，检查供应电源电压是否异常或转速设置过高。如有请调整。 如正常，请更换控制箱并通知厂方。 |
| E10 | 电磁铁过流保护 | 关闭系统电源，检查电磁铁（电磁阀）连接线或电磁铁（电磁阀）是否损坏。 |
| E09 E11 | 定位信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知厂方。 |
| E14 | 编码器信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知厂方。 |
| E15 | 电力模块不正常过流保护 | 关闭系统电源，再重新开启。若仍不能正常工作，请更换控制箱并通知厂方。 |
| E17 | 机头保护开关没到正确位置 | 关闭系统电源，检查机头是否掀开，控制箱内滚珠开关是否移位或损坏。 |
| E20 | 开机电机启动失败 | 关闭系统电源，检查电机编码器接口和电机电源接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控制箱并通知厂方。 |

4：端口示意图

4.1：各个端口说明：

Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign   must be observed or executed; otherwise, bodily

injuries might occur.

- The product should be installed and pre-operated by well trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet

for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the  power voltage is lower than 250 VAC and matches the rated voltage indicated on the motor’s name plate.

※Attention: If the Control Box is AC 220V system, please don’t connect the Control Box to AC 380V power outlet. Otherwise, the error will occur and motor will not work. If that happens, please turn off the power immediately and check the power voltage.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

14． Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power

voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note:  We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  ( Such as residual current breaker).

1.1 Keys illustration diagram

1.2  Keys illustrations

| Name | Key | Descripition |
| --- | --- | --- |
| Enter to parameter area function key |  | 1、Under the normal mode press P key enter to user parameter setting mode 2、In the shut-off condition, press P to turn on the machine enter to the technician parameter mode. |
| Parameter check and save key |  | To check and save the parameter item contents: after select parameter item then press this key can check and modify operation, after modified the parameter value press this key to exit and save the value. |
| Up adjustment key |  | 1、In the parameter selection area parameter item increment key 2、In the parameter contents area setting value increment key |
| Down adjustment key |  | 1、In the parameter selection area parameter item decrement key 2、In the parameter contents area setting value decrement key |
| Left key |  | In parameter item and parameter contents, turn left moving |
| Right key |  | In parameter item and parameter contents, turn right moving |
| Up/down needle position |  | 1、The second decimal point LED ON indicate on the up needle position when stop machines. 2、The second decimal point LED OFF indicate on the down needle posion when stop machines. |
| Trimming switch |  | 1、Long press this key, the fourth decimal point LED ON indicate trimming is open 2、Long press this key, the fourth decimal point LED OFF indicate trimming is off. |
| Restore factory setting |  | Long press 3 sec restore factory setting |

2  Parameter definition

2.1   User parameter ( turn on the machine long-press P key 3 sec to enter)

| No. | Function parameter | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum Sewing Speed（r/S） | 200-5000 | 3500 | Maximum sewing speed setting |
| P02 | Speed Curve Adjustment（%） | 10-100 | 80 | The speed controller climbing slope setting. The large of slope value = the fast of the speed. The small of slope value = the slow of the speed. |
| P03 | Needle UP / DOWN Selection | 0-1 | 1 | Same as panel shortcut function key 0: Needle Stops at Up Position 1: Needle Stops at Down Position |
| P07 | Soft Start Speed（r/S ） | 200-1500 | 800 | Soft Start Speed Adjustment |
| P08 | Stitch Numbers for Soft Start（SLS） | 0-99 | 1 | Soft Start Stitches Setting ( one unit = half stitch) |
| P14 | Soft Start | 0-1 | 1 | 1：Soft start function is turn on. 0: Soft start function is turn off. |
| P15 | Stitch Compensation Mode | 0-2 | 2 | 0： Fill half needle，1：Fill a needle 2：Continuous half needle； |
| P21 | The voltage of step prior to the pedal | 30-1000 | 520 |  |
| P22 | The voltage of half step prior to pedal | 30-1000 | 480 |  |
| P23 | The voltage of half heeling to pedal | 30-1000 | 240 |  |
| P24 | The voltage of reversing pedal | 30-500 | 110 |  |
| P29 | Brake pressure after trimming | 1-50 | 30 |  |
| P35 | Counting mode selection | 0-1 | 0 | 0：Manual counting 1：Automatic Counting |
| P38 | Trimmer Function Selection | 0-1 | 1 | ON：Trimmer Valid OFF: Trimmer Invalid |
| P41 | Display the sewing finished quantity | 0-9999 | 0 | Counting the finished-sewing quantity |
| P42 | Information Display | N01-N07 | N01 | NO1 Electrically controlled version serial numbers NO2 Selected needle cassette version NO3 Speed NO4 Pedals AD, NO5 Positioning angle NO6 Under the positioning angle NO7 Bus voltage AD N12 Knee position sensor AD |

2.2  Technician parameter contents list (Press P key + S key to turn on the machine access)

| No. | Function parameter | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P44 | Brake efforts | 1-50 | 25 | Adjustment of efforts while machine stop |
| P46 | Inversion of the needle angle of choice after trimming | 0-1 | 0 | ON: motor automatic running after trimming(needle angle decided by the setting of 【P47.TR8】 OFF: inaction |
| P47 | Adjustment of reverse angles after trimming | 50-200 | 160 | Reverse running is subject to the adjustment of needle angle after trimming |
| P48 | Low (Positioning) Speed（rpm ） | 100-500 | 210 | Setting Positioning Speed |
| P49 | Trimming Speed（rpm） | 100-500 | 250 | Adjustment of motor running speed while trimming proceeding |
| P54 | Trimming Time（ms） | 5-990 | 200 | Setting time trimming sequence required |
| P56 | Automatically find UP position | 0-2 | 2 | 0: Never look for needle up position 1: Always look for needle up position 2: If in the needle up position，do not look for the position again |
| P58 | Up Position Adjustment | 0-1799 | 1125 | Needle stops ahead of time with value decrease; Needle delayed stopping with value increase. |
| P59 | Down Position Adjustment | 0-1799 | 225 | Needle stops ahead of time with value decrease; Needle delayed stopping with value increase. |
| P60 | Testing Speed（rpm ） | 100-4000 | 2000 | Testing speed setting. |
| P61 | Testing A | 0-1 | 0 | After setting A, continuous running at the speed of【P60. TV】. |
| P62 | Testing B | 0-1 | 0 | Option of Testing B, after setting press 【p60. TV】to set the speed execute the cycle of Start – Sewing –Stop - Trimming |
| P63 | Testing C | 0-1 | 0 | Option of Testing C, after setting press【060. TV】to set the speed execute the cycle of Start – Sewing –Stop without positioning function |
| P64 | Running Time of Testing B and C | 1-250 | 20 | Setting running time of testing B and C |
| P65 | Stop Time of Testing B and C | 1-250 | 20 | Setting stop time of testing B and C |
| P66 | Machine Protection Switch Testing | 0-2 | 1 | 0：Disable， 1：Testing zero signal， 2：Testing positive signal |
| P72 | UP needle position adjustment |  |  |  |
| P73 | DOWN needle position adjustment |  |  |  |
| P80 | Feeding angle of thread trimming | 0-359 | 18 | Feeding angle of thread trimming setting(Down position as 0°) |
| P81 | Torque angle of thread trimmming | 0-359 | 110 | Torque angle of thread trimming setting(Down position as 0°) |
| P82 | Feeder back angle of thread trimming | 0-359 | 175 | Feeder back angle of thread trimming setting Down position as 0°) |
| P84 | Starting torque angle of Thick material machine | 0-359 | 9 |  |
| P85 | Stopping torque angle of Thick material machine | 0-359 | 57 |  |
| P92 | Encoder start angle |  |  | Press cramping key to read the starting angle encoder. Don't change the factory settings. Freely change will lead to the control box, motor abnormal or damaged |

3  Error Code List

| No, | Problem | status / measurement |
| --- | --- | --- |
| E01 | 1) Power ON, the main voltage detection is too high 2) When the supply voltage is too high | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use. ) If correct, please replace the control box and inform the factory |
| E02 | 1) Power ON, the main voltage detection is too low 2) When the supply voltage is too low | Turn off the system power supply, and detect whether the supply voltage is correct. (Or exceed the rated voltage specified in use. ) If correct, please replace the control box and inform the factory |
| E03 | Operation panel and CPU transmission communication exception | Turn off the system power, check if the operation panel interface is loose If contact is good, please change the operation panel. If it is not operated, the control box is damaged, please change |
| E05 | Abnormal contact of speed controller (pedal) | Off the system power supply, check if the connector is loose or fall off. Make the connector normal situation and restart the system. If it still can not work normally, please replace the speed controller and notify the manufacturer.. |
| E07 | a) Abnormal contact of motor plug b) Lock head or motor belt foreign body involved in c) The material is too thick, the motor torque is not enough. d) Output exception of module driven | Rotating the hand wheel of head motor and check if stuck. If it is stuck, there is no mechanical failure of motor head. If the rotation is normal, check if the motor encoder connector and motor power cable joints are loose or not. If any connector is loose, fix it. If connection is good ,check if the supply voltage of the power is too high. If it is too high, adjust it. If it is normal, please replace the control box and notify the manufacturer. |
| E08 | Continuous manual pour over 15 seconds | Restart the control box. |
| E10 | Electromagnetic overcurrent protection | Power off the system. Check the solenoid (solenoid valve) connection line or if the solenoid (solenoid valve) is damaged. |
| E09 E11 | Abnormal locating signal | Power off the system. Check if the motor encoder interface is loose or fall off. Fix it and restart system. If it still can not work normally, please replace the motor and notify the manufacturer. |
| E14 | Abnormal Encoder signal | Power off the system. Check if the encoder interface is loose or fall off . Fix it and restart system. If it still can not work normally, please replace the motor and notify the manufacturer. |
| E15 | Abnormal overcurrent protection of power module | Restart system. If it still can not work normally, please replace the motor and notify the manufacturer. |
| E17 | Error position of head protection switch | Power off the system. Check if the machine head is opened, and the head protection switch is removed or damaged. |
| E20 | Failure start of power motor | Power off the system. Check if the motor encoder interface and motor power connector is loose or fall off. Fix it and restar the system. If it still can not work normally, replace the control box and notify the manufacturer. |

4：Connector diagram

4.1：Each port illustration