安全指示

- 在安装或使用本产品前，使用者必须详细阅读本操作手册。

- 本产品须由受过正确训练的人员来安装或操作。安装作业时必须关闭所有电源，切记不可带电操作。

3） 所有标有符号的指示，必须特别注意并按照说明书上的执行，以免造成不必要的损害。

4） 为安全起见，禁止以延长线作电源座供应二项以上的电器产品使用。

5） 在连接电源线时，必须确定工作电压低于AC 250V，且符合本产品标识中规定的额定电压值。

※注意  ：电控箱电源规格如为AC220V时，请勿插接至AC380V的电源插座上，否则将出现异常且电机无法动作。此时请立即关闭电源开关，重新检查电源。持续供应380V超过五分钟以上，将可能烧损电控箱内器件，而危及人身安全。

6） 请不要在日光直接照射的场所、室外及室温45℃以上或0℃以下的场所操作。

7） 请不要在暖气（电热器）旁、有露水的场所及在相对湿度10%以下或90%以上的场所操作。

8） 请不要在灰尘多的场所、具有腐蚀性物质的场所及有挥发性气体的场所操作。

9） 请注意所有电源线、信号线、接地线等接线时不要受压或过度扭曲，以确保使用安全。

10）电源线的接地端须以适当大小的导线和接头连接到生产工厂的系统地线，此连接必须被永久固定。

11）所有可转动的部分，必须以所提供的零件加以防范露出。

12）在安装完成第一次开电后，先关闭切线功能以低速操作缝纫机并检查转动方向是否正确、运转是否稳定。

13）在进行以下操作前，请先关闭所有电源：

1.在控制箱与马达上插拔任何连接插头时。

2.穿针线时。

3.翻抬缝纫机机头时。

4.修理或做任何机械上的调整时。

5.机器闲置不用时。

14）修理或高层次的保养工作，仅能由受过训练的机电技师来执行。

所有维修用的零件，须由本公司提供认可，方可使用。

15）使用本产品请远离高频电磁波和电波发射器等，以免所产生的电磁波干扰伺服驱动装置而发生误动作。

16）请不要以不适当物体来敲击或撞击本产品及各装置。

保修期限

本产品保修期限为购买日期起一年内或出厂月份起两年内。

保修内容

本产品在正常情况使用且无人为操作失误的前提下，于保修期间无偿为客户维修使能正常操作。

但以下情况于保修期间将收取维修费用：

1.不当使用包括误接高压电源、将产品移做其它用途、自行拆卸、维修、更改、或不依规格范围使用、进水进油及插入异物于本产品。

2.火灾、地震、闪电、风灾、水灾、盐蚀、潮湿、异常电压及其它天灾或不当场所造成的损害。

3.客户购买后摔落本产品，或客户自行运输（或托付运输公司）造成的损害。

* 本产品在生产及测试上皆尽最大努力和严格控制使其达到高品质及高稳定的标准，但外部的电磁或静电干扰或不稳定的供应电源，仍可能对本产品造成影响或损害，因此操作场所的接地系统一定要确实做好，并建议用户安装故障安全防护装置（如漏电保护器）

- 按键显示及操作说明

| 功能 | 按键 | 车缝动作说明 |
| --- | --- | --- |
| 功能参数编辑键 |  | 进入或退出功能参数的编辑。 |
| 参数查看保存键 |  | 对所选参数号内容进行查看和保存：选择好参数号后按此键可以进行查看和修改操作，修改参数值后按此键则退出并保存参数。 |
| 加键/加速键 |  | 增大参数/提高运行速度 |
| 减键/减速键 |  | 减小参数/降低运行速度 |
| 机头灯亮度键 |  | 点击调节机头灯的亮度。 |
| 慢速起缝键 |  | 设置使用或取消慢速起缝功能。 |
| 自动抬压脚键 |  | 点击切换自动抬压脚关闭、中途停车抬压脚、切线后抬压脚、中途停车与切线后抬压脚。 |
| 吸风键 |  | 设置使用或取消吸风功能。 |
| 停针定位选择键 |  | 点击切换上、下停针位。 |
| 扫线键 |  | 设置使用或取消扫线功能。 |
| 上刀切线键 |  | 设置使用或取消上刀切线功能。 |
| 下刀切线键 |  | 设置使用或取消下刀切线和上刀切线功能。 |

2.参数表

| 参数项 | 中文说明 | 范围 | 初始值 | 内容值名称说明与备注 |
| --- | --- | --- | --- | --- |
| P01 | 最高转速（rpm） | 100-6000 | 4500 | 车缝时的最高转速设置 |
| P02 | 加速曲线调整（%） | 10-100 | 80 | 控速器爬升斜率设置 斜率值愈大，速度愈陡；斜率值愈小，速度愈慢 |
| P03 | 针停定位选择 | UP/DN | DN | UP：上停针；DN：下停针 |
| P07 | 慢速起缝速度（rpm） | 200-1500 | 1000 |  |
| P08 | 慢速起缝针数（针） | 1-99 | 2 |  |
| P14 | 慢速启动 | ON/OFF | OFF |  |
| P15 | 机头灯亮度 | 0-4 | 3 |  |
| P21 | 踏板前踩位置 | 30-1000 | 500 |  |
| P22 | 踏板回中位置 | 30-1000 | 420 |  |
| P23 | 踏板抬压脚位置 | 30-1000 | 280 |  |
| P24 | 踏板切线位置 | 30-1000 | 130 |  |
| P29 | 切线停车力度 | 1-45 | 20 |  |
| P36 | 扫线开关 | ON/OFF | OFF |  |
| P37 | 上刀切线开关 | ON/OFF | ON |  |
| P38 | 下刀切线开关 | ON/OFF | ON |  |
| P39 | 中途停车自动抬压脚 | UP/DN | DN | UP：开启；DN：关闭 |
| P40 | 切线后自动抬压脚 | UP/DN | DN | UP：开启；DN：关闭 |
| P41 | 计数器显示 | 0-9999 |  | 车缝完成件数显示；长按减号键可计数清零； |
| P42 | 信息显示 |  |  | N01 电控版本号 N02 面板版本号 N03 车缝转速 N04 脚踏板AD N05 上定位角度 N06 下定位角度 N07 母线电压AD |
| P43 | 马达转动方向设置 | CCW/CW | CCW | CW：顺时针方向；CCW： 逆时针方向 |
| P44 | 正常停车力度 | 1-45 | 25 |  |
| P46 | 切线后提针开关 | ON/OFF | OFF |  |
| P47 | 切线后提针角度 | 50-200 | 160 |  |
| P48 | 低速（定位速度）（rpm） | 100-500 | 200 | 最低速度限制调整 |
| P50 | 抬压脚全额出力的工作时间（ms） | 10-990 | 150 |  |
| P51 | 压脚出力的周期信号（%） | 1-50 | 30 | 压脚动作时，以周期性省电输出，避免电磁铁发烫 |
| P52 | 延迟马达启动，保护压脚下放时间（ms） | 10-990 | 150 | 踩下时延迟启动时间，以配合自动抬压脚放下的确认 |
| P53 | 抬压脚功能选择 | 0-2 | 1 | 0：关闭； 1：反踏和半反踏都有抬压脚； 2：半反踏无抬压脚，反踏有抬压脚； |
| P54 | 下切线动作时间（ms） | 10-990 | 100 | 下切线时序所需的动作时间 |
| P55 | 上切线动作时间（ms） | 10-990 | 45 | 上切线时序所需的动作时间 |
| P56 | 开机自动找定位 | 0-2 | 0 | 0：始终不找上定位 1：始终找上定位 2：若电机已经处于上定位时不再找上定位 |
| P57 | 抬压脚保护时间（s） | 1-120 | 10 | 抬压脚保持时间后强制关闭 |
| P58 | 上定位调整 | 0-1439 | 40 | 上定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P59 | 下定位调整 | 0-1439 | 750 | 下定位调整，数值减少时会提前停针，数值增加时会延迟停针 |
| P60 | 测试速度（rpm） | 100-6000 | 4000 | 设置测试速度 |
| P61 | A项测试 | ON/OFF | OFF | 持续运行测试模式 |
| P62 | B项测试 | ON/OFF | OFF | 全功能启停测试模式 |
| P63 | C项测试 | ON/OFF | OFF | 无定位、无功能启停测试模式 |
| P64 | 测试运行时间 | 1-250 | 50 |  |
| P65 | 测试停止时间 | 1-250 | 50 |  |
| P66 | 机头保护开关 | 0-2 | 0 | 0：不检测；1：检测零信号；2：检测正信号 |
| P67 | 切线保护开关 | ON/OFF | ON |  |
| P70 | 机型选择 |  |  |  |
| P71 | 压脚释放缓冲占空比(%) | 0-50 | 0 | 压脚释放时的力度调整 |
| P72 | 上定位快捷调整 | 0-1439 |  | 调整上停针位，显示的数值会随手轮位置变化而变化，按“S”键可保存当前位置（数值）为上停针位 |
| P73 | 下定位快捷调整 | 0-1439 |  | 调整下停针位，显示的数值会随手轮位置变化而变化，按“S”键可保存当前位置（数值）为下停针位 |
| P87 | 上切线回程延时（ms） | 10-990 | 50 |  |
| P89 | 交流过压值设置 | 500-1023 | 880 |  |
| P92 | 校正电机角度 |  |  | 按停针定位选择键读取电机角度，出厂已设置，请勿随意更改（随意更改会损坏电机） |
| P93 | 半反踏延迟时间（ms） | 10-900 | 150 |  |
| P105 | 电动/气动选择 | 0-1 | 0 | 0：电动；1：气动 |
| P108 | 吸气选择 | ON/OFF | ON |  |
| P109 | 上切线前延时（ms） | 5-990 | 55 | 找到上定位后进入上切线动作前的间隔时间 |
| P110 | 下切线回程时间（ms） | 60-990 | 60 |  |
| P112 | 下切线出力的动作时间（ms） | 10-990 | 100 |  |
| P113 | 下切线出力的周期信号（%） | 0-100 | 100 | 下切线出力时的力度调整 |
| P117 | 压脚释放缓冲延时（ms） | 0-990 | 20 |  |
| P119 | 电磁铁过流保护 | 0-1 | 1 | 0：停用；1：启用 |

注：参数初始值仅供参考，实际参数值以实物为准。

3.错误代码表

| 错误码 | 问题描述 | 解决措施 |
| --- | --- | --- |
| E01 | 超电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否超过使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E02 | 低电压 | 关闭系统电源，检测供应电源电压是否正确。（或是否低于使用规定的额定电压）。 若正确，请更换控制箱并通知售后服务。 |
| E03 | CPU通信异常 | 关闭系统电源，检查控制面板的连线是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E05 | 控速器接触异常 | 关闭系统电源，检查控速器接头是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控速器并通知售后服务。 |
| E07 | 电机堵转 | 转动机头手轮观察是否卡住。如卡住则先排除机械故障。 如转动正常，检查电机编码器接头和电机电源线接头是否松动。如有松动请修正。 如接触良好，检查供应电源电压是否异常或转速设置过高。如有请调整。 若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E10 | 电磁铁过流保护 | 关闭系统电源，检查电磁铁（电磁阀）是否损坏或短路。 |
| E09 E11 | 定位信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E14 | 编码器信号异常 | 关闭系统电源，检查电机编码器接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换电机并通知售后服务。 |
| E15 | 电力模块不正常过流保护 | 关闭系统电源，再重新开启。若仍不能正常工作，请更换控制箱并通知售后服务。 |
| E16 | 切线开关没到正确位置 | 关闭系统电源，检查切线机构是否回到正确位置。 |
| E17 | 机头保护开关没到正确位置 | 关闭系统电源，检查机头是否掀开，控制箱内滚珠开关是否移位或损坏。 |
| E20 | 电机启动失败（电角度错误） | 关闭系统电源，检查电机编码器接口和电机电源接口是否松动或脱落，将其恢复正常后重启系统。若仍不能正常工作，请更换控制箱并通知售后服务。 |

4.端口示意图

电机电源接口

2P衣车灯接口

6P功能接口

10P功能接口

电机编码器接口

4.1功能端口说明

  - Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Function Parameter Edit |  | Press this key to enter or exit function parameter edit mode. |
| Setting Parameter Check And Save |  | For the content of the selected parameters check and save: after select parameter press this key to check and modify operation, after modified parameter value press this key to exit and save the parameter. |
| Parameter Increase/ Speed Increase |  | Increase the parameter/ Increase the sewing speed |
| Parameter Decrease/ Speed Decrease |  | Decrease the parameter/ Decrease the sewing speed |
| LED brightness adjustment |  | Press this key to adjust LED brightness. |
| Slow Launch Setting |  | Set used or cancelled slow launch function. |
| Automatic Presser Foot Lifting Setting |  | Click to shift auto presser foot lift OFF, intermediate stop presser foot lift, presser foot lift after trimming, presser foot lift after intermediate stop and trimming |
| Auto-suction |  | Set used or cancelled auto-suction function. |
| Needle Stop Position Selection |  | Set needle stop position. |
| Wiper Key |  | Set used or cancelled wiper function. |
| Upper knife trimming |  | Set used or cancelled upper knife trimming function. |
| Lower knife trimming |  | Set used or cancelled lower knife trimming function. |

2 User Parameter

| No. | Items | Range | Default | Description |  |
| --- | --- | --- | --- | --- | --- |
| P01 | Maximum Sewing Speed（rpm） | 100-6000 | 4500 | Maximum speed of machine sewing |  |
| P02 | Set accelerated curve（%） | 10-100 | 80 | Set the acceleration slope |  |
| P03 | Needle UP/ DOWN | UP/DN | UP | UP: Needle Stops at Up Position DN: Needle Stops at Down Position |  |
| P07 | Soft Start Speed（rpm ） | 200-1500 | 1000 |  |  |
| P08 | Stitch Numbers for Soft Start | 1-99 | 2 |  |  |
| P14 | Slow start function selection | ON/OFF | OFF |  |  |
| P15 | Mending stitch mode | 0-4 | 3 |  |  |
| P21 | The position of the pedal for running | 30-1000 | 500 |  |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |  |
| P23 | The position of the pedal for lifting presser foot | 30-1000 | 280 |  |  |
| P24 | The position of the pedal for thread trimming | 30-500 | 130 |  |  |
| P29 | The rate of thread trimming stop | 1-45 | 20 |  |  |
| P36 | Thread wiping switch selection | ON/OFF | OFF |  |  |
| P37 | Upper knife trimming output function setting | ON/OFF | ON |  |  |
| P38 | Lower knife trimming output function setting | ON/OFF | ON |  |  |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN | UP：ON DN：OFF |  |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN | UP：ON DN：OFF |  |
| P41 | Counter display | 0-9999 |  | Display the quantity of finished sewing piece |  |
| P42 | Information Display |  |  | N01 The control system version number N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(up position) N06 The mechanical angle(down position) N07 Busbar voltage AD |  |
| P43 | Direction of Motor Rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |  |
| P44 | The rate of normal stop | 1-45 | 25 |  |  |
| P46 | Motor stop with a reverse angle after trimming function | ON/OFF | OFF |  |  |
| P47 | Adjust the reverse angle when motor stop after trimming | 50-200 | 160 |  |  |
| P48 | The minimum speed Limit（rpm ） | 100-500 | 200 | Adjust the minimum speed. |  |
| P50 | Toot lifter operation time to full output（ms） | 10-990 | 150 |  |  |
| P51 | Presser foot lifter operation duty cycle (%) | 1-50 | 30 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |  |
| P52 | Motor starting delay time（ms） | 10-990 | 150 | Delay the start time, with automatic presser foot down. |  |
| P53 | Lifting presser foot function selection | 0-2 | 1 |  | 0: OFF 1: ON 2: Half back pedaling without function, back pedaling with function |
| P54 | Lower knife trimming action time（ms） | 10-990 | 100 |  | Lower knife trimming sequence time |
| P55 | Upper knife trimming action time（ms） | 10-990 | 45 |  | Upper knife trimming sequence time |
| P56 | Power on and positioning | 0-2 | 0 |  | 0: always not to find the up needle position 1: always to find the up needle position 2:If the motor is not into the up needle position, it will find the up needle position |
| P57 | Hold time of presser foot lifter（s） | 1-120 | 10 |  | Forced shut-down after hold time. |
| P58 | Up needle position adjustment | 0-1439 | 40 |  | Up Position Adjustment The needle will advance stop when the value decreased. The needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-1439 | 750 |  | Down Position Adjustment The needle will advance stop when the value decreased. The needle will delay stop when the value increased. |
| P60 | Testing Speed（rpm ） | 100-6000 | 4000 |  | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF |  | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF |  | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF |  | Start and stop testing without all function. |
| P64 | Running Time of Testing | 1-250 | 50 |  |  |
| P65 | Stop Time of Testing | 1-250 | 50 |  |  |
| P66 | Machine protection switch selection | 0-2 | 0 |  | 0：Disable 1：Testing zero signal 2：Testing positive signal |
| P67 | Thread trimming protection switch selection | ON/OFF | ON |  |  |
| P70 | The factory type selection |  |  |  |  |
| P71 | Presser foot release operation duty cycle (%) | 0-50 | 0 |  | ### The buffer force of presser foot release operation. |
| P72 | Up needle position adjustment | 0-1439 |  |  | Hand rotation manual to the appropriate position, press S key to save |
| P73 | Down needle position adjustment | 0-1439 |  |  | Hand rotation manual to the appropriate position, press S key to save |
| P87 | Upper knife trimming back delay times( ms ) | 10-990 | 50 |  |  |
| P89 | Exchange overvoltage settings | 500-1023 | 880 |  |  |
| P92 | Correct the electrical angle of motor |  |  |  | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delay time of the half back pedaling function( ms ) | 10-900 | 150 |  |  |
| P105 | Electric/ pneumatic selection | 0-1 | 0 |  | 0: Electric 1: Pneumatic |
| P108 | Interlock suction function selection | ON/OFF | ON |  |  |
| P109 | The delay time before upper knife trimming( ms ) | 5-990 | 55 |  | The interval time of upper knife trimming action after finding up needle stop position |
| P110 | Lower knife trimming back delay times( ms ) | 60-990 | 60 |  |  |
| P112 | Lower knife trimming output action time( ms ) | 10-990 | 100 |  |  |
| P113 | Periodic signal of lower knife trimming output( % ) | 0-100 | 100 |  | Force adjustment of lower knife trimming output |
| P117 | The delay time of the presser foot release damping force (ms) | 0-990 | 20 |  |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 1 |  | 0: OFF 1: ON |

3 Error Code List

| Error Code | Problem description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage (Or whether exceed the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage (Or whether under the rated voltage of use). If there are still errors, please replace the control box and call for customer service. |
| E03 | CPU communication error | Turn off the machine, please check if the operation panel in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E05 | Pedal connection error | Turn off the machine, please check if the pedal in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E07 | Motor locked-rotor error | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. If there are still errors, please replace the control box and call for customer service. |
| E10 | Solenoid over-current | Turn off the machine, please check the related circuit is shorted or damaged. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E15 | Power supply module abnormal over-current | Turn off the machine, then turn on again. If there are still errors, please replace the control box and call for customer service. |
| E16 | Thread trimming switch is not in the correct position | Turn off the machine, check if the thread trimming switch back to the correct position. |
| E17 | Machine protection switch is not in the correct position | Turn off the machine, check if the machine is turn over, or the mercury switch is shifted or damaged. |
| E20 | Motor failed to start(Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |

4 Port Diagram

Power connector socket

10P function port table illustration

2P LED

Motor encoder connector socket

6P function port table illustration

4.1