Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, you must make sure that the operating voltage is lower than AC250V and that it meets the rated voltage value specified in the product logo.

※ Note: If the power specification of the electric control box is AC220V, please do not plug it into the AC380V power socket, otherwise abnormality will occur and the motor will not operate. At this time, immediately turn off the power switch and recheck the power supply. Continuous supply of 380V for more than five minutes may burn the components in the electric control box and endanger personal safety.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Key Description

| Name | Key | Indicate |
| --- | --- | --- |
| Stitch length setting |  | If click ➕，Stitch length increased. If click ➖，Stitch length decreased. |
| Start back-tacking sewing |  | If click, switch starting back seam (A, B segment) 1 time, starting back seam (A, B segment) 2 times or turn off. |
| End back-tacking sewing |  | If click, switch ending back seam (C, D segment) 1 time, ending back seam (C, D segment) 2 times or turn off. |
| Clamping function |  | If click, set used or cancelled clamping function. |
| Needle stop position selection |  | If click, shift to the needle stop position (up position / down position). |
| Press foot lifting after trimming / Press foot lifting after pause |  | If click, set used or cancelled press foot lifting after trimming function. If long-press, set used or cancelled press foot lifting after pause function. |
| Free sewing / Thread trimming |  | If click, shift to free sewing If long-press, set used or cancelled thread trimming function. |
| Press foot lifting / Auto function |  | If click, set used or cancelled press foot lifting function. If long-press, set used or cancelled auto function (Effective in multi-segment mode). |
| W sewing / Multi-segment sewing |  | If click, shift to W sewing. If long-press, shift to Multi-segment sewing. |
| Lock needle key |  | If click, set to use front lock needle, back lock needle, front and back lock needle or cancel front and back lock needle function. (The needle lock function is tight seam) |
| Pattern sewing mode |  | If click, switch the pattern sewing mode and pattern sewing editing. |
| Function parameter edit |  | Press this key can enter or exit the parameter setting interface. Save and exit the selected parameter: Press this key after modifying the parameter value to exit and save the parameter. |
| Parameter increase |  | The main interface, if clicked, increase the speed of 50 rpm each time. Parameter interface, if click, increase the parameter. |
| Parameter decrease |  | The main interface, if clicked, reduces the speed of 50 rpm each time. Parameter interface, if click, decrease the parameter. |
| Left Selection Parameter Content |  | Left selection parameter content |
| Right Selection Parameter Content |  | Right selection parameter content |
| Restore Factory |  | Long-press can restore factory setting. |

2 Auxiliary Function

2.1 Parameter adjustment

2.1.1 User parameter adjustment

Boot, long press  key enter to user parameter mode. Press  or  to change parameter item, remove cursor, the corresponding digit value is flashing, then press or  to modify this digit value, when the cursor is blinking at a numerical position, press  key, the whole parameter value is flashing, save the parameter. Press  key again to exit.

2.1.2 Technician parameter adjustment

In the off state, hold on  key to turn on the machine (show P92) enter to technician parameter mode. Press  or  change parameter item, remove cursor, the corresponding digit value is flashing, then press  or  to modify this digit value, when the cursor is blinking at a numerical position, press  key, the whole parameter value is flashing, save parameter. Press  key again to exit.

2.2 Soft start

P14 adjustment in user parameters, the parameter is ON means open, the parameter is OFF means close (The default value).

2.3 Needle stop position function setting

In the off state, press and hold  key to enter the technician parameter mode (the P92 parameter interface is displayed). Adjust the parameter to P72, manually rotate the handwheel to up position, the value displayed will change with the change of the wheel position. Press  key to save the current position (value) as the up needle stop position and calculate down needle stop position automatically.

2.4 Stepper motor zero correction setting

In the shutdown state, press and hold  key to boot into the technician parameter mode (display the P92 parameter interface). Adjust the parameter to item P129, use ordinary A4 paper to measure, the speed is 4000, adjust the parameter to make the needle stick more than 10 stitches in place, and press the  key to save; (if the stitch length is along the seam, the number increases upwards, if the stitch length is reverse stitching, the number decreases downward, the value is between -50 and -80.)

2.5 Stitch length setting

In the shutdown state, press and hold  key to boot into the technician parameter mode (display the P92 parameter interface). Adjust the parameter to P197 (tacking stitch length compensation), adjust the stitch length to 5, use ordinary A4 paper, speed 200 to measure, adjust the parameter so that the stitch length is 4.9-5.1mm, press  to save, (The larger the stitch pitch, the greater the stitch length, The stitch length of 1 value changed to about 0.05), then enter the P198 (back-tacking stitch length compensation) parameter, use ordinary A4 paper, speed 200 to measure, press and hold the back-tacking switch, speed 200, adjust parameters to make the sewing overlap more than 11 stitches, press  key to save; (the greater the value, the greater the back-tacking stitch length)

3 Parameter Definition

3.1 User Parameter (Turn on the machine and long-press P key to enter)

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed（rpm） | 100-5000 | 4000 |  |
| P02 | Set accelerated curve（%） | 10-100 | 80 | The speed controller climbing slope setting. The large of slope value = the fast of the speed. The small of slope value = the slow of the speed. |
| P03 | Needle up/ down | DN/UP | DN | UP: Needle stops at up position DN: Needle stops at down position |
| P04 | Start back-tacking speed（rpm） | 200-3200 | 1800 |  |
| P05 | End back-tacking speed（rpm ） | 200-3200 | 1800 |  |
| P06 | W sewing speed（rpm） | 200-3200 | 1800 |  |
| P07 | Soft start speed（rpm ） | 200-1500 | 400 |  |
| P08 | Stitch numbers for soft start (needle) | 1-15 | 2 |  |
| P09 | Automatic multi-segment sewing speed（rpm） | 200-4000 | 3700 | Speed adjustment for automatic constant-stitch sewing |
| P10 | Automatic end back-tacking sewing after multi-segment sewing (without mending stitch function) | ON/OFF | ON | ON: After executing the last multi-segment sewing, the back-tacking sewing will be executed automatically. Note: In other words, in any sewing mode, mending stitch function cannot be used. OFF: After executing the last multi-segment sewing, the back-tacking sewing function will not be automatically executed, and the front step or full back step must be performed again. |
| P12 | Start back-tacking running mode Selection | 0-1 | 1 | 0: Could be arbitrarily stop and start 1: Automatically perform actions |
| P13 | Operation mode selection of the end point of start back-tacking sewing | CON/STP | CON | CON: Start Back-Tacking is completed automatically continued for next action. STP: After the number of stitches is completed, stop automatically. |
| P14 | Slow start function selection | ON/OFF | OFF |  |
| P16 | Speed limit of manual back-tacking | 0-3200 | 3200 | When the parameter is 0, the speed limit function is OFF. |
| P17 | Automatic count mode selection | 0-9 | 1 | 0: P41 parameter disable auto-count. 1-9: P41 item counter automatically counts one piece every 1-9 times of tangent. |
| P18 | Start back-tacking compensation 1 | 0-200 | 162 | 0~200 action gradually delay. |
| P19 | Start back-tacking compensation 2 | 0-200 | 162 | 0~200 action gradually delay. |
| P20 | End back-tacking running mode selection | 0-1 | 1 | 0: Could be arbitrarily stop and start 1: Automatically perform actions |
| P21 | The position of the pedal for running | 30-1000 | 520 |  |
| P22 | The position of the pedal for stopping | 30-1000 | 420 |  |
| P23 | The position of the pedal for presser foot lifting | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming 1 | 30-500 | 130 | The position of the pedal for thread trimming when presser foot function on |
| P25 | End back-tacking compensation 3 | 0-200 | 162 | 0~200 action gradually delay. |
| P26 | End back-tacking compensation 4 | 0-200 | 162 | 0~200 action gradually delay. |
| P27 | The position of the pedal for thread trimming 2 | 30-500 | 220 | The position of the pedal for thread trimming when presser foot function off |
| P28 | W sewing running mode selection | 0-1 | 1 | 0: Could be arbitrarily stop and start 1: Automatically perform actions |
| P29 | The strength of thread trimming stop | 1-45 | 20 |  |
| P32 | W sewing compensation 5 | 0-200 | 162 |  |
| P33 | W sewing compensation 6 | 0-200 | 162 |  |
| P34 | Multi-segment sewing running mode selection | A/M | A | A: Automatically perform actions M: Could be arbitrarily stop and start |
| P37 | Thread wiping function / Thread clamping function selection | 0-11 | 5 | 0: OFF 1: Thread wiping function 2-11: Thread clamping function, the higher the value, the stronger the action. |
| P38 | Automatic thread trimming function selection | ON/OFF | ON | ON: With thread trimming OFF: Without thread trimming |
| P39 | Automatic presser foot lifting when pause function selection | DN/UP | DN | UP: When the sewing stops, the presser foot will lift automatically DN: When the sewing stops, the presser foot will not lift automatically (Controlled by heeling pedal) |
| P40 | Automatic presser foot lifting after trimming function selection | DN/UP | DN | UP: When the sewing stops, the presser foot will lift automatically DN: When the sewing stops, the presser foot will not lift automatically (Controlled by heeling pedal) |
| P41 | Counter display | 0-9999 | 0 | The number of sewing completed pieces is displayed; long press parameter decrease key to clear the count; |
| P42 | Information display |  | N01 | N01 The control system version number I N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(up position) N06 The mechanical angle(down position) N07 Busbar voltage AD N12 Kneeling device position sensor AD N13 Presser foot height sensing AD N14 The control system version number II N15 Stepping version number |
| P43 | Direction of motor rotation | CCW/CW | CCW | CW: Clockwise CCW: Counter Clockwise |

3.2 Technician Parameter (Hold P key to boot)

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P44 | Brake strength during stopover | 1-45 | 16 | Adjustment of efforts while machine stop. |
| P46 | Motor stop with a reverse angle after trimming function | ON/OFF | OFF | ON: Function on OFF: Function off |
| P47 | Adjust the reverse angle when motor stop after trimming | 40-200 | 160 | After trimming, count from the up needle position and adjust the angle of the needle lift by reverse rotation. |
| P48 | Minimum speed (Positioning speed) (rpm) | 100-500 | 200 | Minimum speed limit adjustment |
| P49 | Thread trimming speed（rpm） | 100-500 | 250 | Adjust the motor speed during the trimming cycle |
| P50 | Foot lifter operation time to full output（ms） | 10-990 | 200 |  |
| P51 | Presser foot lifter operation duty cycle (%) | 1-50 | 37 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Delay the start of the motor to protect the lowering time of presser foot (ms) | 10-990 | 120 | Delay the start time, with automatic presser foot down. |
| P53 | Half back pedaling lifting presser foot function to cancel | 0-2 | 1 | 0: OFF 1: Back pedaling and half back pedaling with lifting presser foot. 2: half back pedaling without lifting presser foot, back pedaling with lifting presser foot. |
| P54 | Thread trimming action time（ms） | 10-990 | 200 | Completion of the thread trimming action required time. |
| P55 | Thread wiping action time（ms） | 10-990 | 10 | Completion of the thread wiping action required time. |
| P56 | Power on and positioning | 0-1 | 1 | 0: always not to find the up needle position 1: always to find the up needle position |
| P57 | Presser foot solenoid protection time (s) | 1-60 | 10 | Forced shutdown after hold time to prevent the electromagnet from being hot for a long time |
| P58 | Up needle position adjustment | 0-359 | 240 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-359 | 050 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed（rpm ） | 100-4000 | 3500 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | After setting A, continuous running at the speed of 【P60.TV】 |
| P62 | Testing B | ON/OFF | OFF | Option of Testing B, after setting press 【P60. TV】to set the speed execute the cycle of Start – Sewing –Stop - Trimming |
| P63 | Testing C | ON/OFF | OFF | Option of Testing C, after setting press【060.TV】to set the speed execute the cycle of Start – Sewing –Stop without positioning function |
| P64 | Running time of testing B and C | 1-250 | 30 | Setting running time of testing B and C |
| P65 | Stop time of testing B and C | 1-250 | 10 | Setting stop time of testing B and C |
| P66 | Machine protection switch selection | 0-1 | 1 | 0: OFF 1: ON |
| P68 | Maximum speed limit | 100-5000 | 4000 |  |
| P69 | The speed of pattern freedom sewing | 100-3000 | 2000 |  |
| P72 | Up needle position adjustment | 0-359 | 240 |  |
| P73 | Down needle position adjustment | 0-359 | 240 |  |
| P76 | Thread clamping full output time | 0-990 | 4 |  |
| P78 | The start angle of thread clamping | 1-359 | 130 | Positioning angle when start clipping. |
| P79 | The stop angle of thread clamping | 1-359 | 320 | Positioning angle when finish clipping. |
| P80 | The start angle of thread trimming | 0-359 | 5 | Shear line infeed id angle setting(under defined as 0°) |
| P81 | Presser foot release buffer time | 1-800 | 200 |  |
| P82 | The stop angle of thread trimming | 0-359 | 175 | Feeder back angle of thread trimming setting Down position as 0°. |
| P83 | Stopping strength after trimming | 10-100 | 20 |  |
| P86 | The distance of up and down position | 15-345 | 170 |  |
| P87 | Wiping thread return delay time | 10-990 | 50 | Make sure the wiper return to its original position |
| P88 | Stopover distance | 10-100 | 30 |  |
| P92 | Correct the electrical angle of motor | 0-1200 | 130 | Press the needle position key to read the initial angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delay time of the half back pedaling function (ms) | 10-900 | 100 |  |
| P99 | Starting tight seam stitch length | 0-50 | 8 | If the value is 10, the stitch length is 1.0 |
| P100 | Direction of starting tight seam | 0-1 | 0 |  |
| P105 | Pattern freedom sewing mode selection | N0-N9 | N0 | N0: OFF N1-N9: Pattern freedom sewing mode |
| P107 | Starting tight seam speed | 100-2500 | 1000 |  |
| P108 | Starting tight seam stitch number | 0-12 | 2 |  |
| P109 | The delay time before thread wiping | 5-990 | 5 | Interval time before entering thread wiping action after finding the upper positioning |
| P110 | Trimming back time (ms) | 60-990 | 65 | Make sure the thread trimming device returns to its original position |
| P118 | Stitch length display mode | 0-1 | 0 | 0: Digital stitch length mode 1: 1 inch stitch number mode |
| P123 | Maximum stitch length limit | 0-70 | 50 |  |
| P129 | Back-tacking step motor zero-point correction | -500～500 | -130 |  |
| P131 | Normal stitch length | 0-50 | 35 | Stitch length in normal sewing mode |
| P132 | Manual close stitches distance | 0-50 | 20 |  |
| P138 | Foot lifting release buffer duty cycle (%) | 0-100 | 10 |  |
| P139 | Foot lifting release buffer delay time (ms) | 0-200 | 5 |  |
| P143 | Tight seam mode selection | 0-3 | 2 | Tight seam mode selection: 0: OFF 1: Starting tight seam 2: Ending tight seam 3: Full function |
| P153 | Ending tight seam stitch length | 0-50 | 5 | If the value is 10, the stitch length is 1.0 |
| P154 | Ending tight seam speed | 100-2500 | 1800 |  |
| P159 | Direction of ending tight seam | 0-1 | 0 | 0: Forward 1: Backward |
| P160 | Ending tight seam stitches | 0-12 | 2 | When the parameter value is 0, the function off. |
| P175 | 1/2 key method | 7-12 | 7 | 7: 1/2 stitch length 8: 1/4 stitch length 9: 1/8 stitch length 10: Reverse 1/2 stitch length 11: Reverse 1/4 stitch length 12: Reverse 1/8 stitch length |
| P197 | Tacking stitch length compensation | -50～50 | 0 | It is adjusted when main shaft at low speed, the larger the value is, the larger the stitch length is, it is used when the stitch length is inconsistent due to mechanical error. |
| P198 | Back-tacking stitch length compensation | -50～50 | 0 | It is adjusted when main shaft at low speed, the larger the value is, the larger the stitch length is, it is used when the stitch length is inconsistent due to mechanical error. |
| P201 | Start sewing presser foot lifting switch | 0-1 | 0 | 0: OFF 1: ON |
| P202 | The start angle of start sewing presser foot lifting | 0-359 | 1 |  |
| P203 | The stop angle of start sewing presser foot lifting | 0-359 | 200 |  |
| P204 | Start sewing presser foot lifting strength | 0-100 | 60 |  |
| P205 | Start sewing first stitch speed limit | 0-1500 | 600 | The default 0 is off |
| P206 | Start sewing second stitch speed limit | 0-2000 | 1200 | The default 0 is off |
| P207 | Start sewing third stitch speed limit | 0-2500 | 0 | The default 0 is off |
| P208 | Panel stitch length lock function | 0-1 | 0 | 0: Off 1: ON(+ key and - key on the main interface are invalid) |
| P216 | The start angle of clamping when start back-tacking sewing and W sewing function on | 1-359 | 100 |  |
| P217 | The stop angle of clamping when start back-tacking sewing and W sewing function on | 1-359 | 270 |  |
| P218 | The start angle of clamping when starting tight seam function on | 1-359 | 130 |  |
| P219 | The stop angle of clamping when starting tight seam function on | 1-359 | 320 |  |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem description | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the machine, check the power supply voltage, or whether it exceeds the rated voltage used. If there are still errors, please replace the control box and call for customer service. |
| E02 | Low-voltage | Turn off the machine, check the power supply voltage, or whether it is lower than the rated voltage used. If there are still errors, please replace the control box and call for customer service. |
| E03 | CPU communication error | Turn off the power and check whether the connector is connected to the operation panel and the cable condition, restart the system after returning it to normal. If there are still errors, please replace the control box and call for customer service. |
| E05 | Speed control unit connection is abnormal | Please turn off the power and check whether the connector is connected to the speed controller and the cable condition, restart the system after returning it to normal. If there are still errors, please replace the control box and call for customer service. |
| E07 | Main shaft motor rotating abnormally | Please rotate handwheel to check if the spindle motor is locked. Check if the encoder cable of the motor and the power cable of the motor are connected to the connector. Check if the power supply voltage is normal and whether the sewing speed is too high. If it still does not work normally after the removal, please replace the control box and call for customer service. |
| E10 | Solenoid over-current protection | Turn off the system power supply, please check the solenoid is broken or not. |
| E09 | Positioning signal error | Turn off the system power supply, please check motor encoder connector if loose or fall off, restore it to normal then restart system. If there are still errors, please replace the motor and call for customer service. |
| E11 |  |  |
| E14 | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, restore it to normal then restart system. If there are still errors, please replace the motor and call for customer service. |
| E15 | Power module abnormal over-current protection | Turn off the power, and then turn it on again. If there are still errors, please replace the control box and call for customer service. |
| E17 | Safety switch alarm | Turn off the system power, check whether the machine head is opened, and whether the ball switch in the control box is displaced or damaged. |
| E20 | Motor start failed | Turn off the system power supply, please check motor encoder connector and motor power socket if loose or fall off, restore it to normal then restart system. If there are still errors, please replace the motor and call for customer service. |
| E30 | Shutdown failure when password is locked | The machine has reached the set time. If you need to continue using it, please contact your local service provider. |
| E40 | Crystal oscillator of panel abnormal | Turn off the power and replace the panel PCB. If there are still errors, please call for customer service. |
| E80 | Abnormal communication between the main CPU and the step driver CPU | Turn off the system power and restart. If you still get an error, please call for customer service. |
| E82 | Back-tacking step motor overcurrent | Turn off the power and check the step motor connector, then turn it on again. If you still get an error, please call for customer service. |
| E84 | Back-tacking step motor encoder Z signal abnormal | Turn off the power and check the step motor connector, then turn it on again. If you still get an error, please call for customer service. |
| E85 | Back-tacking step motor encoder AB signal abnormal | Turn off the power and check the step motor connector, then turn it on again. If you still get an error, please call for customer service. |
| E86 | Back-tacking step motor failed to start | Turn off the power and check whether the connector connecting the step motor coding cable and the power cable is loose or fall off, restore it to normal and restart the system. If it still does not work properly, please replace back-tacking step motor and call for customer service. |
| E87 | Back-tacking step motor locked-rotor | Turn off the power and check whether back-tacking step motor is locked, restore it to normal and restart the system. If it still does not work properly, please replace back-tacking step motor and call for customer service. |
| Oil | Low oil warning | Turn off the power, check whether the oil amount of the sewing machine is too low, and restart the system after filling the oil amount. |

Note: 1. Stepping motor encoder wire alarm troubleshooting: when alarm E84 please kindly check whether the encoder wire black (ground wire), blue (zero position signal) and pink (power wire) fall off or not; when alarm E85 please kindly check whether the encoder connector falls off or not, if not please kindly check whether the encoder wire black (ground wire), brown (a signal) and orange (B signal) fall off or not; when the machine after start or normal operation alarm E87 please kindly check the encoder connector falls off or not.

2. Stepping motor power wire alarm troubleshooting: when alarm E82 please kindly check whether the power wire red (A power wire), blue (B power wire), green (C power wire) falls off or not; when starting up then alarm E82 please kindly check whether the connector of power wire falls off or not; if not, please kindly check whether the blue (B power wire), green (C power wire) falls off or not; when the machine after start or normal operation alarm E87 please kindly check the power wire connector falls off or not.

If the above problem is that the wire or connector falls off, please disconnect the power supply and reconnect again. If not, please refer above to solve.

3. The motor disc problem caused alarm troubleshooting: when alarm E85 and the above wire and connectors all not falls off, then please kindly check whether the outer ring motor disc is covered by oil or not. If the outer ring motor disc is covered by oil, please contact after-sales service to replace the motor disc.

5 Port Diagram

5.1 Port Description

14P function port

Suction

口

18P function port

Pedal

口

5.2 18P function port description

1. Half-needle button: 4、14(GND)

2. Back-tacking key switch: 5、14(GND)

3. Fill needle key switch: 6、14(GND)

5.3 14P function port description

1. Trimming electromagnet: 2、9

2. Clamping electromagnet: 5、12

3. Presser foot electromagnet: 7、14