Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, it must be determined that the operating voltage conforms to the rated voltage value specified in the product identification.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Button Displays and Operating Instructions

| Name | key | Indicate |
| --- | --- | --- |
| Start back-tacking |  | If click, switch to execute start back-tacking B segment, execute start back-tacking (A, B segment) once or execute start back-tacking (A, B segment) 2 times. |
| End back-tacking |  | If click, switch to execute end back-tacking C segment, execute end back-tacking (C, D segment) once or execute end back-tacking (C, D segment) 2 times. |
| Consecutive reverse sewing |  | If click, set to consecutive reverse sewing mode. |
| Free sewing |  | If click, set to free sewing mode. |
| Constant stitch sewing |  | If click, set to section E constant stitch sewing mode. |
| Multi-segment sewing |  | If click, switch to four-segment sewing, seven-segment sewing, eight-segment sewing, and fifteen-segment sewing in sequence. |
| Auto function |  | (Valid only the constant stitch sewing) when the function on. One-shot pedal, it can automatically complete the set sewing process. |
| Trimming setting |  | If click, set used or cancelled trimming function. |
| Clamp function |  | If click, set used or cancelled clamp function. |
| Presser foot lifting |  | 1. Automatic presser foot lifting after trimming function when icon is lighting. 2. Automatic presser foot lifting after pause function when icon is lighting. 3. Automatic presser foot lifting after pause function or trimming function when two icons are lighting. 4. Presser Foot is inactive when two icons are not lighting. |
| Needle stop position selection |  | If click, the needle stop position after shift the sewing mode (up position / down position). |
| Slow launch |  | If click, set used or cancelled slow launch function. |
| Lift or mend needle |  | When sewing stop, press this key to actuate lift or mend needle function. |
| Function parameter edit |  | Enter or exit the user parameter setting interface. |
| Setting parameter check and save |  | View and save the content of the selected parameter number: After selecting the parameter number, press this key to view and modify the operation. After modifying the parameter value, press this key to exit and save the parameter. |
| Parameter increment |  | Increase the parameter. |
| Parameter decrement |  | Decrease the parameter. |
| Speed increment |  | Increase running speed |
| Speed decrement |  | Decrease running speed |

2 Auxiliary Function

2.1 Restore factory settings

In the power-on state, press and hold the auto function key and restart to restore the factory settings.

2.2 Quick setting of needle stop position

In the shutdown state, press and hold the S key to turn on, enter P72, manually turn the handwheel to the desired up position, the displayed value will change with the change of the handwheel position, press the S key to save the current position (value) as up position will automatically calculate down position at the same time.

3 User Parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed (rpm) | 100-2200 | 2200 | Maximum speed of machine sewing |
| P02 | Set accelerated curve (%) | 10-100 | 80 | Set the acceleration slope The greater the slope value, the steeper the speed; the smaller the slope value, the slower the speed |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle stops at up position DN: Needle stops at down position |
| P04 | Start back-tacking speed (rpm) | 200-3200 | 1200 |  |
| P05 | End back-tacking speed (rpm ) | 200-3200 | 1200 |  |
| P06 | Bar-tacking speed (rpm) | 200-3200 | 1200 |  |
| P07 | Soft start speed (rpm ) | 200-1500 | 400 |  |
| P08 | Stitch numbers for soft start | 1-15 | 2 |  |
| P09 | Automatic constant-stitch sewing speed (rpm) | 200-2200 | 2200 | Speed adjustment for automatic constant-stitch sewing |
| P10 | Automatic end back-tacking sewing after constant-stitch sewing | ON/OFF | ON | ON: After executing the constant-stitch sewing, the back-tacking sewing will be executed automatically. In any sewing mode, mending stitch function cannot be used. OFF: After executing the last constant-stitch sewing, the back-tacking sewing function will not be automatically executed, and the front step or full back step must be performed again. |
| P11 | Manual back-tacking mode selection | J/B | J | J: JUKI Mode (It will execute back-tacking during sewing or when it stops halfway) B: BROTHER Mode (It will execute back-tacking during sewing, and it will mend needle when it stops halfway) |
| P12 | Start back-tacking running mode selection | 0-2 | 1 | 0: Controlled by foot pedal, can be stopped and started at will 1: Touch the foot pedal to automatically execute back-tacking action 2: Pause, pattern sewing mode |
| P13 | Ending mode of start back-tacking | CON/STP | CON | CON: Start back-tacking is completed automatically continued for next action. STP: After the number of stitches is completed, stop automatically. |
| P14 | Slow start function selection | ON/OFF | OFF |  |
| P15 | Mending stitch mode | 0-4 | 2 | 0: half stitch; 1: one stitch; 2: continuous half stitch; 3: continuous one stitch; 4: continuous one stitch, quick stop. |
| P16 | Speed limit of manual back-tacking | 0-3000 | 0 | The function is disabled when the value is 0. |
| P17 | Auto count selection | 0-50 | 1 | 0: P41 item counter not count automatically. 1-50: Counter thread trimming times |
| P18 | Start back-tacking compensation 1 | 0-200 | 160 | Stitch compensation for start back-tacking A section, 0~200 action gradually delay; The large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start back-tacking compensation 2 | 0-200 | 170 | Stitch compensation for start back-tacking A section, 0~200 action gradually delay; The large value, the longer of the B section last stitch. |
| P20 | End back-tacking running mode selection | 0-2 | 1 | 0: Controlled by foot pedal, can be stopped and started at will 1: Touch the foot pedal to automatically execute back-tacking action 2: Pause, pattern sewing mode |
| P21 | The position of the pedal for accelerating | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for presser foot lifting | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P25 | End back-tacking compensation 3 | 0-200 | 160 | Stitch compensation for end back-tacking C section, 0~200 action gradually delay; The large value, the shorter of the C section first stitch. |
| P26 | End back-tacking compensation 4 | 0-200 | 170 | Stitch compensation for end back-tacking D section,0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P27-N12 | Boot counter interface selection | 0-1 | 0 | 0: OFF 1: ON |
| P27-N13 | Automatic piece counting mode selection | 0-1 | 0 | 0: Incremental piecework mode 1: Diminishing piecework mode |
| P28 | Bar-tacking running mode selection | 0-2 | 1 | 0: Controlled by foot pedal, can be stopped and started at will 1: Touch the foot pedal to automatically execute back-tacking action 2: Pause, pattern sewing mode |
| P29 | The strength of thread trimming stop | 1-45 | 23 |  |
| P32 | Bar-tacking compensation 5 | 0-200 | 160 | Stitch compensation for bar-tacking A (C) section, 0~200 action gradually delay; the large value, the longer of the A (C) section last stitch; the shorter of the B (D) section first stitch. |
| P33 | Bar-tacking compensation 6 | 0-200 | 170 | Stitch compensation for bar-tacking B (D) section, 0~200 action gradually delay; the large value, the longer of the B (D) section last stitch; the shorter of the C section first stitch. |
| P34 | Constant-stitch sewing running mode selection | A/M | A | A: Touch the foot pedal to automatically execute constant-stitch sewing action M: Controlled by foot pedal, can be stopped and started at will |
| P35 | Thread tension releasing function setting when presser foot lifting action | 0-2 | 0 | 0: OFF 1: Thread tension releasing output function ON when presser foot lifting, thread tension releasing output function OFF when stopover 2: Full function |
| P36 | Thread tension releasing function selection | 0-5 | 5 | 0: OFF 1: The strength of thread tension releasing gradually increases |
| P37 | Thread wiping function / Thread clamping function selection | 0-11 | 8 | 0 : OFF 1: Thread wiping function. 2-11: Thread clamping Function, the larger the value, the stronger the strength of automatic thread clamping. |
| P38 | Automatic thread trimming function selection | ON/OFF | ON |  |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN |  |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN |  |
| P41 | Thread trimming counter display | 0-9999 |  | Display the quantity of finished sewing piece. Long-press “-” key to clear the count |
| P42-N01 | The control system version number |  |  |  |
| P42-N02 | The panel version number |  |  |  |
| P42-N03 | Speed |  |  |  |
| P42-N04 | The pedal AD |  |  |  |
| P42-N05 | The mechanical angle(up position) |  |  |  |
| P42-N06 | The mechanical angle(down position) |  |  |  |
| P42-N07 | Bus bar voltage AD |  |  |  |
| P43 | Motor rotation direction setting | CCW/CW | CCW | CW: clockwise CCW: counter clock wise |
| P44 | Brake strength during stopover | 1-45 | 20 |  |
| P45 | Back-tacking operation duty cycle (%) | 1-60 | 50 | Back-tacking operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P46 | Motor stop with a reverse angle after trimming function | ON/OFF | OFF |  |
| P47 | Adjust the reverse angle when motor stop after trimming | 50-300 | 160 | Start from the upper needle position and adjust the angle of the needle lift in reverse operation after trimming. |
| P48 | The minimum speed (positioning speed) (rpm ) | 100-500 | 210 | Adjust the minimum speed |
| P49 | Thread trimming speed (rpm) | 100-500 | 200 | Adjust thread trimming speed |
| P50 | Presser foot operation time to full output (ms) | 10-990 | 150 |  |
| P51 | Presser foot operation duty cycle (%) | 1-60 | 40 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Delay the start of the motor to protect the lowering time of presser foot (ms) | 10-990 | 300 | Delay the start time, with automatic presser foot down. |
| P53 | Half back pedaling lifting presser foot function to cancel | ON/OFF | OFF | ON: Half back pedaling with lifting presser foot OFF: Half back pedaling without lifting presser foot |
| P54 | Thread trimming action time (ms) | 10-990 | 200 |  |
| P55 | Thread wiping action time (ms) | 10-990 | 10 |  |
| P56 | Power on and positioning | 0-2 | 0 | 0: Always not to find the up needle position 1: Always to find the up needle position 2: If the motor is already in the up needle position, no longer find the up needle position |
| P57 | Presser foot solenoid protection time (s) | 1-60 | 5 | Forced shutdown after hold time to prevent the electromagnet from being hot for a long time |
| P58 | Up needle position adjustment | 0-2399 | 40 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-2399 | 650 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed (rpm ) | 100-3500 | 2200 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Test run time | 1-250 | 30 |  |
| P65 | Test stop time | 1-250 | 10 |  |
| P66 | Machine protection switch selection | 0-1 | 1 | 0: Disable 1: Testing zero signal |
| P69 | Motor type selection | 0-50 | 0 | 0: 600 CPR, 6 slots, level 4 1: 360 CPR, 6 slots, level 4 2: 600 CPR, 6 slots, level 4 (large size rotating hook) 3: 360 CPR, 6 slots, level 4 (large size rotating hook) |
| P70 | Type |  |  |  |
| P71 | Presser foot release operation duty cycle (%) | 0-50 | 0 | ### The buffer force of presser foot release operation. |
| P72 | Up needle position adjustment | 0-2399 |  | Adjust up needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as up needle position. |
| P73 | Down needle position adjustment | 0-2399 |  | Adjust down needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as down needle position. |
| P76 | Back-tacking operation time to full output（ms） | 10-990 | 200 | Back tacking start action, full output action times. |
| P77 | Opportunity point of back-tacking for end back-tacking in high speed in free sewing mode | 20-350 | 235 |  |
| P78 | The start angle of thread clamping | 5-359 | 100 |  |
| P79 | The stop angle of thread clamping | 5-359 | 350 |  |
| P80 | Trimming engage angle | 5-359 | 18 | Trimming engage angle setting (down position defined as 0°)) |
| P83 | Stopping strength after trimming | 10-100 | 40 |  |
| P84 | Trimming full output time (ms) | 10-990 | 60 |  |
| P85 | Periodic signal of trimming output (*10%) | 1-10 | 7 |  |
| P86 | Up and down needle position distance | 10-2300 | 1020 |  |
| P87 | Wiping thread return delay time | 10-990 | 50 |  |
| P89 | AC overvoltage setting | 500-1023 | 880 |  |
| P92 | Correct the electrical angle of motor |  | 160 | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delay time of the half back pedaling function (ms) | 10-900 | 100 |  |
| P101 | The start angle of thread tension releasing | 1-359 | 30 | Thread tension releasing start angle (defined as 0°under calculation) |
| P102 | The stop angle of thread tension releasing | 1-359 | 180 | Thread tension releasing end angle (defined as 0°under calculation, must be greater than P101 parameter value) |
| P109 | The delay time before thread wiping | 0-990 | 5 |  |
| P110 | Trimming back time (ms) | 60-990 | 60 |  |
| P113 | Presser foot operation duty cycle to full output (%) | 0-100 | 100 |  |
| P117 | Press foot release buffer delay time (ms) | 0-990 | 0 |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 1 | 0: No alarm, turn off the output 1: Alarm and shutdown; |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4 Error Code List

| Error Code | Problem description | Solutions |
| --- | --- | --- |
| E01 | High voltage | Turn off the machine, check the power supply voltage. (Or whether exceed the rated voltage of use). |
| E02 | Low voltage | Turn off the machine, check the power supply voltage. (Or whether under the rated voltage of use). |
| E03 | CPU communication abnormal | Turn off the machine, please check if the operation panel in bad connection or is too loose, to get it right and turn on. |
| E05 | Pedal signal abnormal | Turn off the machine, please check if the pedal in bad connection or is too loose, to get it right and turn on. |
| E07 | Main shaft motor locked-rotor | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. |
| E08 | Manual back-tacking operation more than 15 seconds | Back-tacking electromagnet response time is too long, you have only to restart the machine. If there are still errors E-08 after restart the machine, check if the manual back-tacking switch is damaged. |
| E10 | Electromagnet overcurrent protection | Turn off the system power and check whether the electromagnet (solenoid valve) is damaged or short-circuited. |
| E09 E11 | The positioning signal is abnormal | Turn off the machine, please check if the encoder in bad connection or is too loose, to get it right and turn on. |
| E14 | Encoder signal is abnormal | Turn off the machine, please check if the encoder in bad connection or is too loose, to get it right and turn on. |
| E15 | Main shaft motor drive overcurrent | Turn off the machine, then turn on again. |
| E17 | Machine overturned | Turn off the machine, check if the machine is turn over, or the mercury switch is shifted or damaged. |
| E20 | Main shaft motor failed to start (Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose, to get it right and turn on. |

5 Port Diagram

1. Thread trimming electromagnet: 1, 8 (+32V)

2. Thread wiping/ thread clamping electromagnet: 2, 9 (+32V)

3. Thread tension releasing electromagnet: 3, 10 (+32V)

4. LED Light: 4 (DGND), 11 (+5V)

5. Back-tacking key: 5 (signal)

6. Back-tacking electromagnet: 6, 13 (+32V)

7. Mending stitch key: 7 (signal)