Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well-trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage matches the rated voltage indicated on the motor’s name plate.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail:

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note: We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker).

1. Key Description

| Icon | Name | Function Description |
| --- | --- | --- |
| Function parameter editing |  | Enter or exit the editing of function parameters. |
| Setting parameter check and save |  | Check and save the content of the selected parameter number: After selecting the parameter number, press this key to view and modify the operation, after modifying the parameter value, press this key to exit and save the parameter |
| Parameter increase |  | 1. Increase parameter items in the parameter selection area 2. Increase setting value in the parameter content area |
| Parameter decrease |  | 1. Decrease parameter items in the parameter selection area 2. Decrease setting value in the parameter content area |
| LED brightness adjustment |  | Press this key in turn to adjust the brightness of the lamp. |
| Soft start setting / Needle stop position selection |  | 1. In the boot interface, short press this key to select the soft start mode: when the corresponding decimal point lights up, it means that the function is turned on, and when it is off, it means that the function is off; 2. On the boot interface, long press this key to select the needle stop position: the corresponding decimal point lights up for up positioning, and off for down positioning; |

2. Operation Definition

2.1 Reset

1. Under the shutdown state, hold on  and  to boot, then press  to confirm it, shutdown restart.

2.2 Enter User Mode and Modify or Save Parameters

1. In the normal mode, press  to enter the [User Parameter Area].

2. Press  to enter the parameter content value; use ,  keys to change the content value, press to save.

2.3 Enter Technician Mode and Modify or Save Parameters

1. In the shutdown state, press  and boot-up to enter the [Technician parameter area].

2. Press  to enter the parameter content value; use ,  keys to change the content value, press to save.

3. User Parameters & Technician Parameters

3.1 User Parameters

| No. | Function parameters | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed | 200-5000 | 4500 | Maximum speed setting during sewing |
| P02 | Needle stop positioning selection | 0-2 | 0 | 0: Up needle position 1: Down needle position 2: OFF |
| P03 | Soft Start Switch | 0-1 | 0 | 0: OFF 1: ON |
| P04 | Soft Start Sewing Speed | 200-1500 | 400 |  |
| P05 | Stitches Number of Soft Start | 1-15 | 1 |  |
| P06 | Minimum sewing Speed | 200-500 | 200 | Minimum speed setting during sewing |

3.2 Technician Parameters

| No. | Function parameters | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P07 | Up needle position adjustment | 0-23 |  |  |
| P08 | Down position adjustment | 0-23 |  |  |
| P09 | Needle goes up as power on | 0-1 | 1 | 0: OFF 1: ON |
| P11 | Set accelerated curve (%) | 1-100 | 32 | Set the acceleration slope |
| P16 | Machine lamp brightness | 0-4 | 4 | 0: OFF 1→4: the brightness grade, the more value the more brightness. |
| P21 | Motor running direction | 0-1 | 0 | 0: CW 1: CCW |
| P22 | Test speed | 200-5000 | 3500 | Setting testing speed |
| P23 | Test working time | 1-250 | 20 |  |
| P24 | Test stop time | 1-250 | 20 |  |
| P25 | Item A Test | 0-1 | 0 | Speed continuous operation |
| P26 | Item B Test | 0-1 | 0 | Perform positioning cycle operation |
| P36 | Type | 1-10 |  |  |
| P40 | Parameter reference | N1-N4 |  | N1: Control box software version N3: Rotation N4: Pedal AD |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

4. Error Code List

| Error Code | Problem | Status / Measurement |
| --- | --- | --- |
| E01 | System overvoltage | Please cut off the power supply and confirm whether the power supply voltage is correct. |
| E02 | System undervoltage | Please cut off the power supply and confirm whether the power supply voltage is correct. |
| E05 | Speed control failure | Please cut off the power and confirm whether the speed controller is correctly connected to the control box. |
| E07 | Motor locked-rotor | Please turn the handwheel to confirm whether main shaft motor is locked. Turn the handwheel 45° after the alarm can automatically eliminate the alarm. Please confirm whether the encoder cable of the motor and the power cable of the motor are properly connected. |
| E09 | Positioner signal is abnormal | Please cut off the power supply, check whether the connection of the positioning signal board on the control box is loose, or whether magnetic steel plate installation location is standardized, restore it to normal and restart the system. |
| E14 | Encoder signal is abnormal | Please cut off the power, check whether the motor encoder connector is loose or fall off, restore it to normal and restart the system. |
| E15 | System overcurrent protection | Please cut off the power, check whether the motor power supply cable is damaged or abnormally connected, and restart after troubleshooting. |

5. Port Diagram

Clothing lights Port Description