Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- The product should be installed and pre-operated by well-trained persons.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting power supply cords to power sources, it is necessary to make sure that the power voltage matches the rated voltage indicated on the motor’s name plate.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 5°C.

- Please avoid operating near the heater at dew area or at the humidity below 30% or above 95%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation：

a) Connecting or disconnecting any connectors on the control box or motor.

b) Threading needle.

c) Raising the machine head.

d) Repairing or doing any mechanical adjustment.

e) Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

- Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

- Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

- Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

Note: We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device.  (Such as residual current breaker)

1 Button Displays and Operating Instructions

  - Key Description

| Name | key | Indicate |
| --- | --- | --- |
| Function Parameter Edit |  | If click, enter or exit the user parameter setting interface. |
| Setting Parameter Check and Save |  | For the content of the selected parameters check and save: after select parameter press this key to check and modify operation, after modified parameter value press this key to exit and save the parameter. |
| Up Key |  | If click, increase the parameter / increase speed. If long-press, continuously increase the parameter / continuously increase speed. |
| Down Key |  | If click, decrease the parameter / decrease speed. If long-press, continuously decrease the parameter / continuously decrease speed. |
| Trimming Setting |  | If click, set used or cancelled trimming function. If long-press, Quickly enter the P41 item counter parameter value interface. |
| Needle Stop Position Selection |  | If click, the needle stop position after shift the sewing mode ( Up position / Down position) |
| Slow Launch Setting |  | If click, set used or cancelled slow Launch Setting function. |

1.2 Auxiliary Function

1.2.1 Enter the Advanced Parameter Item Interface

Press and hold  to boot up, and enter the advanced parameter item interface.

1.2.2 Reset the System

Press and hold  and  to boot up, enter the P44 parameter interface, and then press  to jump to the main interface, indicating that the factory settings are restored.

1.2.3 Factory Parameter Value Saving Function

In the parameter value interface, press and hold  for 3 seconds to save the current parameter as the factory default value, and the successful display shows "SAVE".

2 User Parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed (rpm) | 100-5000 | 3700 | Maximum speed of machine sewing |
| P02 | Set accelerated curve (%) | 10-100 | 80 | Set the acceleration slope The greater the slope value, the steeper the speed; the smaller the slope value, the slower the speed |
| P03 | Needle UP/ DOWN | UP/DN | DN | UP: Needle stops at up position DN: Needle stops at down position |
| P07 | Soft start speed (rpm) | 200-1500 | 400 |  |
| P08 | Stitch numbers for soft start | 1-9 | 2 |  |
| P14 | Slow start function selection | ON/OFF | OFF |  |
| P15 | Mending stitch mode | 0-4 | 2 | 0: half stitch 1: one stitch 2: continuous half stitch 3: continuous one stitch 4: Back-tacking when machine stop or pause |
| P21 | The position of the pedal for accelerating | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P27-N6 | Auto count selection | 0-50 | 1 | 0: P41 item trimming counter not count automatically. 1-50: Counter trimming times |
| P27-N12 | Boot display counter interface selection | 0-1 | 0 | 0: OFF 1: ON |
| P27-N13 | Thread trimming counter mode selection | 0-1 | 0 | 0: Additive sewing counter 1: Subtractive sewing counter |
| P29 | The strength of thread trimming stop | 1-45 | 32 |  |
| P38 | Automatic thread trimming function selection | ON/OFF | ON |  |
| P41 | Counter display | 0-9999 | 0 | Display the quantity of finished sewing piece. Long-press “-” key to clear the count |
| P42-N01 | The control system version number |  |  |  |
| P42-N02 | The panel version number |  |  |  |
| P42-N03 | Speed |  |  |  |
| P42-N04 | The pedal AD |  |  |  |
| P42-N05 | The mechanical angle(up position) |  |  |  |
| P42-N06 | The mechanical angle(down position) |  |  |  |
| P42-N07 | Busbar voltage AD |  |  |  |
| P43 | Direction of motor rotation | CCW/CW | CCW | CW: Clockwise CCW: Counterclockwise |
| P44 | Brake strength during stopover | 1-45 | 30 |  |
| P46 | Motor stop with a reverse angle after trimming function | ON/OFF | OFF |  |
| P47 | Adjust the reverse angle when motor stop after trimming | 50-200 | 160 | After thread trimming, start from the upper needle position and adjust the angle of the needle lift in reverse operation. |
| P48 | The minimum speed (positioning speed) (rpm) | 100-500 | 210 | Adjust the minimum speed |
| P49 | Thread trimming speed (rpm) | 100-500 | 300 | Adjust the motor speed during the thread trimming cycle |
| P54 | Thread trimming action time (ms) | 10-990 | 200 |  |
| P56 | Power on and positioning | 0-2 | 0 | 0: Always not to find the up needle position 1: Always to find the up needle position 2: If the motor is not into the up needle position, it will find the up needle position |
| P58 | Up needle position adjustment | 0-2399 | 1950 | Up position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down needle position adjustment | 0-2399 | 750 | Down position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed (rpm ) | 100-3700 | 3500 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Test run time | 1-250 | 30 |  |
| P65 | Test stop time | 1-250 | 10 |  |
| P66 | Machine protection switch selection | 0-1 | 1 | 0：Disable 1：Testing zero signal |
| P70 | Type | 1-45 |  |  |
| P72 | Up needle position adjustment | 0-2399 |  | Adjust up needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as up needle position. |
| P73 | Down needle position adjustment | 0-2399 |  | Adjust down needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as down needle position. |
| P80 | Trimming engage angle | 5-359 | 18 |  |
| P82 | Trimming retract angle | 5-359 | 172 |  |
| P83 | Stopping strength after trimming | 10-100 | 20 |  |
| P86 | Up and down needle position distance | 100-1400 | 1200 | Up and down positioning distance angle (1 degree for every 4 values) |
| P88 | Stopover distance | 10-100 | 40 |  |
| P89 | AC overvoltage setting | 500-1023 | 880 |  |
| P92 | Correct the electrical angle of motor |  | 160 | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P110 | Trimming back time (ms) | 60-990 | 60 | Make sure the thread trimming device returns to its original position |
| P119 | Solenoid overcurrent protection alarm selection | 0-1 | 1 | 0: no alarm, turn off the output 1: alarm, and stop |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Error Code List

| Error Code | Problem | Measure |
| --- | --- | --- |
| E01 | Over-voltage | Turn off the system power and check whether the power supply voltage is correct. (Or whether it exceeds the specified rated voltage). |
| E02 | Low-voltage | Turn off the system power and check whether the power supply voltage is correct. (Or whether it is lower than the specified rated voltage). |
| E03 | Abnormal communication between the main CPU and the driver CPU | Turn off the power and check whether the connector is connected to the operation panel and the cable condition. |
| E05 | Speed control unit connection is abnormal | Please turn off the power and check whether the connector is connected to the speed controller and the cable condition. |
| E07 | Main shaft motor rotating abnormally | Please rotate handwheel to check if the spindle motor is locked. Check if the encoder cable of the motor and the power cable of the motor are connected to the connector. Check if the power supply voltage is normal and whether the sewing speed is too high. |
| E10 | Solenoid over-current protection | Turn off the system power supply, please check the solenoid (solenoid valve) connector or solenoid (solenoid valve) is broken or not. |
| E09 E11 | Positioning signal error | Turn off the machine, please check if the encoder in bad connection or is too loose. To get it right and turn on. If there are still errors, please replace the control box and call for customer service. |
| E14 | Encoder signal error. | Turn off the system power supply, please check motor encoder connector if loose or fall off, renew to normal then restart system. |
| E15 | Main shaft motor overcurrent error | Turn off the power, and then turn it on again. |
| E17 | Head fall error | Please raise machine head and turn on the power again. Check if the head down switch is damaged. |
| E20 | Main shaft motor rotating abnormally at startup | Turn off the power and check whether the encoder cable of the motor and the power cable of the motor are connected to the connector. |

4 Port Diagram

6P Function Port Description

1. Thread trimming electromagnet: 1, 4 (+32V)

2. Mending stitch key: 2(signal), 5 (DGND)

3. LED Light: 3 (+5V), 6 (DGND)